3GPP TSG-SA WG6 Meeting #64	S6-245054

Orlando, USA, 18th – 22nd November 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.180 | CR | 0004 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause 5 update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Generic_IOPS |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR updates several subclauses related to the architectural requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The terms mentioned in the architectural requirement subclauses are updated to make them applicable to a generic IOPS solution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing IOPS solution is not agnostic to the 3GPP network access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2, 5.3, 5.3.2, 5.4.2, 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The reference [23.289] is included in CR0010. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.1.2	Requirements

[AR-5.1.2-a] For the case of a backhaul failure between the radio access network and the macro3GPP core network, the IOPS MC system shall provide support for MC services in the IOPS mode of operation until the failure is recovered.

[AR-5.1.2-b] MC service UEs within the coverage of the local IOPS 3GPP system shall be able to provide MC services over the IOPS MC system.

[AR-5.1.2-c] The IOPS MC system shall provide mechanisms to verify the availability of MC service UEs within the IOPS MC system.

# * * * * Second change * * * *

## 5.3	Network broadcast/multicast session requirements

### 5.3.1	Description

This clause specifies the network bearer requirements in the IOPS mode of operation.

### 5.3.2	Requirements

[AR-5.3.2-a] The IOPS MC system shall provide support for unicast session establishment and modification to support MC services via unicast transmissions in the IOPS mode of operation.

[AR-5.3.2-b] The IOPS MC system shall provide support for broadcast/multicast session establishment and modification to support MC services via multicast transmissions in the IOPS mode of operation.

[AR-5.3.2-c] The IOPS MC system shall support announcement of broadcast/multicast session to the MC service UEs.

# * * * * Third change * * * *

### 5.4.2	General requirements

[AR-5.4.2-a] The IOPS MC connectivity function supporting the IP connectivity functionality shall support discovering MC service users within the coverage of the local IOPS 3GPP system.

[AR-5.4.2-b] The IOPS MC connectivity function supporting the IP connectivity functionality shall support announcement, subscription and notification mechanisms for discovered MC service users supporting the IP connectivity functionality.

[AR-5.4.2-c] The IOPS MC connectivity function supporting the IP connectivity functionality shall provide IP connectivity for communication among MC service users requesting the support of the IP connectivity functionality.

[AR-5.4.2-d] The MC service UEs supporting the IP connectivity functionality in the IOPS mode of operation shall enable transmitting the IP packets related to a MC service communication over the IOPS MC connectivity function.

### 5.4.3	Bearer management requirements

[AR-5.4.3-a] The MC service UEs shall use specific APNs or DNNs for the support of the IP connectivity functionality, as follows:

-	An APN/DNN for the IOPS related application signalling related to the IP connectivity functionality support, e.g. for IOPS discovery, subscription and notification.

-	An MC service APN/DNN for the communication with another MC user or group via the IOPS MC connectivity function based on the IP connectivity functionality, e.g. for the signalling and media.

[AR-5.4.3-b] The APNs for the IP connectivity functionality support shall be made available to the MC service UEs either via UE (pre)configuration or via initial UE configuration, as described in 3GPP TS 23.280 [3], and 3GPP TS 23.289 [23.289].

# * * * * End of changes * * * *