3GPP TSG-SA WG6 Meeting #64	S6-245056

Orlando, USA, 18th – 22nd November 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.180 | CR | 0006 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause 7 update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Generic_IOPS |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR updates several subclauses related to the functional model in clause 7 to make it applicable to a generic IOPS solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating terms in the subclauses related to the functional model to make it a generic IOPS solution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing IOPS solution is not agnostic to the 3GPP network access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2, 7.3.3, 7.3.4, 7.4.1.3, 7.5.1.2, 7.5.1.3, 7.5.1.4, 7.5.1.5, 7.5.1.6, 7.5.1.7, 7.5.1.8, 7.5.1.9, and 7.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The references [23.501], [23.503], and [23.274] are included in CR0010. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.3.2	Functional model for the signalling control plane in the IOPS mode of operation

Figure 7.3.2-1 shows the common functional model for the signalling control plane in the IOPS mode of operation.

Figure 7.3.2-1 Functional model for the signalling control plane in the IOPS mode of operation

NOTE:	The functional model for the signalling plane in the IOPS mode of operation is described as a common functional model across MC services.

### 7.3.3	MCPTT functional model for the application plane in the IOPS mode of operation

Figure 7.3.3-1 shows the MCPTT functional model for the application plane in the IOPS mode of operation.

Figure 7.3.3-1 MCPTT functional model for the application plane in the IOPS mode of operation

NOTE:	MC-IOPS-1, MC-IOPS-5 and MC-IOPS-6 reference points are common across MC services.

In the model shown in figure 7.3.3-1, the following apply:

-	MCPTT-IOPS-2 carries signalling related to the support of broadcast/multicast transmissions, e.g. MBMS bearer announcement, or 5G MBS session announcement, between the IOPS MC connectivity function and the MCPTT client.

-	MCPTT-IOPS-3 carries signalling and media over IP unicast transmissions between the IOPS distribution function and the MCPTT client of the MCPTT UE.

-	MCPTT-IOPS-4 carries signalling and media on downlink over IP multicast transmissions between the IOPS distribution function and the MCPTT client of the MCPTT UE.

### 7.3.4	MCData functional model for the application plane in the IOPS mode of operation

Figure 7.3.4-1 shows the MCData functional model for the application plane in the IOPS mode of operation.

Figure 7.3.4-1 MCData functional model for the application plane in the IOPS mode of operation

NOTE:	MC-IOPS-1, MC-IOPS-5 and MC-IOPS-6 reference points are common across MC services.

In the model shown in figure 7.3.4-1, the following apply:

-	MCData-IOPS-2 carries signalling related to the support of broadcast/multicast transmissions, e.g. MBMS bearer announcement, or 5G MBS session announcement, between the IOPS MC connectivity function and the MCData client.

-	MCData-IOPS-3 carries signalling and media over IP unicast transmissions between the IOPS distribution function and the MCData client of the MCData UE.

-	MCData-IOPS-4 carries signalling and media on downlink over IP multicast transmissions between the IOPS distribution function and the MCData client of the MCData UE.

# * * * * Third change * * * *

#### 7.4.1.3	IOPS MC connectivity function

The IOPS MC connectivity function entity provides supporting MC services in the IOPS mode of operation. It includes registering and discovering users within the IOPS MC system. The IOPS MC connectivity functional entity represents a specific instantiation of the GCS AS described in 3GPP TS 23.468 [4] for EPS or Application Function (AF) for 5GS to manage unicast and multicast transmissions for group communication related sessions.

The IOPS MC connectivity function entity is supported by the signalling application server functional entity of the signalling control plane.

# * * * * Fourth change * * * *

#### 7.5.1.2	Reference point MCPTT-IOPS-2 (between the IOPS MC connectivity function and the MCPTT client)

The MCPTT-IOPS-2 reference point, which exists between the IOPS MC connectivity function and the MCPTT client, is used to communicate signalling related to the broadcast/multicast sessions  utilized for multicast transmissions, e.g. MBMS bearer announcement or 5G MBS session announcement in the IOPS mode of operation.

#### 7.5.1.3	Reference point MCData-IOPS-2 (between the IOPS MC connectivity function and the MCData client)

The MCData-IOPS-2 reference point, which exists between the IOPS MC connectivity function and the MCData client, is used to communicate signalling related to the broadcast/multicast sessions utilized for multicast transmissions, e.g. MBMS bearer announcement or 5G MBS session announcement in the IOPS mode of operation.

#### 7.5.1.4	Reference point MCPTT-IOPS-3 (between the IOPS distribution function and the MCPTT client)

The MCPTT-IOPS-3 reference point, which exists between the IOPS distribution function and the MCPTT client, is used for IP unicast transmissions in uplink and downlink. The MCPTT-IOPS-3 reference point uses the SGi reference point as defined in 3GPP TS 23.002 [7] or N6 reference point as defined in 3GPP TS 23.501 [23.501].

#### 7.5.1.5	Reference point MCData-IOPS-3 (between the IOPS distribution function and the MCData client)

The MCData-IOPS-3 reference point, which exists between the IOPS distribution function and the MCData client, is used for IP unicast transmissions in uplink and downlink. The MCData-IOPS-3 reference point uses the SGi reference point as defined in 3GPP TS 23.002 [7] or N6 reference point as defined in 3GPP TS 23.501 [23.501].

#### 7.5.1.6	Reference point MCPTT-IOPS-4 (between the IOPS distribution function and the MCPTT client)

The MCPTT-IOPS-4 reference point, which exists between the IOPS distribution function and the MCPTT client, is used for IP multicast transmissions in downlink. The MCPTT-IOPS-4 reference point uses the MB2-U reference point as defined in 3GPP TS 23.468 [4] or Nmb8/ Nmb6 reference point as defined in 3GPP TS 23.247 [23.247].

#### 7.5.1.7	Reference point MCData-IOPS-4 (between the IOPS distribution function and the MCData client)

The MCData-IOPS-4 reference point, which exists between the IOPS distribution function and the MCData client, is used for IP multicast transmissions in downlink. The MCData-IOPS-4 reference point uses the MB2-U reference point as defined in 3GPP TS 23.468 [4] or Nmb8/ Nmb6 reference point as defined in 3GPP TS 23.247 [23.247].

#### 7.5.1.8	Reference point MC-IOPS-5 (between the IOPS MC connectivity function and the local IOPS 3GPP system )

The MC-IOPS-5 reference point, which exists between the IOPS MC connectivity function and the local IOPS 3GPP system , is used to obtain unicast sessions with appropriate QoS from the local IOPS 3GPP system. The MC-IOPS-5 reference point utilizes the Rx interface of the EPS according to 3GPP TS 23.203 [8], or N5 interface of 5GS according to 3GPP TS 23.503[23.503].

#### 7.5.1.9	Reference point MC-IOPS-6 (between the IOPS MC connectivity function and the local IOPS 3GPP system )

The MC-IOPS-6 reference point, which exists between the IOPS MC connectivity function and the 3GPP system (IOPS EPS), is used to request the allocation and activation of broadcast/multicast transport resources, i.e. MBMS bearers or 5G MBS sessions, during the IOPS mode of operation. The MC-IOPS-6 reference point uses the MB2-C interface as defined in 3GPP TS 23.468 [4], or Nmb10, Nmb13, or N33, as defined in 3GPP TS 23.247 [23.247].

# * * * * Fifth change * * * *

#### 7.5.2.1	Reference point MC-IOPS-X1 (between the signalling application server and the signalling user agent client)

The MC-IOPS-X1 reference point, which exists between the signalling application server and the signalling user agent client, is used to support the IOPS related application signalling (e.g. registration, publication, subscription and notification events).

NOTE:	The IOPS related signalling supporting, e.g., the IOPS discovery, subscription and notification events can be SIP-based. In such a case, the signalling application server acts as a SIP application server (AS) or application function (AF) and the signalling user agent client acts as the SIP user agent for all SIP related transactions.

# * * * * End of changes * * * *