3GPP TSG-SA6 Meeting #64	S6-245060

# Orlando, USA, 18th - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0218 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API invoker obtaining authorization from resource owner |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (UK) Limited, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the pre-condition of clause 8.31.3 the ROF communicates with the CCF hosted AF, rather than API invoker. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct clause 8.31.3 pre-condition. Expand the steps to include the obtain service API authorization request and response (in alignment with clause 8.11). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect interpretation of the pre-conditions for API invoker obtaining authorization from resource owner procedure described in clause 8.31. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 3.2, 8.31.1 & 8.31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

AEF	API Exposing Function

AF	Application Function

AMF	API Management Function

APF	API Publishing Function

API	Application Program Interface

AS	Application Server

BM-SC	Broadcast Multicast Service Centre

CAPIF	Common API Framework

CDR	Charging Data Record

CRUD	Create, Read, Update, Delete

DDoS	Distributed Denial of Service

E-UTRA	Evolved Universal Terrestrial Radio Access

EPS	Evolved Packet System

ETSI	European Telecommunications Standards Institute

GS	Group Specification

IP	Internet Protocol

MBMS	Multimedia Broadcast and Multicast Service

MEC	Multi-access Edge Computing

NEF	Network Exposure Function

NGSI	Next Generation Service Interfaces

NR	New Radio

OMA	Open Mobile Alliance

OAM	Operations, Administration and Maintenance

OWSER	OMA Web Services

PC	Protocol Converter

PLMN	Public Land Mobile Network

REST	REpresentational State Transfer

RNAA	Resource owner-aware Northbound API Access

RO	Resource Owner

ROF	Resource Owner Function

RPC	Remote Procedure Call

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SNPN	Stand-alone Non-Public Network

UDDI	Universal Description, Discovery and Integration

URI	Uniform Resource Identifier

WSDL	Web Services Description Language

* * * Second Change * * * *

## 8.31	API invoker obtaining authorization from resource owner

### 8.31.1	General

Using the procedure described in clause 8.11 the CAPIF may authorize the API invoker to invoke the service API. This clause describes an extension to that procedure to enable the CAPIF to obtain authorization information from the resource owner prior to providing the authorization response to the API invoker.

Clause 8.31.3 shows the procedure for obtaining the authorization information.

### 8.31.2	Information flows

NOTE:	The security aspects of this procedure are specified in TS 33.122 [12].

### 8.31.3	Procedure

Figure 8.31.3-1 illustrates the procedure for API invoker obtaining authorization from resource owner.

Pre-conditions:

1.	The resource owner function (ROF) can communicate with the CCF (Authorization function).

2.	Access to an API exposing function offered service API requires obtaining authorization from a resource owner (RO).

Figure 8.31.3-1: Procedure for API invoker obtaining authorization from resource owner

1.	The API invoker requests to obtain RO authorization information to invoke the service API exposed by the API exposing function by sending an obtain service API authorization request to the CCF.

2.	The CCF validates the authentication of the API invoker (using authentication information). Then the authorization function determines the authorization by interacting with the RO via the ROF.

NOTE:	The detailed procedure to obtain the RO's authorization information is specified in TS 33.122 [12].

3.	Based on the RO provided authorization, the CCF sends an obtain service API authorization response to the API invoker.

2.	The API invoker sends service API invocation request to the API exposing function with the RO authorization information received in step 1.

3.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the RO authorization information.

* * * End of Changes * * * *