3GPP TSG-SA WG6 Meeting #64	S6-245079

Orlando, USA, 18th – 22nd November 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on SM data source APIs

Spec:	3GPP TS 23.437 v0.3.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes normative text to define SM data source APIs.

2. Reason for Change

SEAL APIs for SM data source must be specified.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.3.0.

* * * First Change * * * *

## 9.7	SEAL APIs for spatial map management

### 9.7.1	General

Table 9.7.1-1 illustrates the SEAL APIs for spatial map management.

Table 9.7.1-1: List of SEAL APIs for spatial map management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SmManagement | Create | VAL server, SEAL SM client | Request/Response |
|  | Discover | VAL server, SEAL SM client | Request/Response |
|  | Get | VAL server, SEAL SM client | Request/Response |
|  | Update | VAL server, SEAL SM client | Request/Response |
|  | Delete | VAL server, SEAL SM client | Request/Response |
|  | Subscribe | VAL server, SEAL SM client | Request/Response |
|  | Notify | VAL server, SEAL SM client | Notify |
|  | Unsubscribe | VAL server, SEAL SM client | Request/Response |
| SS_SmDataSource | Register | SEAL SM client | Request/Response |
|  | Update | SEAL SM client | Request/Response |
|  | Deregister | SEAL SM client | Request/Response |
|  | Notify | SEAL SM client | Notify |
|  | Discover | VAL server, SEAL server | Request/Response |

* * * Next Change * * * *

### 9.7.x	SS_SmDataSource API

#### 9.7.x.1	General

Service description: This API enables a SEAL SM client to manage SM data source registration in a SEAL SM server and enables a VAL server or SEAL server to discover SM data sources registered in a SEAL SM server.

#### 9.7.x.2	Register operation

API operation name: SS_SmDataSource_Register

Description: Register a SM data source.

Known Consumers: SEAL SM client.

Inputs: Refer Table 9.5.2.3.1-1

Outputs: Refer Table 9.5.2.3.2-1

See clause 9.5.2 for the details of usage of this API operation.

#### 9.7.x.3	Update operation

API operation name: SS_SmDataSource_Update

Description: Update a SM data source registration.

Known Consumers: SEAL SM client.

Inputs: Refer Table 9.5.3.3.1-1

Outputs: Refer Table 9.5.3.3.2-1

See clause 9.5.3 for the details of usage of this API operation.

#### 9.7.x.4	Deregister operation

API operation name: SS_SmDataSource_Deregister

Description: Deregister a SM data source.

Known Consumers: SEAL SM client.

Inputs: Refer Table 9.5.4.3.1-1

Outputs: Refer Table 9.5.4.3.2-1

See clause 9.5.4 for the details of usage of this API operation.

#### 9.7.x.5	Notify operation

API operation name: SS_SmDataSource_Notify

Description: Notify a SM data source to trigger a spatial mapping information session.

Known Consumers: SEAL SM client.

Inputs: None

Outputs: Refer Table 9.5.5.3.3-1

See clause 9.5.5 for the details of usage of this API operation.

#### 9.7.x.6	Discover operation

API operation name: SS_SmDataSource_Discover

Description: Discover a SM data source.

Known Consumers: VAL server, SEAL server.

Inputs: Refer Table 9.5.5.3.1-1

Outputs: Refer Table 9.5.5.3.2-1

See clause 9.5.5 for the details of usage of this API operation.

* * * End of Changes * * * *