3GPP TSG-SA6 Meeting #64	S6-245088

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0343 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolution on Geofencing monitoring events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editor's note: Clause 9.3.2.14 "Whether geofencing monitoring events and trigger events can be merged is FFS." needs to be addressed. Both Geofencing monitoring events IE and Triggering events IE are defined in the Table 9.3.2.14-1: Location information monitoring subscription request. Analyzing the Location area monitoring subscribe procedure defined in clause 9.3.12.1, the VAL server can subscribe to a list of UEs moving in/out of the specific location area using Triggering events IE. The VAL server can also use this procedure to subscribe Geofencing monitoring events such as UEs moving in/out of the pre-defined location area, the number of UEs moving in/out of the pre-defined location area, and the time period of UEs enter/leave the pre-defined location area. Both the Triggering events IE and Geofencing monitoring events IE specify the same information indicating the UE(s) moving in/out of the location area. Therefore, these two IEs can be merged into Geofencing monitoring events IE. And this CR suggests some editorial corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Triggering events IE can be merged into Geofencing monitoring events IE and delete the editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | These two IEs are defined redundantly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.14, 9.3.2.16, 9.3.2.17, 9.3.12.1, 9.3.12.2, 9.3.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.14	Location area monitoring subscription request

Table 9.3.2.14-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription request.

Table 9.3.2.14-1: Location area monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| VAL service ID | O | Identity of the VAL service for which the VAL server requests (e.g.Geofencing). |
| Location Information criteria | M | Location information to be monitored. It includes the geographic location information or a VAL service area identified by the VAL service area ID or a reference UE along with the application defined proximity range from the reference UE. This information also applies for Geofencing service. |
| Geofencing monitoring events | O | It indicates the Geofencing events to be monitored. |
| > UEs moved in/out | O | It indicates to monitor the UE(s) moving in/out of the area as defined in location information criteria, |
| > Numbers of UEs moved in/out | O | Indicates the numbers of UE(s) moving in/out of the area as defined in location information criteria are to be monitored, |
| > Time of UEs moved in/out | O | Indicates the time/time duration of UE(s) moving in/out of the area as defined in location information criteria is to be monitored, |
| Time between consecutive reports | O | It indicates the interval time between consecutive reports |
| Immediate Report Indicator | O | Indicates whether an immediate location report is required |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about location area monitoring. |

* * * Next Change * * * *

9.3.2.16	Location area monitoring notification

Table 9.3.2.16-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.16-1: Location area monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription |
| UEs currently present (see NOTE) | O | List of the identities of all VAL UEs who are currently present in the given location area |
| UEs moved in/out (see NOTE) | O | List of UEs either moved in to the location area or moved out of the location area |
| > UEs moved in | O | List of the identities of the VAL UEs who moved in to the given location area since previous notification. |
| > UEs moved out | O | List of the identities of the VAL UEs who moved out of the given location area since previous notification. |
| Number of UEs moved in/out | O | Number of the VAL UE(s) who moved in/out the given location area in a given time period. |
| > Number of UEs moved in | O | Number of the VAL UE(s) who moved in the given location area in a given time period. |
| > Number of UEs moved out | O | Number of the VAL UE(s) who moved out the given location area in a given time period. |
| Time of UEs moved in/out | O | The timestamp and time period of UEs either moved in the location area or moved out of the location area. |
| >Time stamp and time duration of UE moved in | O | The timestamp and time period of UEs who moved in the given location area since previous notification. |
| >Time stamp and time duration of UE moved out | O | The timestamp and time period of UEs who moved out the given location area since previous notification. |
| Triggering event | O | Identity of the event that triggered the sending of the notification |
| Reason | O | It indicates the failure reason for the notification report (e.g., "UE cannot be connected") when the LM Server can’t obtain the UE location data. |
| NOTE: Any one of these shall be present. |  |  |

* * * Next Change * * * *

9.3.2.17	Location area monitoring subscription modify request

Table 9.3.2.17-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription modify request.

Table 9.3.2.17-1: Location area monitoring subscription modify request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| VAL service ID | O | Identity of the VAL service for which the VAL server requests (e.g., Geofencing). |
| Subscription identity | M | Identifies the subscription |
| Location Information criteria | M | Location information to be monitored. It includes the geographic location information or a VAL service area ID or a reference UE along with the application defined proximity range from the reference UE. This information also applies for Geofencing service. |
| Geofencing monitoring events | O | It indicates the Geofencing events to be monitored. |
| > UEs moved in/out | O | It indicates to monitor the UE(s) moving in/out of the area as defined in location information criteria, |
| > Numbers of UEs moved in/out | O | Indicatse the numbers of UE(s) moving in/out of the area as defined in location information criteria are to be monitored, |
| > Time of UEs moved in/out | O | Indicates the time/time duration of UE(s) moving in/out of the area as defined in location information criteria is to be monitored, |
| Time between consecutive reports | O | It indicates the interval time between consecutive reports |
| Immediate Report Indicator | O | Indicates whether an immediate location report is required |
| Notification Target URI | O | Target URI where the VAL server wishes to receive the notifications about location area monitoring. |

* * * Next Change * * * *

9.3.12	Location area monitoring information procedure

9.3.12.1	Location area monitoring subscription procedure

Figure 9.3.12.1-1 illustrates the high level procedure of location area monitoring subscription request. The same procedure can be applied for location management client and other SEAL servers that would like to subscribe to the list of UEs moving in or moving out of the specific location area. The subscription request can be for a reference UE for which the subscriber is authorized to monitor location information.

The procedure can also be applied for Geofencing service which initiated via VAL server to subscribe to the LM server to report the Geofencing monitoring events.

Figure 9.3.12.1-1: Location area monitoring subscription procedure

1.	The VAL server sends a location area monitoring subscription request to the location management server to subscribe to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.14-1. The location information criteria may include the geographic location information or the VAL service area identified by the VAL service area ID where the UEs moving in or moving out to be monitored, or it may include reference UE information where in the UEs moving in or moving out of given application defined proximity range from the reference UE (target UE) to be monitored. The reference UE information may include VAL UE ID.

If the VAL server initiates the subscription for the Geofencing service, the Geofencing monitoring events (i.e. the UEs moving in/out of the pre-defined location area, the number of UEs moving in/out of the pre-defined location area, the time period of UEs enter/leave the pre-defined location area) can be included in the request message as specified in Table 9.3.2.14-1.

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription request.

3.	The location management server replies with a location area monitoring subscription response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.15-1.

9.3.12.2	Location area monitoring subscription modify procedure

Figure 9.3.12.2-1 illustrates the high level procedure of location area monitoring subscription modify request. The same procedure can be applied for location management client and other SEAL servers that would like to modify the subscription to the list of UEs moving in or moving out of the specific location area. The subscribe modify request can be for a reference UE for which the subscriber is authorized to monitor location information.

The procedure can also be applied for Geofencing service that the VAL server would like to modify the subscription to the LM server to report the Geofencing monitoring events.

Figure 9.3.12.2-1: Location area monitoring subscription modify procedure

1.	The VAL server sends a location area monitoring subscription modify request to the location management server to modify the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.17-1.

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription modify request.

3.	The location management server replies with a location area monitoring subscription modify response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.18-1.

* * * Next Change * * * *

9.3.12.4	Location area monitoring notification procedure

Figure 9.3.12.4-1 illustrates the high level procedure of location area monitoring notification. The same procedure can be applied for location management client and other SEAL servers who have subscribe to the list of the UEs moving in or moving out of the specific location area.

The procedure can also be applied for Geofencing service which requests the LM server to report the Geofencing monitoring events.

Figure 9.3.12.4-1: Location area monitoring notification procedure

1.	One of the events occurs at the location management server as specified in the subscribe request. The location management server identifies the UEs which are moved into the area or moved out of the area based on their location data and time stamp of the location. The LMS may report the list of all UEs in the given location or UEs moved in and moved out.

When the Geofencing service is triggered, the LM Server also checks whether the UE is inside/outside the area based on the location information criteria as defined in Table 9.3.2.14-1 and the received UE location data. Further, the LM Server statistics and reports the Geofencing monitoring events (e.g. the list of UEs moving in/out of the location area, the number of UEs moving in/out of the location area, the time period of UEs enter/leave the location area, etc.) to the VAL server.

2.	The location management server sends a location area monitoring notification to the VAL server. In the notification message, the location management server includes the information as specified in Table 9.3.2.16-1.

If the LM server didn't obtain any UE location information due to UE issues or network issues in step 1, it informs the VAL server with the proper reason (e.g., "UE cannot be connected") in the notification report.

3.	After received the Geofencing information report from the LM Server, the VAL server may take some actions to inform the VAL UE (e.g. send the alarms or warnings to the UE).

NOTE:	The step 3 is out of 3GPP scope.

* * * End of Changes * * * *