3GPP TSG-SA WG6 Meeting #64	S6-245093

Orlando (FL), USA, 18th – 22nd November 2024	(revision of S6-xxxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0062 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for clause 3.1 & 8.5.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Clause 3.1, 3GPP TS 22.101 [5] should be [3]. 2. Clause 8.5.8.2.2, in step 1 the request may include the UE identifier such as GPSI, PIN client ID, UE location and PIN ID. Leave PIN request no need UE location IE, and in Table 8.5.8.3.4-1 there is no UE location IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 3.1 & 8.5.8.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent information flow with procedure of PIN elements decides to leave the PIN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 8.5.8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Control Information: A set of information that assists the authorized PINE in a PIN to access 5GS network via PEGC, for example, the username or password.

PIN enabler: Refers to the overall functionality provided by the entities such as PIN Client, PIN Gateway Client, PIN Management Client, and PIN server in support of applications as per the architecture specified in clause 6

PIN management: Refers to the set of operations related to creation, modification, maintenance and removal of PIN.

PIN Profile: A set of data and information about the PIN and PIN elements belonging to a PIN.

NOTE:	3GPP TS 22.101 [3] clause 26a lists information that can be included in a PINE profile.

Service Switch: A mechanism to switch the service traffic flow between Application server and PINE to application server and other PINE.

For the purposes of the present document, the following terms given in 3GPP TS 22.261 [2] apply:

Personal IoT Network

PIN direct connection

PIN Element

PIN Element with Gateway Capability

PIN Element with Management Capability

* * * Next Change * * * *

8.5.8.2.2	Procedure of PIN elements decides to leave the PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has the application layer connection with the PEMC;

Figure 8.5.8.2.2-1: PIN client decides to leave a PIN

1.	The PINE decides to leave a PIN, and sends the PIN Management PINE leave from PIN request to PEMC to leave the PIN. The request includes the security credentials of the UE or PIN client received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID and PIN ID.

2.	The PEMC authorizes the request, and decides to remove a PIN client from a PIN which indicated by PIN client ID or UE GPSI. The authorization procedure is defined in clause 8.10.

3.	The PEMC sends the PIN Management PINE leave from PIN response to PINE to notify that the PIN client is not the member of the PIN anymore.

4-5.	The PEMC sends the PIN status notify Request to the PEGC, PIN server and PIN elements that subscribed for PIN status notification which contains the details of the PIN client that requested to leave the PIN. The details of the PINE include PIN client ID, GPSI and etc.

6-8.	The PEMC/PEGC/PIN server updates the dynamic profile information of the PIN to remove the details of the PIN client that requested to leave the PIN. The PEGC disables the access control information for this PINE. The PEGC stops relaying traffic to the PINE.

* * * End of Change * * * *