3GPP TSG-SA WG6 Meeting #64	S6-245134

Orlando, USA, 18th – 22nd November 2024

Source:	ZTE Corporation

Title:	Limitation Enhancement of ML Model Retrieval Messages

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Yang Li (li.yang1226@zte.com.cn)

1. Introduction

This paper introduces limitations on the successful response IE and failure response IE in messages related to ML model retrieval.

2. Reason for Change

At least one of the successful response IE or failure response IE is required in messages related to ML model retrieval. Therefore, the specifications for these IEs should include the corresponding limitations in the message description.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.3.0.

* * * First Change * * * *

8.2.3.2	ML model retrieval response

Table 8.2.3.2-1 describes information elements for the ML model retrieval response from the AIMLE server.

Table 8.2.3.2-1: ML model retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 1) | Indicates that the ML model retrieval request was successful. |
| List of ML models | O | List of retrieved ML models. |
| > ML model identifier | O | The ML model identifier. |
| > ML model retrieval endpoint (NOTE) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model (NOTE) | O | The ML model containing a retrieved ML model. |
| Failure response | O (NOTE 1) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: At least one of these elements is needed. NOTE 1: At least one of these elements is needed. |  |  |

* * * Second Change * * * *

8.2.3.4	ML model retrieval subscription response

Table 8.2.3.4-1 describes information elements for the ML model retrieval subscription response from the AIMLE server.

Table 8.2.3.4-1: ML model retrieval subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model retrieval request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: At least one of these elements is needed. |  |  |

* * * Third Change * * * *

8.2.3.7	ML model retrieval subscription update response

Table 8.2.3.7-1 describes information elements for the ML model retrieval subscription update response from the AIMLE server.

Table 8.2.3.7-1: ML model retrieval subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model retrieval subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: At least one of these elements is needed. |  |  |

* * * Forth Change * * * *

8.2.3.9	ML model retrieval unsubscribe response

Table 8.2.3.9-1 describes information elements for the ML model retrieval unsubscribe response from the AIMLE server.

Table 8.2.3.9-1: ML model retrieval unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model retrieval unsubscribe request was successful. |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: At least one of these elements is needed. |  |  |

* * * End of Changes * * * *