3GPP TSG-SA WG6 Meeting #64	S6-245145

Orlando, USA, 18th – 122nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0112 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add update and delete service operation of SEALDD client policy configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SEALDD client policy configuration is been approved, but the SEALDD client policy configuration update and delete need to be added. Secondly, the Expiration time is added in the SEALDD client policy configuration request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add update and delete service operation of SEALDD client policy configuration. Add Expiration time in the SEALDD client policy configuration request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEALDD client policy configuration can not be updated and deleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.10.1, 9.10.2.4，9.10.3.7， 9.10.3.8， 9.10.4.5 New clause: 9.10.2.5， 9.10.2.6， 9.10.3.9， 9.10.3.10， 9.10.3.11， 9.10.3.12， 9.10.4.6， 9.10.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Based on S6-244633 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.10	SEALDD policy configuration

9.10.1	General

The following clauses specify procedures, information flow for SEALDD policy configuration. In clause 9.10, the VAL server is a specific server for configuration SEALDD policy, and is different from the VAL server used for VAL application processing in other clauses.

Depending on the configuration, a SEALDD policy may apply to individual SEALDD flows or to multi-modal SEALDD flows.

9.10.2	Procedures

9.10.2.1	SEALDD policy configuration

Figure 9.10.2.1-1 illustrates the procedure for SEALDD policy configuration from the VAL server used for SEALDD policy configuration to the SEALDD server.

Figure 9.10.2.1-1: SEALDD policy configuration

1.  The VAL server sends the SEALDD policy configuration request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE identify, and the SEALDD policy.

2.  The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to configure the SEALDD policy.

3.  Upon successful authorization, the SEALDD server stores the SEALDD policy for later use (e.g. for bandwidth control, transmission quality guarantee) and replies to the VAL server with the SEALDD policy configuration response.

9.10.2.2	SEALDD policy configuration update

Figure 9.10.2.2-1 illustrates the procedure for SEALDD policy configuration update from the VAL server used for SEALDD policy configuration to the SEALDD server.

Figure 9.10.2.2-1: SEALDD policy configuration update

1.  The VAL server used for SEALDD policy configuration determines that the existing SEALDD policy needs to be updated, the VAL server sends the SEALDD policy configuration update request to the SEALDD server.

2. The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to update the SEALDD policy configuration.

3.  Upon successful authorization, the SEALDD server updates the SEALDD policy configuration and replies to the VAL server with the SEALDD policy configuration update response.

9.10.2.3	SEALDD policy configuration delete

Figure 9.10.2.3-1 illustrates the procedure for SEALDD policy configuration delete from the VAL server used for SEALDD policy configuration to the SEALDD server.

Figure 9.10.2.3-1: SEALDD policy configuration delete

1.	The VAL server used for SEALDD policy configuration determines that the existing SEALDD policy needs to be deleted, the VAL server sends the SEALDD policy configuration delete request to the SEALDD server.

2.	The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to delete the SEALDD policy configuration.

3.	Upon successful authorization, the SEALDD server deletes the SEALDD policy configuration and replies to the VAL server with the SEALDD policy configuration update response.

9.10.2.4	SEALDD client policy configuration request

Figure 9.10.2.4-1 illustrates the procedure for SEALDD client policy configuration request from the SEALDD server to the SEALDD client.

Pre-condition:

- The SEALDD connectivity has been established between the SEALDD client and SEALDD server.

Figure 9.10.2.4-1: SEALDD client policy configuration request

- Based on policy received from the VAL server(e.g., Multi-modal flows alignment policy), or SEALDD server determines that the policy needs to be configured to the SEALDD client, the SEALDD server sends the SEALDD client policy configuration request to the SEALDD client.

2.	The SEALDD client performs authorization check to verify whether the SEALDD server can be authorized to configure the SEALDD client policy.

3.	Upon successful authorization, the SEALDD server stores the SEALDD client policy for later use (e.g. for multi-modal flows alignment) and replies to the SEALDD server with the SEALDD client policy configuration response.

9.10.2.5	SEALDD client policy configuration update request

Figure 9.10.2.5-1 illustrates the procedure for SEALDD client policy configuration update request from the SEALDD server to the SEALDD client.

Pre-condition:

- The SEALDD connectivity has been established between the SEALDD client and SEALDD server.

Figure 9.10.2.5-1: SEALDD client policy configuration update request

- Based on policy received from the VAL server, or SEALDD server determines that the policy needs to be updated to the SEALDD client, the SEALDD server sends the SEALDD client policy configuration update request to the SEALDD client.

2.	The SEALDD client performs authorization check to verify whether the SEALDD server can be authorized to update the SEALDD client policy.

3.	Upon successful authorization, the SEALDD server updates the SEALDD client policy and replies to the SEALDD server with the SEALDD client policy configuration update response.

9.10.2.6	SEALDD client policy configuration delete request

Figure 9.10.2.6-1 illustrates the procedure for SEALDD client policy configuration delete request from the SEALDD server to the SEALDD client.

Pre-condition:

- The SEALDD connectivity has been established between the SEALDD client and SEALDD server.

Figure 9.10.2.6-1: SEALDD client policy configuration delete request

- Based on request received from the VAL server, or SEALDD server determines that the policy needs to be deleted, the SEALDD server sends the SEALDD client policy configuration delete request to the SEALDD client.

2.	The SEALDD client performs authorization check to verify whether the SEALDD server can be authorized to delete the SEALDD client policy.

3.	Upon successful authorization, the SEALDD server deletes the SEALDD client policy and replies to the SEALDD server with the SEALDD client policy configuration delete response.

9.10.3	Information flows

9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| -Identity | O | Identifier of the VAL UE or VAL user for which SEALDD policy applies |
| SEALDD policy | O (See NOTE 4) | The SEALDD policy associated with application traffic identifiers, VAL UE identity |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the event (e.g. measurement threshold) to be measured for the quality guarantee |
| > Quality optimization policy | O (See NOTE 1) | Indicates the data transmission adjustment policy (e.g., adjustment need) to be performed in SEALDD layer. |
| > Bandwidth control policy | O (See NOTE 2) | Indicate the bandwidth control preference, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL |
| > Geofence policy | O (See NOTE 3) | Indicates the geofence policy per VAL service |
| >> Geofence location | O (See NOTE 3) | Indicates the geofence location information for the mentioned VAL service |
| >>Geofence policy action | O (See NOTE 3) | Indicates the policy action like allowed or blocked |
| > temporal policy | O (See NOTE 3) | Indicates the time period for which the sealdd traffic is allowed |
| > policy expiration time | O | Indicates the validity of the policy |
| Multi-modal SEALDD policy | O (See NOTE 4) | Multi-modal SEALDD policy associated with set of individual SEALDD flows. |
| > Synchronization policy | O | Indicates the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2] |
| > Multi-modal flows alignment policy | O | Indicates the information to do the multi-modal flows alignment, e.g., Multi-modal Service ID, maximum acceptable duration for traffic flow alignment. |
| > Policy expiration time | O | Indicates the validity period of the policy |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. NOTE 3: This IE is used for the SEALDD connection establishment and data delivery, as specified in clause 9.2 NOTE 4: At least one of these IEs shall be present. |  |  |

9.10.3.2	SEALDD policy configuration response

Table 9.10.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD policy configuration.

Table 9.10.3.2-1: SEALDD policy configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| > Configuration ID | O (See NOTE) | Identifier of the SEALDD policy configuration. |
| > Expiration time | O (See NOTE) | Indicates the expiration time of the configured SEALDD policy |
| NOTE: These IEs are used for the successful case for SEALDD policy configuration request. |  |  |

9.10.3.3	SEALDD policy configuration update request

Table 9.10.3.3-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration update.

Table 9.10.3.3-1: SEALDD policy configuration update request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identifier of the SEALDD policy configuration. |
| Updated SEALDD policy | O | The updated SEALDD policy as described in Table 9.10.3.1-1. |

9.10.3.4	SEALDD policy configuration update response

Table 9.10.3.4-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD policy configuration update.

Table 9.10.3.4-1: SEALDD policy configuration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| > Expiration time | O (See NOTE) | Indicates the expiration time of the configured SEALDD policy |
| NOTE: This IEs is used for the successful case for SEALDD policy configuration update request. |  |  |

9.10.3.5	SEALDD policy configuration delete request

Table 9.10.3.5-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration delete.

Table 9.10.3.5-1: SEALDD policy configuration delete request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identifier of the SEALDD policy configuration. |

9.10.3.6	SEALDD policy configuration delete response

Table 9.10.3.6-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD policy configuration delete.

Table 9.10.3.6-1: SEALDD policy configuration delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |

9.10.3.7	SEALDD client policy configuration request

Table 9.10.3.7-1 describes the information flow from the SEALDD server to the SEALDD client for requesting SEALDD client policy configuration.

Table 9.10.3.7-1: SEALDD client policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor |
| SEALDD flow ID | O(NOTE 1) | Identity of the SEALDD flows |
| Traffic descriptor | O(NOTE 1) | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD server side used to establish SEALDD multi-modal connection |
| VAL service ID | M | Identity of the VAL service |
| UE ID | O | Identifier of specific UE or VAL user |
| Multi-modal flows alignment policy | M | Multi-modal flows alignment Policy |
| >Multi-modal Service ID | O(NOTE 2) | Identifier of Multi-modal Service |
| >Flows transmission requirement | O(NOTE 2) | Flows transmission requirement including the delay requirement, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow |
| >Expiration time | O | When the time reached, the policy will automatically expire. |
| NOTE 1: At least one of these IEs shall be present in the message. NOTE 2: At least one of these IEs shall be present in the Multi-modal flows alignment policy. |  |  |

9.10.3.8	SEALDD client policy configuration response

Table 9.10.3.8-1 describes the information flow from the SEALDD server to the VAL server for response SEALDD client policy configuration.

Table 9.10.3.8-1: SEALDD client policy configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure of the request. |
| > Configuration ID | O (See NOTE) | Identifier of the SEALDD policy configuration. |
| > Expiration time | O (See NOTE) | Indicates the expiration time of the configured SEALDD policy |

9.10.3.9	SEALDD client policy configuration update request

Table 9.10.3.9-1 describes the information flow from the SEALDD server to the SEALDD client for requesting SEALDD client policy configuration update.

Table 9.10.3.9-1: SEALDD client policy configuration update request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identifier of the SEALDD client policy. |
| Updated SEALDD client policy | O | The updated SEALDD client policy. |

9.10.3.10	SEALDD client policy configuration update response

Table 9.10.3.10-1 describes the information flow from the SEALDD server to the VAL server for response SEALDD client configuration policy update.

Table 9.10.3.10-1: SEALDD client policy configuration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure of the request. |

9.10.3.11	SEALDD client policy configuration delete request

Table 9.10.3.11-1 describes the information flow from the SEALDD server to the SEALDD client for requesting SEALDD client policy configuration delete.

Table 9.10.3.11-1: SEALDD client policy configuration delete request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identity of the SEALDD client policy. |

9.10.3.12	SEALDD client policy configuration delete response

Table 9.10.3.12-1 describes the information flow from the SEALDD server to the VAL server for response SEALDD client policy Configuration delete.

Table 9.12.3.10-1: SEALDD client policy configuration delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure of the request. |

9.10.4	APIs

9.10.4.1	General

Table 9.10.4.1-1 illustrates the APIs exposed by SEALDD server for SEALDD policy configuration.

Table 9.10.4.1-1: List of SEALDD server APIs for policy configuration

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_PolicyConfiguration | Request | Request/Response | VAL server |
|  | Update |  |  |
|  | Delete |  |  |
| Sdd_ClientPolicyConfiguration | Request | Request/Response | SEALDD server |

9.10.4.2	Sdd_PolicyConfiguration operation

API operation name: Sdd_PolicyConfiguration_Request

Description: The consumer requests for one time for SEALDD policy configuration.

Inputs: See clause 9.10.3.1.

Outputs: See clause 9.10.3.2.

See clause 9.10.2.1 for details of usage of this operation.

9.10.4.3	Sdd_PolicyConfiguration update operation

API operation name: Sdd_PolicyConfiguration_Update

Description: The consumer requests for one time for SEALDD policy configuration update.

Inputs: See clause 9.10.3.3.

Outputs: See clause 9.10.3.4.

See clause 9.10.2.2 for details of usage of this operation.

9.10.4.4	Sdd_PolicyConfiguration delete operation

API operation name: Sdd_PolicyConfiguration_Delete

Description: The consumer requests for one time for SEALDD policy configuration deletion.

Inputs: See clause 9.10.3.5.

Outputs: See clause 9.10.3.6.

See clause 9.10.2.3 for details of the usage of this operation.

9.10.4.5	Sdd_ClientPolicyConfiguration operation

API operation name: Sdd_ClientPolicyConfiguration_Request

Description: The consumer requests for one time for SEALDD Client policy configuration.

Inputs: See clause 9.10.3.7.

Outputs: See clause 9.10.3.8.

See clause 9.10.2.4 for details of the usage of this operation.

9.10.4.6	Sdd_ClientPolicyConfiguration_Update operation

API operation name: Sdd_ClientPolicyConfiguration_Update

Description: The consumer requests for SEALDD Client policy configuration update.

Inputs: See clause 9.10.3.9.

Outputs: See clause 9.10.3.10.

See clause 9.10.2.5 for details of the usage of this operation.

9.10.4.7	Sdd_ClientPolicyConfiguration_Delete operation

API operation name: Sdd_ClientPolicyConfiguration_Delete

Description: The consumer requests for SEALDD Client policy configuration delete.

Inputs: See clause 9.10.3.11.

Outputs: See clause 9.10.3.12.

See clause 9.10.2.6 for details of the usage of this operation.