3GPP TSG-SA WG6 Meeting #64	S6-245150

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Huawei, Hisilicon

Title:	Gap for the SEAL services within MNO trusted domain

Spec:	3GPP TR 23.700-35

Agenda item:	8.9

Document for:	Approval

Contact:	Yang Yanmei (yangyanmei@huawei.com)

1. Introduction

In existing TS 23.434, the deployment case where SEAL services deployed by MNO is covered. But the architectures and solutions for some SEAL service are not correctly support such case.

This P-CR is to address the gap about the un-agligned description for the MNO deployed SEAL service.

2. Reason for Change

According to 3GPP TS23.501 and TS23.502, a MNO‘s AF(trusted AF) can invoke all the APIs exposed by NF(s) from SBA interface. SEAL sever(s) deployed by MNO exactly taking the role of a trusted AF should be able to invoke service APIs exposed by 3GPP core network functions directly over the Core network SBA.

However, in existing SEAL specification, the SEAL service server invoking 5GC APIs via Core network SBA is missing in some SEAL service. For example, in clause 9.2.2 of TS23.434, the Location management server is represented as the following:

“Figure 9.2.2-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [10].

Figure 9.2.2-3: Architecture for location management utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 9.2.2-4 illustrates the service-based representation for utilization of the Core Network (5GC, EPC) northbound APIs via CAPIF.

Figure 9.2.2-4: Utilization of Core Network Northbound APIs via CAPIF – service based representation

The Location management server acts as authorized API invoker to consume services from the Core Network (5GC, EPC) northbound API entities like SCEF, NEF, SCEF+NEF which act as API Exposing Function as specified in 3GPP TS 23.222 [6].”

So the overall revisit to SEAL specification for the gap and corrections should be addressed in the TR.

4. Proposal

It is proposed to agree the following changes to TR 23.700-35 V0.1.0.

* * * First Change * * * *

minimum parameters principle

* * * Next Change * * * *

4.x	Existing SEAL service: Generic

4.x.1	Analysis

According to 3GPP TS23.501 and TS23.502, a MNO‘s AF(trusted AF) can invoke all the APIs exposed by NF(s) from SBA interface. SEAL sever(s) deployed by MNO exactly taking the role of a trusted AF should be able to invoke service APIs exposed by 3GPP core network functions directly over the Core network SBA.

However, in existing SEAL specification, the SEAL service server invoking 5GC APIs via Core network SBA is missing in some SEAL service. For example, in clause 9.2.2 of TS23.434, the Location management server invoke 5GC location API from NEF/SCEF only, which may cause mis-leading of LMS deployment. The similar gap may also exist in other parts.

4.x.2	Gaps

Gap#4.x.2-1: The overall revisit to SEAL specification for lacking of architeture and solutions for supporting the SEAL services deployed by MNO as trusted AF is required.

* * * Next Change * * * *