3GPP TSG-SA WG6 Meeting #64	S6-245165

Orlando, USA, 18th – 22th Nov 2024	(revision of S6-244729)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0330 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Satellite access with discontinuous coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-01 v1.2.0, the KI#3 for Supporting Satellite access with discontinuous coverage has been concluded as follows: Solution #AE3 resolves open issue about application enabler (SEAL Configuration management server) obtaining the satellite coverage availability information and to retrieve unavailability period for UE (either from UE or from the 5GS), for supporting satellite discontinuous coverage. Further the solution also resolves open issue about leveraging UE unavailability period information for minimizing the service impact or adjusting the application behaviour by the SEALDD server. There is dependency with SA2 for obtaining exposure of loss of coverage event information. Solution #AE3 can be considered for normative work. So it’s needed to specify the functions, procedures and information flows for the KI#3 and Sol#AE3 in TS for the normative work. ======================================================== New changes: The approved clause 21.x.2.1 introduces the application enabler provisioning the SCAI(satellite coverage availability information) to the VAL UE. However, the VAL UE or the VAL server also can request the SCAI information from the network automatically to minimize the service impact in case the network/application enablers can’t provide the SCAI or they can’t receive the SCAI. That’s the reason why we need to add a new clause 21.x.2.2 to introduce such scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add functions of obtaining the satellite coverage availability information and unavailability period for UE. ======================================================== New changes: - Add clause 21.x.2.2 for new procedures - Add clause 21.x.3.3 and 21.x.3.4 for new information flows - Adjust the number for clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The service will be impacted when the UE accesses the network via the satellites. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New{21,21.1,21.x,21.x.1,21.x.2,21.x.2.1,21.x.2.2,21.x.3,21.x.3.1,21.x.3.2, 21.x.3.3,21.x.3.4} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

21 	SEAL services over Satellite Access

21.1 	General

This clause provides clarifications on the procedures and information flows for enabling SEAL services using satellite access.

* * * Next Change * * * *

21.x	Satellite coverage availability information configuration

21.x.1	General

The following clauses describe the satellite coverage availability information provisioning procedure.

21.x.2	Procedures

21.x.2.1	Procedures of the satellite coverage availability information provisioning

The procedure for VAL UE obtaining the satellite coverage availability information configuration for a particular location of UE is illustrated in figure 21.x.2.1-1.

Pre-conditions:

-	The VAL UE has registered to the 3GPP network via satellite access as defined in clause 4.2.2.2.2 of 3GPP TS 23.502 [11].

-	The configuration management server may interact with the SEAL location management server to obtain the VAL UE’s information (e.g., location).

Figure 21.x.2.1-1: The satellite coverage availability information provisioning

1.	The configuration management server obtains the satellite coverage availability information locally or from the satellite service provider, and in order to provide accurate information to the UE, the VAL UE’s information (e.g. location data) may also be considered. If the configuration management server receives the different kinds of satellite information from different satellite information providers for the same satellite, it may consolidate them in a unified format. The example of satellite coverage availability information can refer to the Annex Q of 3GPP TS 23.501 [10].

2.	The configuration management server sends the satellite coverage availability information configuration request to the configuration management client to indicate whether the coverage is available for a particular location and time when the VAL UE uses the satellite access.

3.	The configuration management client acknowledges the configuration management server if it supports the satellite access.

21.x.2.2	Procedures of UE requesting the the satellite coverage availability information(SCAI)

The procedure for VAL UE/VAL server requesting and obtaining the satellite coverage availability information from the application enabler is illustrated in figure 21.x.2.2-1.

Pre-conditions:

-	The VAL UE has registered to the 3GPP network via satellite access as defined in clause 4.2.2.2.2 of 3GPP TS 23.502 [11] and has not received the satellite coverage availability information from the network yet.

-	The configuration management server has obtained the satellite coverage availability information as described in clause 21.x.2.1.

Figure 21.x.2.2.1-1: VAL UE requests the satellite coverage availability information

- The consumer (VAL UE/configuration management client, VAL server) sends Get satellite coverage availability information (SCAI) request to the configuration management server to require satellite coverage availability information when it is using the satellite access. The request may contain the serving satellite ID, the current UE location data, etc.

- The configuration management server authorizes the request. If the request is granted, it checks the requesting SCAI is available. If it’s not available, the configuration management server obtains the SCAI as described in step 1 of clause 21.x.2.1.

- The configuration management server replies the satellite coverage availability information to the consumer (VAL UE/configuration management client, VAL server) to guide them how to minimize the service impact (e.g. indicate when to trigger the UL/DL service flow if the VAL UE is accessing the satellite). And the VAL UE may retrieve the UE unavailability period (e.g., unavailability period duration or the start time for the unavailability period) as defined in clause 5.4.1.4 of 3GPP TS 23.501[10] based on the received satellite coverage availability information.

* * * Next Change * * * *

21.x.3	Information flows

21.x.3.1	Satellite coverage availability information configuration request

Table 21.x.3.1-1 describes the information flow from the configuration management server to the configuration management client to configure the satellite coverage availability information.

Table 21.x.3.1-1: Satellite coverage availability information configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Satellite coverage availability information(NOTE) | M | Indicates when the satellite coverage is available for a particular location and time. |
| NOTE: The example of satellite coverage availability information can refer to the Annex Q of 3GPP TS 23.501 [10]. |  |  |

* * * Next Change * * * *

21.x.3.2	Satellite coverage availability information configuration response

Table 21.x.3.2-1 describes the information flow from the configuration management client to the configuration management server for the response of the satellite coverage availability information configuration request.

Table 21.x.3.2-1: Satellite coverage availability information configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| Result | M | Indicates the request result. |

* * * Next Change * * * *

21.x.3.3	Get satellite coverage availability information request

Table 21.x.3.3-1 describes the information flow from the configuration management client or VAL server to the configuration management server to obtain the satellite coverage availability information.

Table 21.x.3.3-1: Get satellite coverage availability information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identities of the requesting VAL user or VAL UE or VAL server. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Satellite ID | O | Indicates the serving satellite ID for the VAL UE. |
| Location | O | Indicates the current location data for the VAL UE. |

21.x.3.4	Get satellite coverage availability information response

Table 21.x.3.4-1 describes the information flow from the configuration management server to the configuration management client or VAL server to notify the satellite coverage availability information.

Table 21.x.3.4-1: Get satellite coverage availability information response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Satellite coverage availability information(NOTE) | M | Indicates when the satellite coverage is available for a particular location and time. |
| NOTE: The example of satellite coverage availability information can refer to the Annex Q of 3GPP TS 23.501 [10]. |  |  |

* * * End of Change * * * *