3GPP TSG-SA WG6 Meeting #64	S6-245176x

Orlando, USA, 18th – 22nd November 2024	(revision of S6-244740)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0040 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE RAT connectivity analytics for non terrestrial access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-01 studied the below open issue as part of Key issue#1 and captured the new analytics required for satellite access : - Whether and what new analytics required for the satellite access for providing a better service experience by the existing application enablement servers or application servers. And identified the new analytics (UE RAT connectivity analytics) required for satellite access and defined the procedures and information flows for supporting the same by the ADAE enabler. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR brings the procedure and information flows for UE RAT connectivity analytics as part of TS 23.436 Revision changes : Creation of new section specific to satellite access and fit in the content accordingly. Revision: This CR updates the analytics notification to add more details so that stage 3 can implement it accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service experience over the satellite access will be impacted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All new clauses: X, X.1, X.2,X.2.1, X.2.2,X.2.3, X.2.3.1, X.2.3.2, X.2.3.3, X.2.3.4, X.2.3.5, X.2.4, X.2.4.1, X.2.4.2, X.2.4.3, X.2.4.3.1, X.2.4.3.2, X.2.4.3.3, X.2.5, X.2.5.1, X.2.5.2, X.2.5.3, X.2.5.3.1, X.2.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# X	Analytics related to satellite access

## X.1	General

This clause describes the procedures and information flows for analytics related to satellite access.

## X.2	Support for UE RAT connectivity analytics

### X.2.1	General

This clause describes the procedure for supporting UE RAT connectivity analytics for satellite access. The ADAES service consumer can subscribe and receive notifications about UE RAT connectivity analytics events. This analytics helps to predict the type of satellite RAT that a UE will latch onto on a particular location and/or particular time and helps the analytics consumer to decide on the actions to improve the service experience over the satellite access.

### X.2.2	Procedure

Figure X.2.2-1 illustrates the procedure for UE RAT connectivity analytics.

Pre-conditions:

1.	The ADAES is registered and capable of interacting with 5GS to UE mobility analytics.

Figure X.2.2-1: Procedure for supporting UE RAT connectivity analytics

1.	The analytics consumer (e.g.VAL server) makes a subscription request to ADAE server for UE RAT connectivity prediction/stats, including an analytics event ID for UE RAT Connectivity analytics. The request may include also the target area, a target VAL service, or a VAL UE, or group of UEs of the VAL service, time validity. If the VAL UEs are provided by the VAL server, this request may also include the expected route or a set of waypoints for the UEs of the VAL application.

2.	The ADAE server sends a UE RAT connectivity analytics subscription response as an ACK to the VAL server.

3.	The ADAE server subscribes to the SEAL Location management server to get the location information of the VAL UE along with the RAT type. It can set the trigger criteria of receiving the location information of the UE whenever UE RAT type changes as specified in 3GPP TS 23.434 [2] clause 9.3.5.

4.	The ADAE server subscribes for NWDAF UE mobility analytics per VAL UE (for all the VAL UEs) and gets notification on the per UE location/mobility analytics based on 3GPP TS 23.288 [4] clause 6.7.2.

5. 	If the data is collected from multiple sources, the ADAES combines or correlates the data/analytics from steps 3-4 and stores the data into A-ADRF if needed.

6.	The ADAE server optionally requests UE RAT connectivity historical analytics/data from A-ADRF for the corresponding VAL UEs.

7.	Based on the request, the ADAE server receives UE RAT connectivity historical analytics/data from A-ADRF for the corresponding VAL UEs.

8.	The ADAE server abstracts or correlates the data/analytics from steps 5-6 and provides analytics on the UE RAT connectivity for the target VAL application.

9.	The ADAE server sends the UE RAT connectivity analytics notification to the consumer.

### X.2.3	Information flows

#### X.2.3.1	UE RAT connectivity analytics subscription request

Table X.2.3.1-1 describes information elements for the UE RAT connectivity analytics subscription request from the VAL server/Analytics consumer to the ADAE server.

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server. |
| Analytics ID | M | The identifier of the UE RAT connectivity analytics event. |
| VAL UE ID(s) or Group ID | M | The identity of the VAL UE(s) or group of UEs for which the analytics subscription applies |
| VAL service ID | O | The identifier of the VAL service for which location accuracy analytics is requested. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| UE mobility / route information | O | Information on the target UE or group UE mobility including the expected route/set of waypoints. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |

#### X.2.3.2	UE RAT Connectivity analytics subscription response

Table X.2.3.2-1 describes information elements for the UE RAT connectivity analytics subscription response from the ADAE server to the VAL server/Analytics consumer.

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement) |

#### X.2.3.3	UE RAT Connectivity data retrieval request

Table X.2.3.3-1 describes information elements for the UE RAT connectivity data retrieval request from the ADAE server to the A-ADRF.

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Analytics ID | M | The identifier of the analytics event i.e. UE RAT connectivity analytics |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the data/analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Reporting configuration | O | The configuration for data reporting. This requirement may include e.g. the frequency of reporting (periodic), the reporting periodicity in case of periodic, and reporting thresholds, whether data abstraction is needed or not. |
| Data collection requirements | O | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

#### X.2.3.4	UE RAT Connectivity data retrieval response

Table X.2.3.4-1 describes information elements for the UE RAT connectivity data retrieval response from the A-ADRF to the ADAE server.

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Analytics Output | M | The reported analytics for the UE RAT connectivity, which can be in form of offline stats/historical data for a specific VAL service or for particular UE(s) or group of UEs |

#### X.2.3.5	UE RAT Connectivity analytics notification

Table X.2.3.5-1 describes information elements for the UE RAT connectivity analytics notification from the A-ADRF to the ADAE server.

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event (UE RAT connectivity) |
| VAL UE ID(s) | O | The identity of the VAL UE(s) for which the analytics applies. |
| VAL service ID | O | The identifier of the VAL service for which the analytics applies. |
| Analytics Output | M | The analytics output which can be predictive or statistical parameter |
| > RAT type prediction for given waypoints | O | A predicted or expected RAT type changes for a particular VAL service or UEs for the given waypoints. |
| >> Applicable area | O | The set of location co-ordinates or waypoints for which the analytics output is applicable. |
| >> Applicable time period | O | The time period that the analytics applies to. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| > RAT type prediction for a given time period | O | A predicted or expected RAT type changes for a particular VAL service or UEs for the given time period. |
| >> Applicable time period | O | The time period that the analytics applies to. |
| >> Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

### X.2.4	ADAE server APIs

#### X.2.4.1	General

This clause provides the APIs provided by ADAES related to satellite access.

#### X.2.4.2	ADAE server APIs

Table X.2.4.2-1 illustrates the ADAE server APIs related to satellite access.

Table X.2.4.2-1: List of ADAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ADAE_UE_RAT_connectivity_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |

#### X.2.4.3	SS_ADAE_UE_RAT_connectivity_analytics API

##### X.2.4.3.1	General

API description: This API enables the analytics consumer (e.g., the VAL server) to communicate with the ADAE server for requesting or subscribing to UE RAT connectivity analytics.

##### X.2.4.3.2	Subscribe

API operation name: UE_RAT_connectivity_analytics_subscribe

Description: The consumer subscribes to the ADAE server for UE RAT connectivity analytics.

Inputs: See clause X.2.3.1.

Outputs: See clause X.2.3.2.

See clause X.2.2 for details of usage of this operation.

##### X.2.4.3.3	Notify

API operation name: UE_RAT_connectivity_analytics_notify

Description: The consumer is notified by the ADAE server on the Server-to-server performance analytics.

Inputs: -

Outputs: See clause X.2.3.5.

See clause X.2.2 for details of usage of this operation.

### X.2.5	A-ADRF APIs

#### X.2.5.1	General

This clause provides the APIs provided by A-ADRF related to satellite access.

#### X.2.5.2	A-ADRF APIs

Table X.2.5.2-1 illustrates the A-ADRF APIs related to satellite access.

Table X.2.5.2-1: List of A-ADRF APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ADRF_UE_RAT_connectivity_analytics | Get | ADAES | Request/Response |

#### X.2.5.3	SS_AADRF_UE RAT connectivity analytics API

##### X.2.5.3.1	General

API description: This API enables the ADAE server to communicate with the A-ADRF to request UE RAT connectivity analytics of VAL UEs.

##### X.2.5.3.2	Get

API operation name: UE_RAT_connectivity_analytics_Data_Get

Description: The consumer is receiving offline UE RAT connectivity analytics/data from A-ADRF.

Inputs: See clause X.2.3.3.

Outputs: See clause X.2.3.4.

See clause X.2.2 for details of usage of this operation.