3GPP TSG-SA6 Meeting #64	S6-245186

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0138 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of reference points in 23.289 Rel-19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hytera Communications Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In comparison to the architecture outlined in TS 23.247 (V18.7.0), the reference points "Nmb4" and "Nmb6," as described in the functional module sections for MCPTT/MCVideo/MCData in TS 23.289, do not exist. The "Nmb4" reference point should be updated to "Nmb8," while "Nmb6" should be revised to "Nmb10 or N33 + Nmb5". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace “Nmb4” with “Nmb8”. Replace “Nmb6” with “Nmb10 or N33+Nmb5”. Remove “Nmb6” in the MCPTT/MCVideo/MCData functional model figures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description may cause confusion to the readers and downstream groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.5.1, 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4	MCPTT functional model description

5.4.1	On-network functional model

Figure 5.4.1-1 shows the functional model for the application plane for an MCPTT system using the 5GS.

Figure 5.4.1-1: MCPTT functional model for application plane

In the functional model shown in figure 5.4.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.379 [6] applies.

-	MCPTT-1, uses the 5G-GC1 reference point as described in clause 4.7 and fulfils the requirements of the 5G-GC1 reference point for MCPTT.

-	The description of the MCPTT-4 and MCPTT-7 reference points in 3GPP TS 23.379 [6] applies considering that it utilizes the N6 reference point defined in 3GPP TS 23.501 [7].

-	The description of the MCPTT-5 reference point in 3GPP TS 23.379 [6] applies considering that it exists between the MCPTT server and the 5GS. It is used for resource management of MCPTT sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], 3GPP TS 23.503 [9] and 3GPP TS 23.247 [15].

-	MCPTT-5, utilizing Rx reference point or N5 reference point, may be used when the MCPTT service provider directly interacts with operator's relevant 5GS network function for QoS control for both unicast PDU sessions and MBS sessions (if dynamic PCC is applicable).

-	MCPTT-5, utilizing N33 reference point, may be used when the MCPTT service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	The MCPTT-6 reference point, which exists between the MCPTT server and the 5GS, is used to create an MBS session obtaining multicast or broadcast resources for MCPTT application usage.

-	The MCPTT-6 reference point utilizes Nmb13 reference point when the MCPTT service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCPTT service provider domain.

-	The MCPTT-6 reference point utilizes Nmb10 reference point when MBSF is used.

-	The MCPTT-6 reference point utilizes N33 reference point when the MCPTT service provider is limited by the operational agreement for QoS control, i.e. indirect interaction with operator's 5GS network functions for QoS control is only allowed.

-	The MCPTT-6 reference point utilizes Nmb10 or N33+Nmb5 reference point when MCPTT service provider interacts with the Joint BM-SC, MBSF and MBSTF entity to facilitate interworking with LTE.

-	For MCPTT-8 reference point, the reference point definition in 3GPP TS 23.379 [6] applies. The MCPTT-8 reference point utilizes the N6mb reference point according to 3GPP TS 23.247 [15].

-	The MCPTT-8 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15] when MBSTF is used.

-	For interworking with LTE via Joint BM-SC, MBSF and MBSTF entity, the MCPTT-8 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15].

-	For MCPTT-9 reference point, the reference point definition in 3GPP TS 23.379 [6] applies. The MCPTT-9 reference point utilizes the N6mb reference point according to 3GPP TS 23.247 [15].

-	The MCPTT-9 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15] when MBSTF is used.

-	For interworking with LTE via Joint BM-SC, MBSF and MBSTF entity, the MCPTT-9 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15].

5.5	MCVideo functional model description

5.5.1	On-network functional model

Figure 5.5.1-1 shows the functional model for the application plane for an MCVideo system using the 5GS.

Figure 5.5.1-1: MCVideo functional model for application plane

In the functional model shown in figure 5.5.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.281 [4] applies.

-	MCVideo-1, uses the 5G-GC1 reference point as described in clause 4.7 and fulfils the requirements of the 5G-GC1 reference point for MCVideo.

-	The description of the MCVideo-4 and MCVideo-7 reference points in 3GPP TS 23.281 [4] applies considering that it utilizes the N6 reference point defined in 3GPP TS 23.501 [7].

-	The description of the MCVideo-5 reference point in 3GPP TS 23.281 [4] applies considering that it exists between the MCVideo server and the 5GS. It is used for resource management of MCVideo sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], 3GPP TS 23.503 [9] and 3GPP TS 23.247 [15].

-	MCVideo-5, utilizing Rx reference point or N5 reference point, may be used when the MCVideo service provider directly interacts with operator's relevant 5GS network function for QoS control for both unicast PDU sessions and MBS sessions (if dynamic PCC is applicable).

-	MCVideo-5, utilizing N33 reference point, may be used when the MCVideo service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	The MCVideo-6 reference point, which exists between the MCVideo server and the 5GS, is used to create an MBS session obtaining multicast or broadcast resources for MCVideo application usage.

-	The MCVideo-6 reference point utilizes Nmb13 reference point when the MCVideo service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCVideo service provider domain.

-	The MCVideo-6 reference point utilizes Nmb10 reference point when MBSF is used.

-	The MCVideo-6 reference point utilizes N33 reference point when the MCVideo service provider is limited by the operational agreement for QoS control, i.e. indirect interaction with operator's 5GS network functions for QoS control is only allowed.

-	The MCVideo-6 reference point utilizes Nmb10 or N33+Nmb5 reference point when MCVideo service provider interacts with the Joint BM-SC, MBSF and MBSTF entity to facilitate interworking with LTE.

-	For MCVideo-8 reference point, the reference point definition in 3GPP TS 23.281 [4] applies. The MCVideo-8 reference point utilizes the N6mb reference point according to 3GPP TS 23.247 [15].

-	The MCVideo-8 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15] when MBSTF is used.

-	For interworking with LTE via Joint BM-SC, MBSF and MBSTF entity, the MCVideo-8 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15].

-	For MCVideo-9 reference point, the reference point definition in 3GPP TS 23.281 [4] applies. The MCVideo-9 reference point utilizes the N6mb reference point according to 3GPP TS 23.247 [15].

-	The MCVideo-9 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15] when MBSTF is used.

-	For interworking with LTE via Joint BM-SC, MBSF and MBSTF entity, the MCVideo-9 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15].

5.6	MCData functional model description

5.6.1	On-network functional model

Figure 5.6.1-1 shows the generic functional model for the application plane for an MCData system using the 5GS.

Figure 5.6.1-1: Generic MCData functional model for application plane

In the functional model shown in figure 5.6.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.282 [5] applies.

-	MCData-cap-1, uses the 5G-GC1 reference point as described in clause 4.7 and fulfils the requirements of the 5G-GC1 reference point for MCData.

-	The description of the MCData-5 reference point in 3GPP TS 23.282 [5] applies considering that it exists between the MCData server and the 5GS. It is used for resource management of MCData sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], 3GPP TS 23.503 [9] and 3GPP TS 23.247 [15].

-	MCData-5, utilizing Rx reference point or N5 reference point, may be used when the MCData service provider directly interacts with operator's relevant 5GS network function for QoS control for both unicast PDU sessions and MBS sessions (if dynamic PCC is applicable).

-	MCData-5, utilizing N33 reference point, may be used when the MCData service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	The respective functional models supporting MCData capabilities (e.g., SDS, FD, DS, IPcon) over unicast transmissions along with the corresponding reference points (i.e., MCData-cap-1 to MCData-cap-n) described in 3GPP TS 23.282 [5] also apply when the 5G system is used.

-	The MCData-6 reference point, which exists between the MCData server and the 5GS, is used to create an MBS session obtaining multicast or broadcast resources for MCData application usage.

-	The MCData-6 reference point utilizes Nmb13 reference point when the MCData service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCData service provider domain.

-	The MCData-6 reference point utilizes Nmb10 reference point when MBSF is used.

-	The MCData-6 reference point utilizes N33 reference point when the MCData service provider is limited by the operational agreement for QoS control, i.e. indirect interaction with operator's 5GS network functions for QoS control is only allowed.

-	The MCData-6 reference point utilizes Nmb10 or N33+Nmb5 reference point when MCData service provider interacts with the Joint BM-SC, MBSF and MBSTF entity to facilitate interworking with LTE.

-	The MCData-6 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15] when MBSTF is used.

-	For interworking with LTE via Joint BM-SC, MBSF and MBSTF entity, the MCData-6 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15].

-	The respective functional models supporting MCData capabilities (e.g., SDS, FD, DS, IPcon) over eMBMS transmissions along with the corresponding reference points (i.e., MCData-cap-1 to MCData-cap-n) described in 3GPP TS 23.282 [5] also apply when the 5G MBS is used.

# * * * * End of changes * * * *