3GPP TSG-SA WG6 Meeting #64	S6-245191

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Convida Wireless LLC

Title:	AIMLE in edge data networks

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, Liu.Lu@convidawireless.com

1. Introduction

Add AIMLE service support in edge data networks.

2. Reason for Change

The conclusion for key issue #1 in TR 23.700-82 specified that solution #23 will be considered for normative work to support AIMLE services in edge computing scenarios. AIML operations can be offloaded to UEs in the network and managed by AIMLE servers hosted on edge data networks. In such a scenario, UEs that are mobile may need assistance from AIMLE servers on the EDNs to manage service continuity issues when UEs cross service area boundaries during AIML operations. Multiple AIMLE servers may communicate with each other using the AIML-E interface.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[7]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

…

[y]	3GPP TS 23.558: “Architecture for enabling Edge Applications”.

* * * Next Change * * * *

## 8.x	AIMLE service in edge data networks

### 8.x.1	General

AIMLE servers can be deployed at edge data networks to support AIML operations performed by AIMLE clients spread across multiple edge service areas. Each edge AIMLE server manages the AIMLE clients within its service area. Service functions provided by the edge enabler layer (e.g. service continuity) can be leveraged where an edge AIMLE server functions as an edge application server (EAS). On top of the edge enabler layer procedures, the request splitting and context transfer procedure between AIMLE servers (over AIML-E reference point) are defined in this clause.

### 8.x.2	Procedure

Pre-conditions:

1.	The central AIMLE server has been pre-configured with the service area information of edge AIMLE servers.

2.	AIMLE clients have registered to the corresponding edge AIMLE server or central AIMLE server as described in clause 8.7. Each edge AIMLE server aggregates the registration information (e.g. AIMLE client profile in Table 8.7.3.1-2) of AIMLE clients within its service area and shares the aggregated information with the central AIMLE server.

Figure 8.x.2-1. AIML-E service procedures

- The central AIMLE server receives a request from an AIMLE service consumer (e.g. for ML model training in clause 8.3, for AIMLE data management assistance subscription in 8.15, and for AIML service operations control and management in clause 8.20). The central AIMLE server processes the received request and determines to split the request to multiple edge AIMLE servers if the request needs to be fulfilled by AIMLE clients across multiple edge service areas. For example, if the consumer requires AIML operations to be performed by a certain number of AIMLE clients, the request can be split to multiple edge AIMLE servers based on the number of available and capable AIMLE clients in each edge service area so that the total number of AIMLE clients meets the requirement.

- The central AIMLE server sends an AIML-E subscription request to each edge AIMLE server with the information elements listed in Table 8.x.3.1-1.

- Each edge AIMLE server processes the AIML-E subscription request from the central AIMLE server and sends a response with the information elements listed in Table 8.x.3.2-1.

- Each edge AIMLE server manages the AIMLE clients within its service area to fulfil the request and maintains AIMLE context information for the managed AIMLE clients as listed in Table 8.x.3.3-2. The AIMLE context information is generated based on the responses/notifications received from the AIMLE clients (e.g. step 7 of clause 8.12.2 or step 3 of clause 8.20.1).

- The UE associated with an AIMLE client moves from a source service area (managed by the source edge AIMLE server) to a target service area (managed by the target edge AIMLE server). The transition triggers application context relocation (ACR) procedure between the two edge AIMLE servers (as source EAS and target EAS, respectively) as specified in 3GPP TS 23.558 [y]. Particularly, the AIMLE context information is transferred between the edge AIMLE servers (EASs) as part of the application context.

- When managing the transitioned AIMLE client, the target edge AIMLE server determines whether the information received from the transitioned AIMLE client should be transferred to the source edge AIMLE server based on the AIMLE context information. For example, the target AIMLE server can forward the AIMLE service results received from the AIMLE client to the source AIMLE server if the results are only applicable to the source service area or the AIMLE client is part of a split operation pipeline formed in the source service area.

- Each edge AIMLE server aggregates responses/notifications received from the AIMLE clients (and other edge AIMLE servers) and the corresponding AIMLE context information to generate an AIML-E subscription notification (Table 8.x.3.3-1). The notification is sent to the notification target as specified in the subscription request.

### 8.x.3	Information flows

#### 8.x.3.1	AIML-E service subscription request

Table 8.x.3.1-1 shows the AIML-E subscription request that is sent by a central AIMLE server to an edge AIMLE server.

Table 8.x.3.1-1: AIML-E subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g. the central AIMLE server). |
| Original requestor identifier | O | The identifier of the original requestor (e.g. the AIMLE service consumer). |
| Original request identifier | O | The identifier of the original AIMLE service request (before splitting). |
| Split AIMLE Service Request | M | The split AIMLE service request for the edge AIMLE server, which describes the required AIML operations (tasks) to be performed by the AIMLE clients in the service area of the edge AIMLE server. |
| Notification criteria | O | Specifies the notification criteria for AIML-E subscription notifications. The notification criteria can also be indicated in the split AIMLE service request if the request includes notification criteria. |
| Notification target | M | Specifies whether the AIML-E subscription notification should be sent to the requestor (e.g. the central AIMLE server) or to the original requestor (e.g. AIMLE service consumer). |

#### 8.x.3.2	AIMLE service subscription response

Table 8.x.3.2-1 shows the response sent by the edge AIMLE server to the central AIMLE server in response to the subscription request.

Table 8.x.3.2-1: AIML-E subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the subscription request: success, fail. |
| Subscription ID | O | An identifier for the subscription only if the status is success. |

#### 8.x.3.3	AIMLE service subscription notification

Table 8.x.3.3-1 shows the notification sent by the edge AIMLE server to the central AIMLE server.

Table 8.x.3.3-1: AIML-E subscription notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | The identifier of the subscription associated with this notification. |
| Aggregated AIMLE context information | M | Aggregated context information as described in Table 8.x.3.3-2 to associate with AIMLE clients that are managed by the AIMLE server, which can include aggregated or individual AIMLE service status, AIMLE service results, AIMLE service applicability. |
| Timestamp | O | A timestamp for the notification. |

Table 8.x.3.3-2: AIMLE context information

| Information element | Status | Description |
| --- | --- | --- |
| AIMLE client ID | M | The identifier of the AIMLE client associated with the context. |
| Subscription ID | M | The AIML-E subscription ID associated with the context. |
| Current managing AIMLE server | O | The identifier of the AIMLE server that is currently managing the AIMLE client, i.e. the AIMLE server associated with the service area that the AIMLE client is currently in. |
| Previous managing AIMLE server | O | List of identifiers of AIMLE servers that have been associated with the AIMLE client. |
| AIMLE service status | O | Status of the AIML operations (task) at the AIMLE client, e.g. “active”, “paused”, “completed”, percentage of completion. |
| AIMLE service results | O | Results of the AIML operations (task) performed by the AIMLE client. |
| AIMLE service applicability | O | Applicability information of the AIML operations performed by the AIMLE client, e.g. the operation results are applicable within a certain edge service area, the operations are applicable within a certain split operation pipeline. |

* * * End of Changes * * * *