3GPP TSG-SA WG6 Meeting #64	S6-245195

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Convida Wireless LLC

Title:	Update to ML model retrieval

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, Liu.Lu@convidawireless.com

1. Introduction

This contribution proposes updates to clause 8.2 ML model retrieval.

2. Reason for Change

Clause 8.2 ML model retrieval describes the procedure enabling an AIMLE client to obtain ML models of interest. When registering to the AIMLE server, an AIMLE client provides its AIMLE client profile which includes information indicating what ML models can be supported, such as Supported AI/ML model types, Application layer AIMLE client capabilities, AIML client task capability (Table 8.7.3.1-2). After receiving a model retrieval request from a registered AIMLE client, the AIMLE server should check the AIMLE client profile to determine and select ML models that can be supported by the AIMLE client.

When changes of the AIMLE client are detected by the AIMLE server, the supported ML models for the AIMLE client may change. The AIMLE server may re-select ML models appropriate for the AIMLE client and send a ML model retrieval notification to the AIMLE client with information of the re-selected ML models.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

#### 8.2.2.2	ML model retrieval

Figure 8.2.2.2-1 illustrates the procedure for an AIMLE client or VAL server to discover and obtain ML models from the AIMLE server.

Pre-conditions:

1.	The AIMLE client or VAL server has received information (e.g., URI, IP address) related to the AIMLE Server.

2.	The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server.

Figure 8.2.2.2-1: ML model retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval request to the AIMLE server. The request includes the requestor identifier along with security credentials and may include ML model retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request.

If the requestor is authorized, the AIMLE server may determine if the requested ML model is available with the AIMLE server based on ML model retrieval filters. If the request is received from an AIMLE client that has registered with the AIMLE server, the AIMLE server can determine the ML model based on the AIMLE client profile (Table 8.7.3.1-2).

Editor’s Note: Details of AIMLE server interactions with the ML model repository are FFS.

3.	The AIMLE server sends a ML model retrieval response to the requestor. If the AIMLE server has determined ML models (e.g., locally, or from the ML repository), the response includes an indication of success and may include the ML models. Otherwise, the response includes an indication of failure and may include a reason for failure.

Upon receiving a response including ML models, the requestor can store the ML models and may provide the ML model to the VAL client(s) if the requestor is the AIMLE client.

* * * Next Change * * * *

##### 8.2.2.3.3	Notify

Figure 8.2.2.3.3-1 illustrates the ML model retrieval  notify operation between the AIMLE server and an AIMLE client or VAL server.

Pre-conditions:

1.	The AIMLE client or VAL server has subscribed for ML model retrieval with the AIMLE Server.

Figure 8.2.2.3.3-1: ML model retrieval notification

1.	An event occurs at the AIMLE server that satisfies trigger conditions for notifying a subscriber (e.g., AIMLE client or VAL server). The event may be the detection of availability of new ML model(s) (e.g., new ML model or new revision of a ML model) that satisfies the ML model retrieval filters of the subscription, or changes in the AIMLE client profile.

Editor’s Note: Details of AIMLE server interactions with the ML model repository to detect trigger conditions are FFS.

2.	The AIMLE server sends a ML model retrieval notification to the requestor indicating newly available ML models. The notification includes a subscription identity and newly available ML model(s).

* * * End of Changes * * * *