3GPP TSG-SA WG6 Meeting #63	S6-245262

Hyderabad, India, 14th – 18th October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0120 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Multi-modal flows synchronization monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SEALDD (as XR application enabler) Multi-modal flow synchronzation monitoring was concluded in TR 23.700-23 (KI#1, KI#2, sol#6). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds support for Multi-modal flows synchronization monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting new feature in Rel19 XR work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.10.3.y, (new) 9.12.2.v |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.10.3.y	SEALDD client policy configuration request

Table 9.12.3.y-1 describes the information flow from the SEALDD server to the SEALDD client for requesting SEALDD client policy configuration.

Table 9.10.3.y-1: SEALDD client policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor |
| SEALDD flow ID | O(NOTE 1) | Identity of the SEALDD flows |
| Traffic descriptor | O(NOTE 1) | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD server side used to establish SEALDD multi-modal connection |
| VAL service ID | M | Identity of the VAL service |
| UE ID | O | Identifier of specific UE or VAL user |
| Multi-modal flows alignment policy | O | Multi-modal flows alignment Policy |
| >Multi-modal Service ID | O | Identifier of Multi-modal Service |
| >Flows transmission requirement | O | Flows transmission requirement including the delay requirement, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow |
| >Flow alignment monitoring requirements | O | Flow alignment monitoring requirements including flows for which alignment reporting is required and reporting triggers (e.g., time, threshold, etc.) |
| NOTE 1: At least one of these IEs shall be present in the message. |  |  |

* * * Next Change * * * *

#### 9.12.2.v	SEALDD enabled multi-modal flow synchronization monitoring

##### 9.12.2.v.1	General

The following clauses specify procedures, information flows and APIs about SEALDD enabled multi-modal flow synchronization monitoring.

##### 9.12.2.v.2	SEALDD enabled multi-modal flow synchronization monitoring

Pre-condition:

- The SEALDD client has been configured with a multi-modal flows alignment policy including flow alignment monitoring requirements.

Figure 9.12.2.2.2-1: SEALDD enabled multi-modal flow synchronization monitoring procedure

1.	The SEALDD client detects a synchronization monitoring event. Event detection is based on the flow alignment monitoring requirements received with the multi-modal flows alignment policy and can include a time based event (e.g., a timer) for periodic reporting of the multi-modal flow alignment or a threshold based event indicating that a multi-modal flow alignment has crossed a threshold (e.g., packet buffering/flows misalignment exceeds a threshold for an given amount of time).

2.	The SEALDD client sends a SEALDD client synchronization reporting notification to the SEALDD server. The SEALDD client synchronization reporting notification includes information about the monitoring event (e.g., event, packet alignment information).

3.	Based on the synchronization report information, the SEALDD server re-evaluates if the service flow(s) need to be adjusted among the multiple associated flows in SEALDD-UU interface. If the SEALDD server determines that a new QoS value is beneficial for flow alignment, the SEALDD server determines a new QoS value.

4.	The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-UU flow) by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [6]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side.

* * * END of Change * * * *