3GPP TSG-SA WG6 Meeting #64	S6-245303

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Ericsson

Title:	Discussion Paper on Support Energy Efficiency and Energy Saving in Enablement Layer

Agenda Item:	10

Contact:	Jing Yue (jing.yue@ericsson.com)

# 1	Decision/action requested

Discuss the possible ways and guidance on enhancing the ADAES functionality.

# 2	References

[1]	3GPP TS 23.482 "Functional architecture and information flows for AIML Enablement Service"

[2]	3GPP TR 22.882 "Study on Energy Efficiency as a service criteria"

[3]	3GPP TR 22.882 "Study on Energy Efficiency as service criteria Ph 2"

[4]	3GPP TS 22.261 "Service requirements for the 5G system; Stage 1"

[5]	3GPP TR 23.700-66 "Study on Energy Efficiency and Energy Saving"

[6]	3GPP TR 26.942 "Study on Media Energy Consumption Exposure and Evaluation Framework"

[7]	3GPP TS 28.310 "Management and orchestration; Energy efficiency of 5G"

[8]	3GPP TS 28.552 "Management and orchestration; 5G performance measurements"

[9]	3GPP TS 28.554 "Management and orchestration; 5G end to end Key Performance Indicators (KPI)"

[10]	3GPP TR 28.880 "Study on energy efficiency and energy saving aspects of 5G networks and services"

# 3	Rationale

Climate change, global energy shortage, and the rising consumption of energy motivate increased energy efficiency. Energy efficiency and energy saving has been considered in many standard groups and specifications, e.g., 3GPP WGs SA1, SA2, SA4, and SA5.

SA1 has topic on energy efficiency and energy saving for Release-19 and Release-20 as described in TR 22.882 [2], TR 22.883 [3], and TS 22.261 [4]. Energy efficiency and energy saving is a critical issue in 5G. The potential to deploy systems in areas without a reliable energy source requires new methods of managing energy consumption not only in the UEs but throughout all components of the 5G system.

In SA2, Release-19 study on 5GS Enhancement for Energy Efficiency and Energy Saving is completed with solutions introduced in TR 23.700-66 [5], and the normative work is ongoing. The study in SA2 focus on potential enhancements to the 5G System that improve energy efficiency, including framework for network energy consumption exposure, enhancements to subscription and policy control to enable network energy savings as service criteria, and general 5G System enhancements which do not rule out impact on the UE.

SA4 Release-19 study on Media Energy Consumption Exposure and Evaluation Framework is ongoing, with solutions introduced in TR 26.942 [6]. The study aims to identify sustainable media metrics, architectural impacts (APIs), functional extensions required for SA4 service enablers and evaluate the feasibility of an evaluation framework to facilitate efficient energy use and energy saving for media services.

The work on the energy efficiency topics of 5G in SA5 started from Release-16, and continuously until now the ongoing in Release-19 work. As defined in TS 28.310 [7], TS 28.552 [8], TS 28.554 [9], and TR 28.880 [10], etc., SA5 focus on defining use cases, requirements, and solutions for the measurement of the energy efficiency of NG-RAN, 5G Core and network slicing and for the optimization of the energy efficiency.

However, support of energy efficiency and energy saving in application enablement layer is still missing. Thus, it is important to have a discussion on a way to support energy efficiency and energy saving in SA6 as potential Release-20 work.

# 4	Discussion

## 4.0	Energy Efficiency and Energy Saving

A general concept of energy efficiency and energy saving: The set of actions that optimize the relationship between the amount of energy consumed and the final products or services obtained. In other words, it aims to make the best possible use of energy so as not to use it unnecessarily and to save as much as possible without losing quality of life or production.

There are many examples in real-world for energy efficiency and energy saving, such as:

-	Dual-mode 5G Core manages traffic for both 4G and 5G, enabling up to 20% savings across control, user, and data layer through dynamic resource allocation and up to 15% savings in infrastructure through cloud-native architecture with shared microservices.

-	Core network capabilities can reduce energy in RAN: By implementing adaptive video bit rate optimization in the core network and reducing traffic volume in radio access networks, can save up to 10% of energy in 4G and 15% in 5G.

## Energy Efficiency and Energy Saving in SA1

The study and work in SA1 for Release-19 and Release-20 focus on energy efficiency as service criteria, as introduced in TR 22.882 [2], TR 22.883 [3] and TS 22.261 [4].

In SA1 Release-19 work and study as introduced in TR 22.882 [2] and TS 22.261 [4], best-effort service was considered with policy to limit energy consumption. That is, ensuring the services are offered as intended by service providers, network operators or subscribers, with energy constraints and no performance consequences. For example:

-	Best-effort services, policies can be defined with a maximum energy credit limit on the total amount of energy consumption according to an energy charging rate.

-	Support of different energy states of network elements and network functions, to save energy according to different working status of telecommunication equipment and manufacturing.

-	Monitoring and measurement on different levels of energy consumption and efficiency, to support energy efficiency and energy saving.

-	Exposure of information related to energy consumption and efficiency, for network internal optimization and the service adjustment for 3rd party.

-	Leverage energy-related information (e.g., energy consumption, energy efficiency), amongst others (e.g., network load), as criteria for network internal optimization actions targeting energy savings.

In Release-20 study as introduced in TR 22.883 [3], services with stringent QoS requirements are proposed to be studied and consider energy service criteria as well (e.g., user preferences or operator policies) and/or be impacted by energy availability limitations. The use cases given focus on the following aspects:

-	Information exposure of energy-related characteristics of the network for the communication service (i.e. energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions) to authorized users or authorized 3rd parties.

-	Potential dynamic adjustments of the delivered communication service from 5G system perspective (including service performance adjustments) resulting from the changes of energy-related characteristics of this service.

-	Other aspects including security, charging and privacy for the scenarios above.

## 4.2	Energy Efficiency and Energy Saving in SA2

The energy efficiency and energy saving work in SA2 Release-19 as described in TR 23.700-66 [5] focus on studying potential enhancements to the 5G System that improve energy efficiency. Three key issues are studied, in detail:

-	KI#1: Network energy related information exposure. Subject to operator policy, network energy related information (e.g. energy consumption related information, energy efficiency related information, renewable energy, and carbon emission related information) may be exposed by the network to the authorized consumers.

-	KI#2: Subscription and policy control to support energy efficiency and energy saving as service criteria. Energy related information as service criteria allows delivering services based on e.g. energy related subscriptions and policies to achieve the goal of energy saving.

-	KI#3: 5GS enhancements for network energy saving and efficiency.

The conclusions for the study of the key issues include:

-	A new network functionality is defined to collect and calculate the energy related information and exposes to the authorized consumer subject to operator's policy. The new functionality supporting the calculation of the Energy Consumption information, determines the E2E energy consumption based on energy consumption per the granularities above at serving NF (i.e. NG-RAN and UPF). The consumer NFs including 5GC NFs and /or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting. And in Release-19, only the energy-related information of user plane communication (not control plane signalling) is supported.

-	The following information is stored as part of the subscription data in the UDM/UDR: Energy saving subscription information per UE to assist the network to perform energy saving strategies for the UE. The PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions (reusing the existing parameters) based on operator policy.

-	For enhancements on NF discovery and (re-)selection based on energy related information: NF profile may be extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy. For enhancements on existing operations and procedures for energy saving and energy efficiency: UP path of PDU session may be adjusted.

## 4.3	Energy Efficiency and Energy Saving in SA4

The energy efficiency and energy saving work in SA4 Release-19 as described in TR 26.942 [6] focus on studying media energy consumption exposure and evaluation framework. Three key issues are studied, in detail:

-	KI#1: Energy-related Information exposure. Study and identify potential enhancements to the energy-related information exposure for media architectures, applications, and services.

-	KI#2: Monitoring and measurement. Consider the end-to-end chain, seeking to understand how decisions made on one end of the chain impact power consumption on other ends of the workflow. Study which UE energy-related information will be collected to measure, correlate, and optimize energy usage across the entire streaming distribution chain. And which UE entity is appropriate to measure this UE energy-related information.

-	KI#3: Evaluation framework. A characterisation framework to evaluate the energy usage/savings of multimedia standards features and proposals for relevant 3GPP scenarios is also needed to justify claims.

## 4.4	Energy Efficiency and Energy Saving in SA5

The work on the energy efficiency topics of 5G in SA5 started from Release-16, and continuously until now the ongoing in Release-19 work. SA5 focus on defining use cases, requirements, and solutions for the measurement of the energy efficiency of NG-RAN, 5G Core and network slicing and for the optimization of the energy efficiency, as defined in TS 28.310 [7], TS 28.552 [8], and TS 28.554 [9], etc. For example, TS 28.310 [7] on Energy efficiency of 5G, it is SA5 umbrella TS for energy efficiency and energy saving aspects. TS 28.552 [8] on management and orchestration of 5G performance measurements, defines Power, Energy and Environment (PEE) measurements for 5G Physical Network Functions (PNF). TS 28.554 [9] on 5G end to end Key Performance Indicators (KPI), defines energy consumption and energy efficiency KPIs for 5G Virtualized Network Functions (VNF), network slices, etc.

The energy efficiency and energy saving work in SA5 Release-19 as described in TR 28.880 [10] focus on investigates the opportunities for defining new energy related information, new Energy Consumption (EC) KPIs, new Energy Efficiency (EE) KPIs and new Energy Saving (ES) solutions for 5G. The use cases and requirements are given, related to energy related information (e.g. carbon emission, carbon emission efficiency, renewable energy factor), energy consumption, energy efficiency and energy saving.

## 4.5	Potential Energy Efficiency and Energy Saving in SA6

There is energy efficiency consideration in the existing SEAL services but without details on how to realize it. For example, in the AIMLE services introduced in TS 23.482 [1], AIML client discovery criteria contain task performance preference capabilities (e.g., Green task, energy-efficient, low costs), and the intention for support split AI/ML operation is to offload the computation-intensive and energy-intensive AI/ML stages to network endpoints. In addition, improve SA6 solutions (SEAL, CAPIF, Application Enablement Layers) with support of energy efficiency and energy saving is in need. In detail:

-	Support of energy efficiency and energy saving in SA6 northbound interfaces including the provisioning of the energy efficiency related objectives.

-	Support of energy efficiency and energy saving in the SEAL clients, e.g., adaptive reporting from UE based on application requirement, and energy efficiency and energy saving requirements.

-	Integration of the energy efficiency and energy saving solutions from other WGs (e.g. SA2) into SA6 solutions, realize service simplification and use of exposure for energy consumption data defined in other WGs (e.g. SA2, SA5). e.g., improve of CAPIF Core Function with solutions from other WGs (e.g. SA2, SA4, SA5) for energy efficiency and energy saving.

## 4.6	Detailed proposal

Based on the background information, it is proposed.

Proposal 1: Consider energy efficiency and energy saving as one of the optional Release-20 5G-advance topics. Support energy efficiency and energy saving in enablement layer, to improve SA6 solutions (SEAL, CAPIF, Application Enablement Layers) with support of energy efficiency and energy saving.

Proposal 2: Study how to support energy efficiency and energy saving in SA6 northbound interfaces including the provisioning of the energy efficiency related objectives.

Proposal 3: Study how to support of energy efficiency and energy saving in the SEAL clients.

Proposal 4: Study how to support integration of energy efficiency and energy saving solutions from other WGs (SA2, SA4, SA5) into SA6 solutions.

# 5	Conclusion

This discussion paper provided the background information and the related justifications of energy efficiency and energy saving in Release-19 and further Proposals for Release-20 in clause 4.6.

Ericsson is opened to discuss the proposals and details of energy saving and energy efficiency and energy saving applications in SA6 solutions. Ericsson encourage other companies to share their view and possible use cases.