3GPP TSG-SA WG6 Meeting #64	S6-245309

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0057 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Service Experience to Support Application Performance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ADAES |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Errors exist in the service description, procedure, request/response tables of service experience to support application performance analytics. This paper proposes to correct the errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the service description. - Corrected the errors in figure and step description of the procedure. - Corrected errors in request/response tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors exist in the service experience to support application performance analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.2.1, 8.9.2.2, 8.9.3.1, 8.9.3.3, 8.9.3.4, 8.9.3.5, 8.9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.9.2.1	Push service experience information

The ADAE client determines the service experience information based on information received from the VAL client. The service experience information includes application specific performance measurements like end-to-end response time, connection bandwidth, request rate, server availability time, etc. On request from VAL client or any other trigger conditions, the ADAE client sends the service experience report about a VAL server to the ADAE server using the mechanism defined in 3GPP TS 26.531 [3]. The information elements as defined in clause 8.9.3.1 are used by ADAE client to send the request and clause 8.9.3.2 are used by ADAE server to send the response.

For direct reporting, if data reporting session is not available, the ADAE client creates data reporting session as specified in clause 5.4 of 3GPP TS 26.531 [3]. Once data reporting session is available, the ADAE client (acting as a UE client) sends reports to ADAE server using direct reporting method as specified in clause 5.5 of 3GPP TS 26.531 [3].

The ADAE server may take further actions based on the analysis of the report as shared by the ADAE client. A service experience information from certain UEs, can trigger the ADAE server to fetch further service experience information from other UEs. The ADAE server can use the service experience information report from other UEs to determine/predict analytics.

-	If most of the UE side entities report similar service experience, then it could be the application server problem across globally.

-	If only some UEs report a bad service experience, the problem could be localized among a group of UEs.

-	If the bad service experience from only one UE, the problem is localized to the UE.

* * * Next Change * * * *

#### 8.9.2.2	Pull service experience information

The procedure can be initiated by the ADAE server upon receiving a service experience from an ADAE client, to fetch service experience information from other ADAE clients or upon receiving VAL server performance analytics request from application service provider (application server) or any other event that requires the ADAE server to determine the service experience data.

Figure 8.9.2.2-1: Pull service experience information from UE

1.	The ADAE server sends Pull service experience information request to the ADAE client. The request contains identifier of the specific VAL server and VAL service ID, for which the service experience report is required, as mentioned in Table 8.9.3.3-1.

2.	Upon receiving the Pull service experience information request from the ADAE server, the ADAE client may take user consent to send the report if the user consent is not available already.

3.	The ADAE client sends the Pull service experience information response to the ADAE server. The ADAE client instructs the Direct Data Collection Client to prioritise immediate delivery of a UE data report to the Data Collection AF as specified in clause 5.5 of 3GPP TS 26.531 [3]. The service experience contains parameters as specified in Table 8.9.3.4-1.

4.	The ADAE server uses the service experience report for derivation of VAL server performance analytics.

* * * Next Change * * * *

#### 8.9.3.1	Push service experience information request

Table 8.9.3.1-1 describes information elements for the Push service experience information request from the ADAE client to the ADAE server.

Table 8.9.3.1-1: Push service experience information request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | The identifier of the VAL UE. |
| VAL service ID | O | The identifier of the VAL service for which the service experience report applies. |
| VAL server ID | M | The identifier of the VAL server for which the service experience report is sent. |
| Timestamp | O | Time stamp of the collected report. |
| VAL service experience report | O | Information related to VAL service experience. It may include end-to-end response time, connection bandwidth, request rate, VAL server availability, etc. |

* * * Next Change * * * *

#### 8.9.3.3	Pull service experience information request

Table 8.9.3.3-1 describes information elements for the Pull service experience information request from the ADAE server to the ADAE client.

Table 8.9.3.3-1: Pull service experience information request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server for which the service experience information is requested. |
| VAL service ID | O | The identifier of the VAL service for which the service experience information is requested. |

* * * Next Change * * * *

#### 8.9.3.4	Pull service experience information response

Table 8.9.3.4-1 describes information elements for the Pull service experience information response from the ADAE client to the ADAE server.

Table 8.9.3.4-1: Pull service experience information request response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |
| VAL UE ID | M | The identifier of the VAL UE. |
| VAL service ID (NOTE) | O | The identifier of the VAL service for which the service experience report applies. |
| VAL Server ID | M | The identifier of the VAL server for which the service experience report is sent. |
| Timestamp (NOTE) | O | Time stamp of the collected report. |
| VAL service experience report (NOTE) | O | Information related to VAL service experience. It may include end-to-end response time, connection bandwidth, request rate, VAL server availability, etc. |
| NOTE: These IEs are included only if the result is success. |  |  |

* * * Next Change * * * *

#### 8.9.3.5	Configure service experience report trigger request

Table 8.9.3.5-1 describes information elements for the Configure service experience report trigger request from the ADAE server to the ADAE client.

Table 8.9.3.5-1: Configure service experience report trigger request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server specific criteria | M | List of VAL server specific criteria. |
| > VAL server ID | M | The identifier of the VAL server. |
| > Triggering Criteria | M | Information about the triggers on which the service experience is to be reported for the VAL server. |
| Common Triggering criteria | O | Information about the triggers (applicable to all VAL servers) on which the service experience is fetched. |
| Service experience measurement to monitor | O | Information about the service experience measurements which needs to be fetched and included in the report. If not present, by default end-to-end response time is measured. |
| Notification Target Address | O | The Notification target address (e.g. URL) where the notifications destined for the ADAE Server should be sent to. |

* * * Next Change * * * *

#### 8.9.3.6	Configure service experience report trigger response

Table 8.9.3.6-1 describes information elements for the Configure service experience report trigger response from the ADAE client to the ADAE server.

Table 8.9.3.6-1: Configure service experience report trigger response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |

* * * END of Changes * * * *