3GPP TSG-SA WG6 Meeting #64	S6-245311

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0059 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to A-DCCF Service on Data Collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ADAES |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Errors exist in the procedure and API clauses of the A-DCCF service on data collection. This paper proposes to correct the errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Correct errors in the procedure and API clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors exist in the procedure and API clauses of A-DCCF service on data collection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.2, 8.12.2.1, 8.12.2.2, 9.4.3, 9.4.3.1, 9.4.3.2, 9.4.3.3, 9.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.12.2	Procedure

#### 8.12.2.1	Subscribe-notify model

Figure 8.12.2.1-1 illustrates the procedure for consumer to request A-DCCF to initiate subscription for data or analytics collection. The procedure may be utilized by the consumer to update the subscription for data or analytics collection.

Pre-conditions:

1.	Consumer (ADAE server) collects data or analytics via A-DCCF from data producer.

Figure 8.12.2.1-1: Support for data collection over A-DCCF

1.	The consumer (ADAE server) sends a data collection subscription request to A-DCCF for collecting data/analytics. As specified in Table 8.12.3.2-1, the request message includes identifier of the consumer (ADAE server ID), Data Collection Event ID, and Data Collection Requirements. The request message may include the identifier of Data Producer, Analytics ID, target data producer profile criteria, process requirements, Area of Interest, Time validity, storage requirements, and notification endpoints. The consumer decides to go via A-DCCF based on internal configuration.

2.	If the data producer is not identified by the consumer, the A-DCCF determines the data procedure that can provide data/analytics. If the consumer requested storage of data/analytics in an A-ADRF but the A-ADRF ID is not provided by the consumer, or the collected data/analytics is to be stored in an A-ADRF according to configuration on the A-DCCF, the A-DCCF selects an A-ADRF to store the collected data/analytics.

3.	The A-DCCF determines whether the data/analytics requested in step 1 are already being collected. If the requested data/analytics are already being collected by a consumer, the A-DCCF adds the new consumer to the list of consumers that are subscribed for these data/analytics.

4.	If the data/analytics subscribed in step 1 is not being collected by the A-DCCF, the A-DCCF subscribes to the data producer for data/analytics.

If the data/analytics subscribed in step 1 partially matches the data/analytics that is already being collected by the A-DCCF, a modification of this subscription to the data producer would satisfy both the existing data/analytics subscriptions as well as the newly requested data/analytics, the A-DCCF requests an update of the previous subscription to the data producer. The A-DCCF adds the consumer to the list of consumers that are subscribed for these data/analytics.

5.	Upon received the data/analytics subscription request from the A-DCCF, the data producer determines whether the required data/analytics can be provided and sends data/analytics subscription response to the A-DCCF.

6.	The A-DCCF sends data/analytics subscription response to the consumer.

7.	When the required data/analytics are available, the data producer notifies the data/analytics to the A-DCCF.

8.	The A-DCCF notifies the data/analytics to all notification endpoints indicated in step 1. Data/analytics sent to notification endpoints may be processed by the DCCF upon to the request in step 1. The DCCF may store the data/analytics in the A-ADRF if requested by the consumer or if required by A-DCCF configuration.

* * * Next Change * * * *

#### 8.12.2.2	Request-response model

Figure 8.12.2.2-1 illustrates the procedure for the consumer to request A-DCCF for data or analytics collection.

Figure 8.12.2.2-1: Support for Get data/analytics over A-DCCF

1.	The consumer (ADAE server) sends a Get data/analytics request to A-DCCF for data/analytics collection. As specified in Table 8.12.3.5-1, the request message includes identifier of the consumer (ADAE server ID), and Data/Analytics Requirements. The request message may include the identifier of Data Producer, Analytics ID, target data producer profile criteria, process requirements, storage requirements, and notification endpoints. The consumer decides to go via A-DCCF based on internal configuration.

2.	The A-DCCF determines whether the data/analytics requested in step 1 are already being collected. If the requested data/analytics are already being collected by a consumer, the A-DCCF adds the new consumer to the list of consumers that are subscribed for these data/analytics.

3.	If the data/analytics requested in step 1 is not being collected by the A-DCCF, and the data producer is not identified by the consumer, the A-DCCF determines the data procedure that can provide data/analytics. If the consumer requested storage of data/analytics in an A-ADRF but the A-ADRF ID is not provided by the consumer, or the collected data/analytics is to be stored in an A-ADRF according to configuration on the A-DCCF, the A-DCCF selects an A-ADRF to store the collected data/analytics.

4.	The A-DCCF sends Get data/analytics request to the data producer for data/analytics.

5.	Upon received the data/analytics request from the A-DCCF, the data producer determines whether the required data/analytics can be provided and sends data/analytics response to the A-DCCF. The required data/analytics may be contained in the response if they are available at the data producer.

6.	The A-DCCF sends data/analytics response to the consumer. The required data/analytics may be contained in the response if get from the data producer.

* * * Next Change * * * *

### 9.4.3	SS_ADCCF_Data_Collection API

#### 9.4.3.1	General

API description: This API enables the consumer to subscribe/unsubscribe for data or analytics via the A-DCCF, be notified about data or analytics exposed by the A-DCCF, fetch the subscribed data and have data delivered via the A-DCCF. Historical data, or runtime data may be obtained using this service.

When the subscription is accepted by the A-DCCF, the consumer (ADAE server) receives from the A-DCCF an identifier (Subscription Correlation ID) allowing it to further manage (modify, delete) the subscription.

NOTE:	The definition of delete service operation is within stage 3 scope.

* * * Next Change * * * *

#### 9.4.3.2	Subscribe

API operation name: Data_Collection_Subscribe

Description: The consumer subscribes to receive data or analytics, or if the data is already requested from the A-DCCF, then the subscription is updated. The subscription includes service operation specific parameters that identify the data or analytics to be provided. The consumer may also request that data be stored in an A-ADRF. When historical data is being obtained, the consumer may specify the A-ADRF ID.

Inputs: See clause 8.12.3.2.

Outputs: See clause 8.12.3.3.

See clause 8.12.2 for details of usage of this operation.

* * * Next Change * * * *

#### 9.4.3.3	Notify

API operation name: Data_Collection_Notify

Description: The A-DCCF notifies the consumer of the requested data or analytics according to the request, or notifies of the availability of previously subscribed data or analytics when data delivery is via the A-DCCF. The A-DCCF may also notify the consumer when data or analytics is to be deleted.

Inputs: See clause 8.12.3.4.

Outputs: -.

See clause 8.12.2 for details of usage of this operation.

* * * Next Change * * * *

#### 9.4.3.4	Request

API operation name: Data_Collection_Request

Description: The consumer retrieves data or analytics from the A-DCCF.

Inputs: See clause 8.12.3.5.

Outputs: See clause 8.12.3.6.

See clause 8.12.2 for details of usage of this operation.

* * * END of Changes * * * *