3GPP TSG-SA WG6 Meeting #64	S6-245319

Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0061 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment and work completion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  |  | 11/11/2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR brings corrections and alignments to 3GPP TS 23.255 in preparation of completing Relase 19 work. The proposed correction and alignments are needed to avoid mis-interpretations and improve quality of 3GPP TS 23.255. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added missing reference; - Fixed/harmonized 3GPP References; - Added missing abbreviations; - Harmonized the term "CAA level UAV ID"; - Removed an EN not handled in Rel-19; - Harmonized the IE name “DAA application policy”; - Removed work item name “EDGEAPP”; - Fixed various editorial mistakes (e.g., many hard-spaces). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP TS 23.255 may be mis-interpreted and will be of lower quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 7.1.2.2, 7.1.3.2, 7.1.4.2, 7.1.5.2, 7.1a.2.2.2, 7.1a.2.3.2, 7.3.2.1, 7.3.2.2, 7.3.2.3, 7.3.3, 7.4.2, 7.4.2.2, 7.4.3.1, 7.6.1, 7.6.2.3, 7.7.3.1, 7.7.3.4, 7.10.2.1, 8.1, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP; Stage 1".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification, and tracking; Stage 2".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

[8]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[9]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[10]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[11]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[12]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[13]	3GPP TS 23.502:	 "Procedures for the 5G System (5GS)".

[14]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[zz]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services”.

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

DAA application policy: The configuration provided to the UAS application specific client by a UAS application specific server for handling of detect and avoid.

DAA assist capability: The functionality for the UAE layer to assist the UAV application specific layer with handling of detect and avoid during flight.

LDGS capability: A UAE client with LDGS capability offers the ground based DAA support for the UAVs in the monitoring range of the LDGS. The Local DAA Ground-Station (LDGS) can be operated by a USS and can serve one or more USSes.

Multi-USS capability: The functionality for the UAE layer to assist at change of USS during flight.

NOTE:	A UAV with Multi-USS capability can be controlled by more than one USS during a flight, but at any given time, the UAV is under the control of only one USS.

Multi-USS policy: The configuration provided by a UAS application specific server to assist at change of USS.

Remote Identification (Remote ID) of UAS: The ability of a UAS to provide identification and tracking information that can be received by other parties, to facilitate advanced operations for the UAS (such as Beyond Visual Line of Sight operations as well as operations over people), assist regulatory agencies, air traffic management agencies, law enforcement, and security agencies when a UAS appears to be flying in an unsafe manner or where the UAS is not allowed to fly.

UAS Service Supplier (USS): An entity that provides services to support the safe and efficient use of airspace by providing services to the operator / pilot of a UAS in meeting UTM operational requirements. A USS can provide any subset of functionality to meet the provider's business objectives (e.g., UTM, Remote Identification). In the scope of this specification, the term USS refers to both USS and USS/UTM.

UAV: The Uncrewed Aerial Vehicle (also called remotely piloted aircraft or drone) of a UAS.

For the purposes of the present document, the following terms given in 3GPP TS 22.125 [2] apply

Command and Control (C2) Communication

Uncrewed Aerial System (UAS)

Uncrewed Aerial System Traffic Management (UTM)

UAV controller

For the purposes of the present document, the following terms given in clause 4.2 of 3GPP TS 22.125 [2] apply

Direct C2 Communication

Network-Assisted C2 communication

UTM-Navigated C2 communication

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BVLOS	Beyond Visual Line Of Sight

CAA	Civil Aviation Authorities

C2	Command and Control

CAPIF	Common API Framework for northbound APIs

DAA	Detect And Avoid

EAS	Edge Application Server

EDN	Edge Data Network

EEC	Edge Enabler Client

EES	Edge Enablement Server

GM	Group Management

GPSI	General Public Subscription Identifier

LDGS	Local DAA Ground Station

LM	Location Management

NRM	Network Resource Management

PQI	PC5 QoS Identifier

QoS	Quality of Service

SEAL	Service Enabler Architecture Layer

UAE	UAS Application Enabler

UAS	Uncrewed Aerial System

UASS	UAS Application Specific Server

UAV	Uncrewed Aerial Vehicle

UAV-C	Uncrewed Aerial Vehicle-Controller

USS	UAS Service Supplier

UTM	UAS Traffic Management

VAL	Vertical Application Layer

* * * Next Change * * * *

#### 7.1.2.2	Information flows

The following information flows of group management service of SEAL as specified in 3GPP TS 23.434 [5] are applicable for the UAS applications:

-	Group creation request specified in clause 10.3.2.1;

-	Group creation response specified in clause 10.3.2.2;

-	Group creation notification specified in clause 10.3.2.3;

-	Group information query request specified in clause 10.3.2.4;

-	Group information query response specified in clause 10.3.2.5;

-	Group membership update request specified in clause 10.3.2.6;

-	Group membership update response specified in clause 10.3.2.7;

-	Group membership notification specified in clause 10.3.2.8;

-	Group deletion request specified in clause 10.3.2.9;

-	Group deletion response specified in clause 10.3.2.10;

-	Group deletion notification specified in clause 10.3.2.11;

-	Group information request specified in clause 10.3.2.12;

-	Group information response specified in clause 10.3.2.13;

-	Group information subscribe request specified in clause 10.3.2.14;

-	Group information subscribe response specified in clause 10.3.2.15;

-	Group information notify request specified in clause 10.3.2.16;

-	Group information notify response specified in clause 10.3.2.17;

-	Store group configuration request specified in clause 10.3.2.18;

-	Store group configuration response specified in clause 10.3.2.19;

-	Get group configuration request specified in clause 10.3.2.20;

-	Get group configuration response specified in clause 10.3.2.21;

-	Subscribe group configuration request specified in clause 10.3.2.22;

-	Subscribe group configuration response specified in clause 10.3.2.23;

-	Notify group configuration request specified in clause 10.3.2.24;

-	Notify group configuration response specified in clause 10.3.2.25;

-	Configure VAL group request specified in clause 10.3.2.26;

-	Configure VAL group response specified in clause 10.3.2.27;

-	Group announcement specified in clause 10.3.2.28;

-	Group registration request specified in clause 10.3.2.29;

-	Group registration response specified in clause 10.3.2.30;

-	Identity list notification specified in clause 10.3.2.31;

-	Group de-registration request specified in clause 10.3.2.32; and

-	Group de-registration response specified in clause 10.3.2.33.

The usage of the above information flows are clarified as below:

-	The identity is the UE ID or CAA level UAV ID;

-	The identity list or identities list is the list of UE IDs or CAA level UAV IDs; and

-	The VAL server is the UAE server or the UAS application specific server.

* * * Next Change * * * *

#### 7.1.3.2	Information flows

The following information flows of location management service of SEAL as specified in 3GPP TS 23.434 [5] are applicable for the UAS applications:

-	Location reporting configuration request specified in clause 9.3.2.0;

-	Location reporting configuration response specified in clause 9.3.2.1;

-	Location information report specified in clause 9.3.2.2;

-	Location information request specified in clause 9.3.2.3;

-	Location reporting trigger specified in clause 9.3.2.4;

-	Location information subscription request specified in clause 9.3.2.5;

-	Location information subscription response specified in clause 9.3.2.6;

-	Location information notification specified in clause 9.3.2.7;

-	Location reporting configuration cancel specified in clause 9.3.2.8;

-	Get UE(s) information request specified in clause 9.3.2.9;

-	Get UE(s) information response specified in clause 9.3.2.10;

-	Monitor location subscription request specified in clause 9.3.2.11;

-	Monitor location subscription response specified in clause 9.3.2.12; and

-	Notify location monitoring event specified in clause 9.3.2.13.

The usage of the above information flows are clarified as below:

-	The identity is the UE ID or CAA level UAV ID;

-	The identity list or identities list is the list of UE IDs or CAA level UAV IDs; and

-	The VAL server is the UAE server or the UAS application specific server.

* * * Next Change * * * *

#### 7.1.4.2	Information flows

The following information flows of network resource management service of SEAL as specified in 3GPP TS 23.434 [5] are applicable for the UAS applications:

-	Network resource adaptation request specified in clause 14.3.2.1;

-	Network resource adaptation response specified in clause 14.3.2.2;

-	Resource request specified in clause 14.3.2.6;

-	Resource response specified in clause 14.3.2.7;

-	Resource modification request specified in clause 14.3.2.8;

-	Resource modification response specified in clause 14.3.2.9;

-	Monitoring Events Subscription Request specified in clause 14.3.2.17;

-	Monitoring Events Subscription Response specified in clause 14.3.2.18; and

-	Monitoring Events Notification message specified in clause 14.3.2.19.

The usage of the information flows are clarified as below:

-	The identity is the 3GPP UE ID or CAA level UAV ID;

-	The identity list or identities list is the list of 3GPP UE IDs or CAA level UAV IDs; and

-	The VAL server is the UAE server or the UAS application specific server.

* * * Next Change * * * *

#### 7.1.5.2	Information flows

The following information flows of network slice capability exposure service of SEAL as specified in 3GPP TS 23.435 [14] are applicable for the UAS applications:

-	VAL server policy provisioning request specified in clause 9.5.3.2;

-	VAL server policy provisioning response specified in clause 9.5.3.3;

-	VAL server policy update request specified in clause 9.5.3.4;

-	VAL server policy update response specified in clause 9.5.3.5;

-	VAL server policy delete request specified in clause 9.5.3.6;

-	VAL server policy delete response specified in clause 9.5.3.7;

-	VAL server policy usage reporting data subscribe request specified in clause 9.5.3.8;

-	VAL server policy usage reporting data subscribe response specified in clause 9.5.3.9;

-	VAL server policy usage reporting data notification specified in clause 9.5.3.10;

-	Network slice optimization subscription request specified in clause 9.5.3.11;

-	Network slice optimization subscription response specified in clause 9.5.3.12;

-	Network slice optimization notification specified in clause 9.5.3.13;

-	Network slice optimization report retrieval request specified in clause 9.5.3.14;

-	Network slice optimization report retrieval response in clause 9.5.3.15; and

-	Policy harmonization subscribe notify specified in clause 9.5.3.16.

The usage of the information flows are clarified as below:

-	The identity is the 3GPP UE ID or CAA level UAV ID;

-	The identity list or identities list is the list of 3GPP UE IDs or CAA level UAV IDs;

-	The VAL server is the UAE server or the UAS application specific server or UAS UE;

-	The area of interest comprises geographical information in form of waypoint(s);

-	The trigger event comprises time information about the route or time point specifying when the UAV reaches the respective waypoint and the corresponding QoS requirement of each waypoint; and

-	The priority comprises priority information about a route/waypoint(s).

* * * Next Change * * * *

##### 7.1a.2.2.2	Procedure

Pre-condition:

-	The UAE client has already registered with the UAE server as described in clause 7.1a.2.1.

Figure 7.1a.2.2.2-1: Procedure for deregistering the UAE client at the UAE server

1.	The UAE client sends a deregistration request to the UAE server.

2.	The UAE server sends a deregistration response to the UAE client.

* * * Next Change * * * *

##### 7.1a.2.3.2	Procedure

Pre-conditions:

-	The UAE client has already registered with the UAE server as described in clause 7.1a.2.1.

Figure 7.1a.2.3.1-1: Procedure for registration update by the UAE client at the UAE server

1.	The UAE client sends a registration update request to the UAE server.

2.	The UAE server sends an acknowledgement to the UAE client.

* * * Next Change * * * *

#### 7.3.2.1	Procedure for group creation for one pair of UAV and UAV-C

Figure 7.3.2.1-1 illustrates a high-level procedure for group creation.

Pre-conditions:

-	Both UAV-C and UAV have successfully registered and connected to the UAE server.

-	A CAA level UAV ID is already assigned to the UAV-C and UAV.

Figure 7.3.2.1-1: Procedure for group creation for one pair of UAV and UAV-C

1.	The UAE server recognizes a unique pair of UAV and UAV-C either by 3GPP UE ID or CAA level UAV ID.

NOTE:	The mechanisms for how the UAE server recognizes a pair of UAV-C and UAV is out of scope of the present document.

2.	The UAE server sends a group creation request to the SEAL GM server, if there is no pre-assigned group ID, by using the GM-S reference link as specified in 3GPP TS 23.434 [5] using the procedure defined in clause 10.3. The SEAL GM server creates one group ID for one pair of UAV and UAV-C as specified in 3GPP TS 23.434 [5].

3.	The UAE server uses the returned group ID for UAS for QoS management.

* * * Next Change * * * *

#### 7.3.2.2	Procedure for group-based approach for C2 QoS provisioning

Figure 7.3.2.2-1 illustrates a high-level workflow of group-based C2 QoS provisioning.

Pre-conditions:

-	Both UAV and UAV-C have registered to 3GPP 5G network respectively. C2 communication is established.

-	The procedure specified in clause 7.3.2.1 is performed and the group ID for the UAS group is available at the UAE server.

Figure 7.3.2.2-1: Procedure of group-based approach for C2 QoS provisioning.

1.	The UAE server monitors the QoS for the UAS group (which includes a UAV and UAV-C) by SEAL NRM as specified in3GPP TS 23.434 [5].

2.	In cases where the network condition for C2 communication does not satisfy the pre-defined QoS requirement, the UAE server may choose to send QoS adaptation request to the SEAL NRM server using the NRM-S reference point as specified in 3GPP TS 23.434 [5]. The QoS adaptation request needs to be sent per group ID for a pair of UAV and UAV-C created in the procedure specified in clause 7.3.2.1. The subsequent network resource adaptation procedure is triggered by the UAE server as specified in clause 14.3.3.3.1 of 3GPP TS 23.434 [5].

3.	The UAE client and UAE server established communication based on new QoS requirements as specified in clause 14.3.3.2.1.2 of 3GPP TS 23.434 [5].

4.	UAS application layer adapts the updated QoS assignment.

NOTE:	The mechanisms for how the UAS application layer is adapting newly assigned QoS is out of scope of the present document.

* * * Next Change * * * *

#### 7.3.2.3	Procedure for group update

Figure 7.3.2.3-1 illustrates the group membership update when UAV-2 is used to replace UAV-1.

Pre-conditions:

-	The UAV-C, UAV-1, and UAV-2 are all previously successfully subscribed with 3GPP Core Network and UAS application specific server (e.g. USS/UTM) and have received a 3GPP UE ID (e.g. GPSI) and a CAA level UAV ID.

-	The UAV-1 and UAV-C have been grouped by a group ID by SEAL GMS as specified in clause 7.3.2.1.

Figure 7.3.2.3-1: Procedure for group update

1.	The UAE server recognizes a new pair of UAV-2 and UAV-C by the new CAA level UAV ID.

NOTE:	The mechanisms for how the UAE server recognizes a new pair of UAV-C and UAV is out of scope of the present document.

2.	The UAE server sends a group membership update request to the SEAL GM server using the procedure specified in clause 10.3.2.6 of 3GPP TS 23.434 [5]. The SEAL GM server sends a group membership update response as specified in clause 10.3.2.7 of 3GPP TS 23.434 [5].

* * * Next Change * * * *

### 7.3.3	Information flows

The usage of information flows between UAE server and SEAL's Group management Server is specified in clause 7.1.2.2.

The usage of information flows between UAE server and SEAL's Network Resource Management Server is specified in clause 7.1.4.2.

* * * Next Change * * * *

### 7.4.2	Procedures

* * * Next Change * * * *

#### 7.4.2.2	C2 communication modes configuration

This procedure enables the configuration of the UAE Client, based on an application request from UAS application specific server (which can be the USS/UTM) to manage the C2 operation modes (direct, network-assisted, UTM-Navigated) of C2 communication for a UAS. For redundancy purposes, two links can be activated to work simultaneously or as active/standby. These links can belong to the same or different networks (e.g., dual subscription functionality) see Figure B.2.1-3.

Figure 7.4.2.2-1 illustrates the C2 communication modes configuration procedure.

Pre-conditions:

1.	The UAS UEs are connected to 5GS and authenticated and authorized by UAS application specific server as specified in clause 5.2 of 3GPP TS 23.256 [4].

2.	UAE Server has established a UAE session with the respective UAE Clients as the UAE clients are successfully registered to the UAE server.

3.	UAE Server has performed the C2 mode selection/switching capability initiation as in clause 7.4.2.1.

Figure 7.4.2.2-1: C2 communication modes configuration

1.	The UAE Server sends a C2 communication modes configuration request including the UAS identifier. If the C2 operation mode management container is provided, the request shall include the allowed C2 communication modes (e.g., direct, network assisted, UTM-Navigated), the primary C2 communication mode and the policy for C2 switching, and may include the secondary C2 communication mode. The UAE server may configure multiple Network-Assisted C2 communication links if the UAE client has indicated this capability to the UAE server at the UAE layer registration and if the UAE server has received such configuration via procedure described in clause 7.4.2.1. The UAE server may configure multiple UTM-Navigated C2 communication links if the UAE client has indicated this capability to the UAE server at the UAE layer registration and if the UAE server has received such configuration via procedure described in clause 7.4.2.1. In the case of removal of C2 communication mode configuration parameters from the UAV or UAV-C, then the request shall only include the UAS identifier.

2.	The UAE Client stores or removes the C2 communication mode configuration parameters as per the information received in step 1.

3.	The UAE Client sends a C2 communication modes configuration response to the UAE Server.

* * * Next Change * * * *

#### 7.4.3.1	C2 operation mode management request

Table 7.4.3.1-1 describes the information flow C2 operation mode management request from the UAS application specific server to the UAE server.

Table 7.4.3.1-1: C2 operation mode management request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the C2 operation mode management. This ID can be the USS/UTM identifier, when the UAS application specific server is the USS/UTM. |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| C2 operation mode management container (see NOTE 2) | O | The C2 operation mode management container consists of the requirements and policy for C2 operation mode management |
| > C2 operation mode management requirement | M | Identification of the type of the C2 mode switching to be supported by the UAE server. This can be either from direct to network-assisted C2, or from network-assisted to direct C2 or to UTM-Navigated. |
| > Allowed C2 communication modes | M | direct, network assisted, UTM-Navigated |
| > Primary C2 communication mode | M | Primary C2 communication mode (direct, network assisted) |
| > Secondary C2 communication mode | O | Secondary C2 communication mode (direct, network assisted) |
| > Policy of C2 switching | M | Parameters for C2 switching - QoS thresholds on active link - QoS thresholds on target link |
| > C2 service area | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the capability is active. |
| > C2 direct mode availability reporting requirement | O | A requirement for C2 direct mode reporting. |
| >> ProSe application codes | O | The ProSe codes used for direct C2 communications. The ProSe codes are used for the ProSe Direct Discovery as specified in 3GPP TS 23.256 [4]. |
| >>Time of validity | O | The time for which the C2 direct mode availability/feasibility checking applies. |
| >>Reporting configuration | O | The configuration of the reporting and periodicity/frequency of reporting required. |
| > Dual Network-Assisted C2 communication links | O | Support for multiple Network-Assisted C2 communication links. |
| >> Policy of C2 switching in case of Network-Assisted C2 communication link 1 | O | Parameters for C2 switching for Network-Assisted C2 communication link 1: - QoS threshold needed for C2 communication per subscription/network, QoS threshold needed for payload (user plane) communication per subscription/network. |
| >> Policy of C2 switching in case of Network-Assisted C2 communication link 2 | O | Parameters for C2 switching for Network-Assisted C2 communication link 2: - QoS threshold needed for C2 communication per subscription to activate simultaneously both subscriptions/networks, QoS threshold needed for payload (user plane) communication per subscription to activate simultaneously both subscriptions/networks, |
| >> C2 service area for Network-Assisted C2 communication link 1 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 1 is active. |
| >> C2 service area for Network-Assisted C2 communication link 2 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 2 is active. |
| > Dual UTM-Navigated C2 communication links | O | Support for multiple UTM-Navigated C2 communication links. |
| >> Policy of C2 switching in case of UTM-Navigated C2 communication link 1 | O | Parameters for C2 switching for UTM-Navigated C2 communication link 1: - QoS threshold needed for C2 communication per subscription/network, QoS threshold needed for payload (user plane) communication per subscription/network. |
| >> Policy of C2 switching in case of UTM-Navigated C2 communication link 2 | O | Parameters for C2 switching for UTM-Navigated C2 communication link 2: - QoS threshold needed for C2 communication per subscription to activate simultaneously both subscriptions/networks, QoS threshold needed for payload (user plane) communication per subscription to activate simultaneously both subscriptions/networks, |
| >> C2 service area for UTM-Navigated C2 communication link 1 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 1 is active. |
| >> C2 service area for UTM-Navigated C2 communication link 2 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 2 is active. |
| NOTE 1: Void NOTE 2: If C2 operation mode management container IE is not included, it indicates removal of the C2 operation mode management related information. |  |  |

* * * Next Change * * * *

### 7.6.1	General

This feature introduces the UAS application enablement services for supporting change of UAS application specific server. In particular, the UAE layer provides support for the following operations:

-	Support of the registration of the UAE clients multi-USS capability to the UAE server as described in clause 7.1a.

-	Support the distribution of the for multi-USS policies from the UAS application specific server to the UAE server and the UAE client, as described in clause 7.6.2.1 and clause 7.6.2.2.

-	Support the change of UAS application specific server, as described in clause 7.6.2.3.

-	Support the UAE server triggered change of USS, as described in clause 7.6.2.5.

NOTE:	The functions of the USS are out of scope of the present specification.

* * * Next Change * * * *

#### 7.6.2.3	UAE layer assisted change of USS

This procedure provides a mechanism for supporting dynamic change of USS which may be performed while the UAV flight is ongoing, due to expected location/mobility of the UAV, emergency events, etc.

Figure 7.6.2.3-1 illustrates the procedure where the UAE server supports the change of USS.

Pre-conditions:

1.	UAE client has indicated support of change of USS by the Multi-USS capability.

2.	UAS application specific server has provided Multi-USS policies to the UAE client and the UAE server.

Figure 7.6.2.3-1: UAE layer assisted change of USS

1.	The UAE server receives a USS change request from a serving USS (UAS application specific server #1), indicating that a target USS (UAS application specific server #2) an take over the communication. The request includes the UAV (UAE client) identification information, target USS information and USS change authorization information (e.g. authorization token). Optionally, updated Multi-USS policies for one or more USSes can be included. The UAE server verifies that the request is authorized (e.g., Multi-USS capability is enabled, new USS part of the allowed USS information).

2.	If required, the UAE server translates this to a UP path change and interacts with NEF as AF for influence UP path (switching to target DNAI). In particular UAE server (acting as AF) checks whether it can serve the target DNAI corresponding to the target USS based on the mapping of USS to DNAI which was performed in step 2 of clause 7.6.2.5. Interaction with 5GC is performed according to functionality for application function influence on traffic routing, see  3GPP TS 23.502 [13] clause 4.3.6.3.

3.	The UAE server forwards the USS change request to the UAE client including target USS information and updated Multi-USS policies.

4.	Perform change of USS.

The UAE client initiates communication with the target USS based on the USS change request and the Multi-USS policies.

5.	The UAE client sends a USS change response indicating to what USS the change of USS has been performed.

6.	The UAE server sends a USS change response to the UAS application specific server indicating that a change of USS has been performed.

* * * Next Change * * * *

#### 7.7.3.1	DAA support management request

Table 7.7.3.1-1 describes the information flow DAA support management request from the UAS application specific server to the UAE server.

Table 7.7.3.1-1: DAA support management request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the DAA management. This ID can be the USS identifier, when the UAS application specific server is the USS. |
| UAS ID | M | The identification of the UAS (UAV or LDGS) for which the DAA support management request applies. This could be in form of identifier for the UAS, e.g., group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| List of UAS ID | O | A list of UAS identification information, e.g. CAA level UAV ID, of the UAVs that are identified as target UAVs, is included when UAE client has LDGS capability. |
| Geographical area | O | The area where the DAA client configuration applies to locate relevant LDGS capable UAE clients. This can be geographical area, or topological area. |
| DAA application policy | O | The DAA application policy. |
| >DAA triggering thresholds | O | Indicate threshold for DAA, that determine when an LDGS may trigger detect and/or resolution of flightpath conflict. |
| >Time validity | O | Time validity where the DAA client configuration applies for an LDGS capable UAE client. |
| > Reporting frequency | O | Indication on how often the LDGS collect target UAVs information and how often UAS application specific server would like to be reported. This can be event based and / or time based. |

* * * Next Change * * * *

#### 7.7.3.4	DAA support configuration request

Table 7.7.3.4-1 describes the information flow DAA support configuration request from the UAE server to the UAE client.

Table 7.7.3.4-1: DAA support configuration request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS (UAV or LDGS) for which the DAA management request applies. This could be in form of identifier for the UAS, e.g., group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| List of UAS ID | O | A list of UAS identification information, e.g. CAA level UAV ID, of the UAVs that are identified as target UAVs, is included when UAE client has LDGS capability. |
| DAA application policy | O | The DAA application policy. |
| >DAA triggering thresholds | O | Indicate threshold for DAA, that determine when an LDGS may trigger detect and/or resolution of flightpath conflict. |
| >Time validity | O | Time validity where the DAA client configuration applies for an LDGS capable UAE client. |
| > Reporting frequency | O | Indication on how often the LDGS collect target UAVs information and how often UAS application specific server would like to be reported. This can be event based and / or time based. |

* * * Next Change * * * *

#### 7.10.2.1	Flight route support

Figure 7.10.2.1-1 describes the flight route support procedure.

Pre-conditions:

-	UAV has performed the UAS UE registration procedure.

Figure 7.10.2.1-1: Flight route calculation procedure

1.	The UAS application specific server or UAE client (UAV or UAV-C) sends flight route request towards the UAE server. This request contains network parameters needed for the specific needs of the specific use case (start point, destination point, specific times of start point and end point, needed QoS, link reliability, shortest route needed, etc).

2.	The UAE server has the role of a route planning unit and based on the received end points and provisioned information (e.g., restricted areas, existing flight paths, available base stations coverage, shortest distance, shortest time duration) calculates possible waypoints as geographical coordinates and specific times.

3.	The UAE server checks with NWDAF (via NEF) the QoS sustainability analytics (clause 6.9 of 3GPP TS 23.288 [zz]) in these areas of interest/waypoints identified in step 2. Location information towards the NWDAF is presented as area of interest or path of interest and can reflect a list of waypoints. Based on the received information from NWDAF the UAE server selects waypoints (geographical coordinates and time) which KPIs map the requested criteria in step1.

4.	The UAE server sends flight route reply with the selected waypoints in step 3 to the entity which has send the request (UTM, UAV or UAV-C).

* * * Next Change * * * *

## 8.1	General

The following UAE capabilities are offered as APIs:

-	UAE server APIs;

The following SEAL service APIs are specified in 3GPP TS 23.434 [5]:

-	Group management server APIs;

-	Location management server APIs;

-	Configuration management server APIs;

-	Identity management server APIs; and

-	Key management server APIs.

The following SEAL service APIs are specified in 3GPP TS 23.435 [14]:

- Network slice optimization APIs based on VAL server policy,

When UAS application specific server invokes a SEAL server API directly, the UAS application specific server acting as VAL server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response as specified in 3GPP TS 23.434 [5].

Requesting specific QoS per planned flight path/waypoint is available using Policy of needed minimum QoS per UE in the SS_NSCE_PolicyManagement API, see 3GPP TS 23.435 [14] clause 9.5.4.1.

* * * Next Change * * * *

Annex A (informative):
Support for edge deployments

The application architecture for supporting edge applications are specified in 3GPP TS 23.558 [7]. The UAS application layer as specified in clause 5 can be deployed in edge computing environment.

Figure A-1 illustrates a UAS application layer deployment in edge computing environments.

Figure A-1: UAS application layer deployment in edge computing environment

Figure A-1 illustrates how the UAS application architecture with can be deployed with the application architecture for supporting edge applications. In a UE, the UAS application specific client(s), UAE client and SEAL clients interact with the Edge Enabler Client (EEC) via EDGE-5 reference point in order to consume edge services. In an Edge Data Network (EDN), the UAS application specific server(s), UAE server and SEAL servers acting as Edge Application Server (EAS), interacts with the Edge Enabler Server (EES) via EDGE-3 reference point. The service provisioning and EAS discovery enable the UAS application layer entities in the UE to communicate with the application layer entities in the EDN. The interactions between the entities and 5GS are not shown for the sake of simplicity.

NOTE:	This clause illustrates an example edge deployment using edge enabler layer. There can be other valid edge deployments enabled for UAS application layer.

* * * End of Changes * * * *