3GPP TSG-SA WG6 Meeting #62	S6-243552

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243241)

Source:	Ericsson

Title:	Support UE to UE communication in different modes via SEALDD

Spec:	3GPP TS 23.700-23 v1.1.0

Agenda item:	8.5

Document for:	Approval

1. Introduction

This paper introduces a new solution for KI#3.

2. Reason for Change

KI#3 has the following open issues:

-	How the application enablement layer can be enhanced to support usage of direct communication between 5G connected AR/VR devices.

-	Whether and how the application enablement layer can be enhanced to make coordination (e.g., the connection redundancy) between network based communication and direct communication between AR/VR devices for AR/VR services communication.

For direct communication between 5G connected UEs, SEALDD off-network communication modal is proposed to accommodate the need to coordinate multi-modal XR traffic between UEs.

For coordination between off-network and on-network communication, the SEALDD server can monitor the proximity of the two UEs and QoS for the UE-to-UE communciation, then determine whether to use off-network or on-network communication for the two UEs.

3. Conclusions

This pCR proposes a solution to address KI#3 Coordination between direct UE connection (i.e. off-network) and network-based connection for AR/VR services.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |  |  |  |  |
| Sol #2 |  | X |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |  |
| Sol #4 | X |  |  |  | X |  |  | X |
| Sol #5 |  | X |  |  |  |  |  |  |
| Sol #6 | X |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |  |
| Sol #9 |  |  |  |  | X |  |  |  |
| Sol #10 |  |  |  |  |  | X |  |  |
| Sol #11 |  |  |  |  | X |  |  |  |
| Sol #12 |  |  |  |  |  | X |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |
| Sol #X |  |  | X |  |  |  |  |  |

* * * Next Change * * * *

## 7.x	Solution #X: Support UE to UE communication in different modes via SEALDD

### 7.x.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6] with additional inter-SEALDD client reference point.

Figure 7.x.1-1: SEALDD architecture with on-network and off-network

VAL UE 1 and VAL UE 2 have on-network connectivity, and both VAL UEs can directly communication with each other via off-network connectivity (i.e. SEALDD-PC5) or via on-nework connectivity (i.e. SEALDD).

### 7.x.2	Solution description

#### 7.x.2.1	General

When two VAL UEs need to communicate with each other for exchanging gaming/interactive data in XR service, they can use servers in the DN as data relay or communicate with each other directly.

NOTE 1:	As an example, XR gaming application client can support LAN-type of multi-player game mode without involvement of XR gaming server to process run-time game data.

In order to improve service experience (esp. for reducing service latency), SEALDD server monitors distance of UEs involved in the XR communication. When SEALDD server finds that the two UEs are within direct communication range, the SEALDD server instructs the SEALDD clients to switch to direct communication mode. Such mode switch decision needs to take E2E communication performance into consideration.

If the UE-to-UE application performance analytics result from ADAE shows the quality in off-network mode will be degraded and the quality in on-network mode is estimated to be good, or vice versa, the mode switch is done in SEALDD layer.

NOTE 2:	For the same XR application service, all XR traffic flows are transmitted via either off-network or on-network between UEs.

#### 7.x.2.2	SEALDD enabled UE-to-UE communication based on policy

The SEALDD servers has Data Delivery (DD) policy being provisioned for UE-to-UE communication. The DD policy is enforced by the SEALDD server to switch the SEALDD connection for UE-to-UE communication (either direct or indirect).

Pre-conditions:

- The SEALDD server has DD policies available.

- The SEALDD server has received VAL server request to subscribe to SEALDD event exposure for connection status for UE1 and UE2 communication.

Figure 7.x.2.2-1: Policy enforced by SEALDD server for connectivity between two UEs

1a.	The UE-to-UE communication is established between UE 1 and UE 2 via SEALDD client.

1b.	The SEALDD client in UE1/2 informs the SEALDD server about the establishment of direct SEALDD communication.

2.	Based on SEALDD UE-to-UE policy, the SEALDD server requests 3GPP CN (NWDAF/NEF) for UE relative proximity analytics for UE1 and UE2, requests 3GPP CN (GMLC/NEF) for SL/Ranging service exposure about the relative locations or distances and directions related to the UE1 and UE2, and requests ADAES for UE-to-UE application performance analytics as described in TS 23.436 clause 8.4.

NOTE:	Whether ADAES UE-to-UE application performance analytics needs to be enhanced (e.g. data collection from SEALDD layer, support both off-network and on-network UE-to-UE communication analytics, more analytics report information like QoE) can be decided in the normative work.

3.	According to the SEALDD UE-to-UE policy, if relative proximity analytics result shows that both UEs are not in vicinity to each other (distance between UEs > proximity threshold in UE-to-UE policy), and/or the QoS/QoE for UE-to-UE direct communication analytics result is no so good (e.g. PLR in analytics report > PLR threshold in UE-to-UE policy), the SEALDD continues with step 4 for indirect communication preparation; otherwise, the SEALDD keeps monitoring (as described in step 2) for the two UEs.

4.	After establishment of SEALDD connections for UE1 and UE2, the SEALDD traffic carrying XR flows are processed in the SEALDD server. The SEALDD server forwards traffic between UE1 and UE2, and may buffer data burst from originating UE and send it to destination UE smoothly.

5.	If later on, both UEs are in proximity which is capable of direct communication and the QoS/QoE analytics result from ADAES for UE-to-UE direct communication shows good quality, the SEALDD server helps to establish direct SEALDD communication by informing the SEALDD client of any of the two UEs with the information of its peer SEALDD client (of UE1 or UE2). Then the SEALDD client 1 or 2 establishes SEALDD connection towards the peer SEALDD client via SEALDD-PC5 and the SEALDD traffic carrying XR flows are exchanged directly between UE1 and UE2 via SEALDD-PC5.

After step 4 or step 5, the SEALDD server notifies the VAL server about the establishment of communication between UE1 and UE2.

Based on the monitoring and analytics result (as requested in step 2), and SEALDD UE-to-UE policy:

-	if the current UE1-UE2 communication mode is direct and SEALDD server decides to switch to indirect communication mode, the SEALDD server establishes SEALDD connections with SEALDD client 1 and SEALDD client 2 as described in step 4, then triggers direct SEALDD communication release towards the SEALDD client which received establishment request in step 5 and the SEALDD client further releases the direct SEALDD connection towards its peer SEALDD client via SEALDD-PC5.

-	if the current UE1-UE2 communication mode is indirect and SEALDD server decides to switch to direct communication mode, the SEALDD server triggers direct SEALDD communication setup as described in step 5 and releases SEALDD connections towards SEALDD client 1 and SEALDD client 2.

When the communication mode is changed, the VAL server is also notified by the SEALDD server about the connection status (e.g. direct/indirect mode, QoS/QoE).

### 7.x.3	Information flows

Table 7.x.3-1 describes the new information elements to be added to the SEALDD policy configuration request in 3GPP TS 23.433 [6] Table 9.10.3.1-1 in order to support UE-to-UE communication flows and policy.

Table 7.x.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| XR traffic flows | M | Identify traffic flows for XR application. |
| > Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| > identity list | M | List of identifiers of the VAL UEs or VAL users for which SEALDD policy applies |
| SEALDD policy | M | The SEALDD policy associated with application traffic identifiers, VAL UE identity |
| > UE-to-UE policy (NOTE) | O | Specifies UE-to-UE direct communication policy including proximity threshold, QoS threshold (e.g. bitrate, latency and jitter, PLR, PER) or QoE threshold (e.g. MOS) for entering direct communication mode, and proximity threshold, QoS threshold or QoE threshold for leaving direct communication mode. |

NOTE:	Remaining information flows (e.g. trigger direct SEALDD communication setup, SEALDD connection status notification) to support the procedure can be done during normative work.

### 7.x.4	Solution evaluation

This solution addresses KI#3 to support:

-	How the application enablement layer can be enhanced to support usage of direct communication between 5G connected AR/VR devices.

-	Whether and how the application enablement layer can be enhanced to make coordination (e.g., the connection redundancy) between network-based communication and direct communication between AR/VR devices for AR/VR services communication.

The SEALDD layer facilitates the communication between UEs, the SEALDD server supervises the UE-to-UE distance and their communication quality to coordinate on-network communication and off-network communication.

# *** End of Changes ***