3GPP TSG-SA WG6 Meeting #62	S6-243617

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243047)

Source:	InterDigital Inc.

Title:	Pseudo-CR on Spatial anchor subscription

Spec:	3GPP TS 23.437 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes normative text on Spatial anchor subscriptions.

2. Reason for Change

The following has been concluded in TR 23.700-21 regarding spatial anchor subscriptions.

i.	for Key issue #1 (Enabler support for managing spatial anchors): Solution #1, Solution#2 and Solution #3 can be consider for normative work with possible merge of the procedures as specified in clause 10.2.1. All solutions will be adapted to the conclusion of the architecture option in the normative work.

3. Conclusions

This pCR provides normative text for KI#1 Solution #3.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.1.0.

* * * First Change * * * *

## 8.3	Spatial anchors management procedures

### 8.3.y	Spatial anchor subscription

#### 8.3.y.1	General

Spatial anchor subscription enables a SAnM client or VAL server to subscribe with the SAnM server to receive notifications related to spatial anchors.

#### 8.3.y.2	Procedures

##### 8.3.y.2.1	Subscribe

Figure 8.3.y.2.1-1 illustrates the spatial anchor subscription procedure.

Pre-conditions:

1.	The SAnM client or VAL server has received information (e.g. URI, IP address) related to the SAnM server; and

2.	The SAnM client or VAL server has received security credentials authorizing it to communicate with the SAnM server.

Figure 8.3.y.2.1-1: Spatial anchor subscription

1.	The requestor (e.g., SAnM client or VAL server) sends a spatial anchor subscribe request to the SAnM server. The request includes a requestor identifier, security credentials, a notification endpoint, and event information for which the requestor is subscribing. The request may include an expiration time.

a.	The requestor may include the "discovery" event type to indicate the SAnM server to notify the requestor when the SAnM server detects events related to the management of spatial anchors (e.g., creation, update, deletion) according to discovery event filters and spatial anchor filters provided in the request. If the requestor is a SAnM client, the request includes a VAL client identifier.

b.	The VAL server may include the "UE with spatial anchor interest" event type to indicate the SAnM server to notify the VAL server when the SAnM server detects UE(s) with interest in specific spatial anchors according to the spatial anchor of interest event filters and spatial anchor filters provided in the request.

c.	The requestor may include the "spatial anchor in range" event type to indicate the SAnM server to notify the requestor when the SAnM server detects that spatial anchors become in range or out of range of the subscriber according to range event filters provided in the request. Range event filters can include a range for detecting spatial anchors, an indication to enable persistent search, the pose of the subscriber. If the requestor is a SAnM client, the spatial anchor subscribe request includes a VAL client identifier.

2.	Upon receiving the request from the requestor, the SAnM server validates if the requestor is authorized for the request. If the requestor is authorized, the SAnM server creates the subscription and stores the subscription information.

a.	If the subscribed event is for "discovery", the SAnM server starts monitoring spatial anchors management events according to the discovery event filters and spatial anchor filters.

b.	If the subscribed event is for "UE with spatial anchor interest", the SAnM server starts monitoring spatial anchors of interest according to the UE with spatial anchor interest event filters and spatial anchor filters. If the spatial anchors of interest have been discovered by a UE, the SAnM can obtain the UE location information from the 5GS or the SEAL Location Management service. Additionally, the SAnM server can consider data analytics, such as UE mobility prediction. The SAnM server may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs enters, exits or is within range of the spatial anchors of interest.

c.	If the subscribed event is for "spatial anchor in range" and the requestor is a SAnM client, the SAnM server starts monitoring the corresponding UE location and pose information according to the range event filters and spatial anchor filters. The UE location information can be obtained by using capabilities of the 5GS or the SEAL Location Management service. Additionally, the SAnM server can consider data analytics, such as UE mobility prediction. The SAnM server monitors the distance between UEs and the spatial anchor of interest to determine whether the UEs becomes in-range or out-of-range of the spatial anchors of interest.

3.	The SAnM server sends a spatial anchor subscribe response to the requestor. If the SAnM server has created the subscription, the response includes an indication of success, the subscription identifier and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the spatial anchor subscription expires. If the SAnM server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.3.y.2.2	Notify

Figure 8.3.y.2.2-1 illustrates the spatial anchor notify operation between the SAnM server and a SAnM client or VAL server.

Pre-conditions:

- The SAnM client or VAL server has subscribed for spatial anchor notifications with the SAnM server;

Figure 8.3.y.2.2-1: Spatial anchor notification

1.	An event occurs at the SAnM server that satisfies trigger conditions for notifying a subscriber (e.g., SAnM client or VAL server) according to subscribed events.

2.	The SAnM server sends a spatial anchor notification to the requestor indicating the event. The notification includes a subscription identity and includes event-specific information.

a.	If the subscribed event is for "discovery", the notification includes the list of determined spatial anchors

b.	If the subscribed event is for "UE with spatial anchor interest", the notification includes information about UE with spatial anchor interest (e.g., UE identifier, spatial anchor identifier(s), range from the spatial anchor(s), entering or exiting the range, etc.).

c.	If the subscribed event is for "spatial anchor in range", the notification includes the list of determined spatial anchors that become in range, out of range of the subscriber.

##### 8.3.y.2.3	Subscription update

Figure 8.3.y.2.3-1 illustrates the procedure for a SAnM client or VAL server to update a subscription with the SAnM server.

Pre-conditions:

- The SAnM client or VAL server has subscribed for spatial anchor notifications with the SAnM server;

Figure 8.3.y.2.3-1: Spatial anchor subscription update

1.	The requestor (e.g., SAnM client or VAL server) sends a spatial anchor subscription update request to the SAnM server. The request includes the requestor identifier, security credentials and the subscription identifier and may include subscribed event information and expiration time.

2.	Upon receiving the request from the requestor, the SAnM server validates if the requestor is authorized for the request. If the requestor is authorized, the SAnM server updates the subscription information.

3.	The SAnM server sends a spatial anchor subscription update response to the requestor. If the SAnM server has updated the subscription, the response includes an indication of success and may include an expiration time. To maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the spatial anchor subscription expires. If the SAnM server has not updated the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.3.y.2.4	Unsubscribe

Figure 8.3.y.2.4-1 illustrates the procedure for a SAnM client or VAL server to unsubscribe with the SAnM server.

Pre-conditions:

- The SAnM client or VAL server has subscribed for spatial anchor notifications with the SAnM server;

Figure 8.3.y.2.4-1: Spatial anchor unsubscribe

1.	The requestor (e.g., SAnM client or VAL server) sends a spatial anchor unsubscribe request to the SAnM server. The request includes the requestor identifier, security credentials and the subscription identifier.

2.	Upon receiving the request from the requestor, the SAnM server validates if the requestor is authorized for the request. If the requestor is authorized, the SAnM server cancels the subscription associated with the subscription identifier.

3.	The SAnM server sends a spatial anchor unsubscribe response to the requestor. If the SAnM server has canceled the subscription, the response includes an indication of success. If the SAnM server has not canceled the subscription, the response includes an indication of failure and may include a reason for failure.

#### 8.3.y.3	Information flows

##### 8.3.y.3.1	Spatial anchor subscribe request

Table 8.3.y.3.1-1 describes information elements for the spatial anchor subscribe request from the SAnM client or VAL server to the SAnM server.

Table 8.3.y.3.1-1: Spatial anchor subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is a SAnM client. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Event type | M | The type of event for which the requestor is subscribing. Supported event types include: - "discovery" - "UE with spatial anchor interest" - "spatial anchor in range" |
| Event detection filters | O | Event type specific detection filters. |
| > Discovery event filters | O | Event filters applicable to the "discovery" event type. The event filter is considered by the SAnM server for triggering the notification and can include management events (e.g., creation, update, deletion). |
| > UE with spatial anchor interest event filters | O | Event filters applicable to “UE with spatial anchor interest” event type. The event filter is considered by the SAnM server for triggering the notification and can include an indication for detecting UEs that have discovered spatial anchors, that are within range or become out of range of spatial anchors, or that enter or leave the range of a spatial anchor, and a value for the range. |
| > Range event filters | O | Event filters applicable to “spatial anchor in range” event type. The event filter is considered by the SAnM server for triggering the notification and can include an indication for spatial anchors becoming in range of the UE, out of range of the UE, or both and a value for the range. |
| Spatial anchor filters | O | The characteristics of spatial anchors that the SAnM server considers for triggering a notification. |
| > Spatial anchor ID list | O | Identifiers of the spatial anchors of interest. |
| > Service ID list | O | Identifiers of the associated application services of interest. |
| > Area of interest | O | The area of interest (e.g., topological and/or geographical). |

##### 8.3.y.3.2	Spatial anchor subscribe response

Table 8.3.y.3.2-1 describes information elements for the spatial anchor subscribe response sent by the SAnM server.

Table 8.3.y.3.2-1: Spatial anchor subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

##### 8.3.y.3.3	Spatial anchor notification

Table 8.3.y.3.3-1 describes information elements for the spatial anchor notification sent by the SAnM server.

Table 8.3.y.3.3-1: Spatial anchor notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Event type | M | The type of event notification. Supported event types include: - "discovery" - "UE with spatial anchor interest" - "spatial anchor in range" |
| Detected event | O | Identifier of the event that was detected and that triggered the server to send a notification (e.g., spatial anchors becoming in range of the UE or out of range of the UE; or, UE entering the range of a spatial anchor or leaving the range of a spatial anchor). |
| Spatial anchor list | O | List of discovered spatial anchors (as detailed in spatial anchor discovery response). Applicable for "discovery" and "spatial anchor in range" events. |
| UE with spatial anchor interest list | O | List of UE with spatial anchor interest. Applicable for "UE with spatial anchor interest" and "spatial anchor in range" events. |
| > UE ID | O | Identifier of the UE with spatial anchor interest. |
| > Spatial anchor ID | O | Identifier of the spatial anchor of interest. |
| > Spatial anchor range | O | The spatial anchor range. |

##### 8.3.y.3.4	Spatial anchor subscription update request

Table 8.3.y.3.4-1 describes information elements for the spatial anchor subscription update request from the SAnM client or VAL server to the SAnM server.

Table 8.3.y.3.4-1: Spatial anchor subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | Security credentials of the requestor. |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Event detection filters | O | Event type specific detection filters that the SAnM server considers for triggering a notification, as detailed in Table 8.3.y.3.1-1. |
| Spatial anchor filters | O | The characteristics of spatial anchors that the SAnM server considers for triggering a notification, as detailed in Table 8.3.y.3.1-1. |

##### 8.3.y.3.5	Spatial anchor subscription update response

Table 8.3.y.3.5-1 describes information elements for the spatial anchor subscription update response sent by the SAnM server.

Table 8.3.y.3.5-1: Spatial anchor subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

##### 8.3.y.3.6	Spatial anchor unsubscribe request

Table 8.3.y.3.6-1 describes information elements for the spatial anchor unsubscribe request from the SAnM client or VAL server to the SAnM server.

Table 8.3.y.3.6-1: Spatial anchor unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | Security credentials of the EEC |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be unsubscribed |

##### 8.3.y.3.7	Spatial anchor unsubscribe response

Table 8.3.y.3.7-1 describes information elements for the spatial anchor unsubscribe response sent by the SAnM server.

Table 8.3.y.3.7-1: Spatial anchor unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the unsubscribe request was successful. |
| Failure response | O | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure |

* * * End of Changes * * * *