3GPP TSG-SA WG6 Meeting #62	S6-243601

# Maastricht, NL, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0039 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Application Layer AI/ML Member capability Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solution#27 is recommended for normative work on application layer AI/ML Member capability analytics to support application layer AI/ML services. See more details in 3GPP TR 23.700-82. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added descriptions and procedures for support of application layer AI/ML member capability analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of application layer AI/ML member capability analytics is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3, 6.A(new), 8.X(new), 8.X.1(new), 8.X.2(new), 8.X.2.1(new), 8.X.2.2(new), 8.X.3(new), 8.X.3.1(new), 8.X.3.2(new), 8.X.3.3(new), 8.X.3.4(new), 8.X.3.5(new), 8.X.3.6(new), 8.X.3.7(new), 8.X.3.8(new), 8.X.3.9(new), 9.2.2, 9.2.Y(new), 9.2.Y.1(new), 9.2.Y.2(new), 9.2.Y.3(new), 9.2.Y.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

### 5.4.3	Application Data Analytics Enablement server

The application data analytics enablement server functional entity provides application layer analytics to support the VAL applications. The application data analytics enablement server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The application data analytics enablement server also supports interactions with the corresponding application data analytics enablement server in distributed SEAL deployments. The ADAE server also interacts with 3GPP core nework over N33 or N6 interface to subscribe to changes in configuration or other application server specific events. The ADAE server also acts as a co-ordinating entity to collect data from different sources and perform necessary actions to provide required analytics.

The ADAE server provides following server side functionalities:

-	monitoring performance of an application (VAL server or EAS, application session) and providing support for application performance analytics;

-	monitoring performance of a given network slice (from a list of subscribed slices for the VAL customer) and also usage pattern, and providing support for slice-specific application performance analytics and slice usage pattern analytics;

-	monitoring performance of an application session among two or more VAL UEs within a service or group, and providing support for UE-to-UE application performance analytics;

-	monitoring accuracy of a location and providing support for location accuracy analytics;

-	monitoring availability and service level for service APIs and providing support for service API analytics;

-	monitoring edge load parameters and providing support for edge load analytics;

-	monitoring application layer AI/ML member capability data and providing support for Application Layer AI/ML Member Capability Analytics.

* * * Next change * * * *

## 6.A	Support for Application Layer AI/ML Member Capability Analytics

This feature supports Application Layer AI/ML Member Capability Analytics to allow an analytics consumer (such as e.g. VAL Server, AIMLE Server) to be notified on analytics for application layer AI/ML Member capability. Such analytics can be used to support application layer AI/ML services, e.g. supporting FL member selection and reselection.

* * * Next change * * * *

## 8.X 	Procedure for Application Layer AI/ML Member Capability Analytics

### 8.X.1	General

This clause describes two procedures (covering both subscribe-notify and request-response models in 8.X.2.1 and 8.X.2.2 respectively) for supporting application layer AI/ML member capability analytics, where the application layer AI/ML member capability analytics are performed based on data collected from the Data Producer (e.g. ADAE client) and A-ADRF.

### 8.X.2	Procedure

#### 8.X.2.1	Subscribe-notify model

Figure 8.X.2.1-1: ADAES support for application layer AI/ML Member capability analytics

- The analytics consumer (e.g. VAL Server, AIMLE Server) sends analytics subscription request for application layer AI/ML Member capability analytics to ADAE server. For analytics subscription request, the request contains message as defined in Table 8.X.3.2-1.

- Upon receiving the event subscription request from the consumer, the ADAE server checks for the relevant authorization for the event subscription. If the authorization is successful, the ADAE server stores the request information. The ADAE server sends a service API event subscription response indicating successful subscription.

- The ADAES maps the analytics event ID to a list of data collection event identifiers, and a list of data producer IDs. Such mapping may be preconfigured by OAM or may be determined by ADAES based on the analytics event type/vertical type and/or data producer profile.

- The ADAE server sends a subscription request to the Data Producers (ADAE client) or a request to the Data Producers (A-ADRF) with the respective Data Collection Event ID and the requirement for data collection. Data collection at the UE(s) reuses the mechanism defined in 3GPP TS 26.531 [3].

- The Data Producers (ADAE client) send subscription response as a positive or negative acknowledgement to the ADAE server.

- The ADAE server based on subscription receive data on the application layer AI/ML Member capability based on the data collection event ID from ADAE client.

- The ADAE server based on request receive data/analytics on the application layer AI/ML Member capability based on the data/analytics collection event ID from A-ADRF.

NOTE 1:	The procedures for data collection for application layer AI/ML Member capability analytics need to take user consent into account.

- The ADAES performs analytics relevant operations to generate the analytics based on the data/analytics received from the ADAEC.

- The ADAES sends notifications to the consumer with the required application layer AI/ML Member capability analytics.

NOTE 2:	The format for the application layer AI/ML Member capability attributes will be standardized in stage 3.

#### 8.X.2.2	Request-response model

Pre-conditions:

-	ADAE server already have the analytics data derived from steps 3-8 in the procedure introduced in clause 8.X.2.1.

Figure 8.X.2.2-1: ADAES support for application layer AI/ML Member capability analytics

1.	The analytics consumer (e.g. VAL Server, AIMLE Server) sends a request message to the ADAE server to receive analytics data for application layer AI/ML Member capability. The request contains message as defined in Table 8.X.3.8-1.

2.	Upon receiving the request, the ADAE server authenticates and authorizes the analytics consumer.

3.	If the analytics consumer is authorized, the ADAE server may get the analytics by performing step 3 to 8 of clause 8.X.2.1.

4.	The ADAE server sends a response message including the analytics data (statistical and/or predictive) of the application layer AI/ML Member capability.

### 8.X.3	Information flows

#### 8.X.3.1	General

The following information flows are specified for application layer AI/ML Member capability analytics based on clause 8.X.2.

#### 8.X.3.2	Application Layer AI/ML Member capability analytics subscription request

Table 8.X.3.2-1 describes the information flow from the consumer (e.g. VAL server, AIMLE server) as a request or update request for the application layer AI/ML Member capability analytics.

Table 8.X.3.2-1: Application Layer AI/ML Member capability analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M (NOTE) | The identifier of the consumer. |
| Analytics ID | M (NOTE) | The identifier of the analytics event. This ID can be for example "Application layer AI/ML Member capability analytics". |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions. |
| List of VAL users or AI/ML Member IDs | M | The VAL users or AI/ML Member (s) identifiers for which the data/analytics apply. |
| VAL service ID | O | The identifier of the VAL service which is associated with application layer AI/ML Member capability. |
| Application layer AI/ML Member capability attributes | M | The application layer AI/ML Member capability attributes to be analyzed at the ADAE client, e.g. communication capability (e.g. maximum/minimum number of supported active connections). |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time validity | O | The time validity of the subscription request. |
| NOTE: This information element shall not be updated. |  |  |

Editor’s Note:	Whether and what more details about member capability need to be added is FFS.

#### 8.X.3.3	Application Layer AI/ML Member capability analytics subscription response

Table 8.X.3.3-1 describes the information elements for the application layer AI/ML Member capability analytics subscription response from the ADAE server to the consumer.

Table 8.X.3.3-1: Application layer AI/ML Member capability analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement). |

#### 8.X.3.4	Application layer AI/ML Member capability analytics notification

Table 8.X.3.4-1 describes the information flow from the ADAES to the consumer (e.g. VAL Server, AIMLE Server) as a response for the application layer AI/ML Member capability analytics.

Table 8.X.3.4-1: Application layer AI/ML Member capability analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example "Application layer AI/ML Member capability analytics". |
| List of VAL users or AI/ML Member IDs | M | The VAL users or AI/ML Member(s) identifiers for which the data/analytics apply. |
| >VAL user or AI/ML Member ID in the list | M | The VAL user or AI/ML Member identifier for which the data/analytics apply. |
| >>Analytics Output | M | The reported analytics for the application layer AI/ML Member capability. The predictive or statistical parameter on, e.g. communication capability (e.g. maximum/minimum number of supported active connections). |
| >>Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

Editor’s Note:	Whether and what more details about member capability need to be added is FFS.

#### 8.X.3.5	Application Layer AI/ML Member capability data collection subscription request

Table 8.X.3.5-1 describes information elements for the application layer AI/ML Member capability data collection subscription request from the ADAE server to the Data Producer at the ADAE client or the A-ADRF.

Table 8.X.3.5-1: Data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Data Collection Event ID | M | The identifier of the data collection event |
| Data collection requirements | O | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. This ID can be for example "Application layer AI/ML Member capability analytics". |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| List of VAL users or AI/ML Member IDs | M | The VAL users or AI/ML Member (s) identifiers for which the data/analytics apply. |
| VAL service ID list | O | The identifier(s) of the VAL service(s) which is associated with application layer AI/ML Member capability. |
| Application layer AI/ML Member capability attributes | M | The application layer AI/ML Member capability attributes to be analyzed at the ADAE client. |
| Area of Interest | O | The geographical or service area for which the requirement request applies. |
| Time validity | O | The time validity of the request. |

#### 8.X.3.6	Application Layer AI/ML Member capability data collection subscription response

Table 8.X.3.6-1 describes information elements for the Data collection subscription response from the application layer AI/ML Member capability data Producer at the ADAE client or the A-DCCF to the ADAE server.

Table 8.X.3.6-1: Data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the application layer AI/ML Member capability data collection subscription request (positive or negative acknowledgement). |

#### 8.X.3.7	Data Notification

Table 8.X.3.7-1 describes information elements for the Data Notification from the Data Producer to the ADAE server.

Table 8.X.3.7-1: Data notification

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data Producer ID | M | The identity of Data Producer. |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "Application layer AI/ML Member capability analytics". |
| Data Type | M | The type of reported data samples which can be network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non-real time). This also indicates whether data are offline (from A-ADRF or not). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter based on subscription. |

#### 8.X.3.8	Get analytics data request

Table 8.X.3.8-1 describes information elements for the Get application layer AI/ML Member capability analytics request from the analytics consumer to the ADAE server.

Table 8.X.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Analytics ID | M | The identifier of the analytics event. The identifier of the analytics event. This ID can be for example "Application layer AI/ML Member capability analytics". |
| Analytics type | M | The type of analytics, e.g. statistics or predictions. |
| List of VAL users or AI/ML Member IDs | M | The VAL users or AI/ML Member (s) identifiers for which the data/analytics apply. |
| VAL service ID | O | The identifier of the VAL service which is associated with application layer AI/ML Member capability. |
| Application layer AI/ML Member capability attributes | M | The application layer AI/ML Member capability attributes to be analyzed at the ADAE client, e.g. communication capability (e.g. maximum/minimum number of supported active connections). |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time window | O | The start and end time requirements on the generation of the analytics data to be collected. |
| Time validity | O | The time validity of the request. |

Editor’s Note:	Whether and what more details about member capability need to be added is FFS.

#### 8.X.3.9	Get analytics data response

Table 8.X.3.9-1 describes information elements for the Get application layer AI/ML Member capability analytics response from the ADAE server to the consumer.

Table 8.X.3.9-1: Get analytics response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics data request (positive or negative acknowledgement). |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "Application layer AI/ML Member capability analytics". |
| List of VAL users or AI/ML Member IDs | M | The VAL users or AI/ML Member(s) identifiers for which the data/analytics apply. |
| >VAL user or AI/ML Member ID in the list | M | The VAL user or AI/ML Member identifier for which the data/analytics apply. |
| >>Analytics Output | M | The predictive or statistical parameter on, e.g. communication capability (e.g. maximum/minimum number of supported active connections). |
| >>Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| >>Timestamp | O | Timestamp of the collected analytics data. |

Editor’s Note:	Whether and what more details about member capability need to be added is FFS.

* * * Next change * * * *

### 9.2.2	ADAE server APIs

Table 9.2.2-1 illustrates the ADAE server APIs.

Table 9.2.2-1: List of ADAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ADAE_VAL_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_UE-to-UE_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_server-to-server_performance_analytics | Subscribe | VAL server / EES | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_location_accuracy_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_service_API_analytics | Subscribe | VAL server / Subscriber/ API invoker | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_usage_pattern_analytics | Subscribe | VAL server / SEAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_edge_analytics | Subscribe | VAL server / EAS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / EAS / EES | Request / Response |
| SS_ADAES_slice_usage_stats | Get | VAL server | Request / Response |
| SS_ADAES_edge_preparation_analytics | Subscribe | VAL server / ECS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / ECS / EES | Request / Response |
| SS_ ADAE_AIML_member_capability_analytics | Subscribe | VAL server / AIMLE server | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / AIMLE server | Request / Response |

* * * Next change * * * *

### 9.2.Y	SS_ADAE_AIML_member_capability_analytics API

#### 9.2.Y.1	General

API description: This API enables the analytics consumer (e.g. VAL Server, AIMLE Server) to communicate with the ADAE server for requesting or subscribing to application layer AIML member capability analytics.

#### 9.2.Y.2	Subscribe

API operation name: AIML_member_capability_analytics_subscribe

Description: The consumer subscribes for application layer AIML member capability analytics.

Inputs: See clause 8.X.3.2.

Outputs: See clause 8.X.3.3.

See clause 8.X.2.1 for details of usage of this operation.

#### 9.2.Y.3	Notify

API operation name: AIML_member_capability_analytics_notify

Description: The consumer is notified by the ADAE server on application layer AIML member capability analytics.

Inputs: -

Outputs: See clause 8.X.3.4.

See clause 8.X.2.1 for details of usage of this operation.

#### 9.2.Y.4	Get

API operation name: AIML_member_capability_analytics_get

Description: The consumer requests application layer AIML member capability analytics.

Inputs: -See clause 8.X.3.8

Outputs: See clause 8.X.3.9.

See clause 8.X.2.2 for details of usage of this operation.

* * * End of changes * * * *