3GPP TSG-SA WG6 Meeting #62	S6-243619

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243120)

Source:	ETRI, Convida Wireless LLC

Title:	New text for clause 9 Spatial Map (SM) management

Spec:	3GPP TS 23.437 v.0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Byung Jun AHN (bjahn@etri.re.kr)

1. Introduction

This pCR proposes new text for 3GPP TS 23.437 clause 9 Spatial Map (SM) management.

2. Reason for Change

Skeleton of the 3GPP TS 23.437 (S6a240151) has been approved in the previous meeting (3GPP SA6#62 Ad-Hoc). Body text for the approved skeleton is yet to be developed.

3. Conclusions

New text for the skeleton, including clause 9, are required to be provided.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.1.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

* * * Next Change * * * *

4.3	Spatial map management

[AR-4.3-a]	The spatial map management service shall provide mechanisms to produce, get, update, delete and subscribe spatial maps.

[AR-5.4-b]	The spatial map management service shall provide mechanisms to expose spatial map information to the authorized third parties.

[AR-5.4-c]	The spatial map management service shall allow multiple spatial maps in a given three-dimensional space for different vertical applications and consumer’s usage.

[AR-5.4-d]	The spatial map management service shall be able to interact with other SEAL servers and 5GS to support vertical applications.

* * * Next Change * * * *

9	Spatial Map (SM) management

9.1	General

The Spatial Map (SM) management is a SEAL service that offers the spatial map related capabilities to vertical application(s).

* * * Next Change * * * *

9.3	Spatial map management procedures

9.3.1	Create spatial map

Figure 9.3.1-1 depicts the procedure for creating a spatial map. For the request from VAL server or SEAL SM client, as a spatial map consumer, SEAL SMM server creates a spatial map.

Figure 9.3.1-1: Create spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SMM server to create a spatial map. The request includes requestor ID, security credentials, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map.

2)	The SEAL SMM server authorizes VAL server (or SEAL SM client), and validates the request. The SEAL SMM server produces a requested spatial map using the processed sensor data.

NOTE:	For current release, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SMM server gets access to such database to create a spatial map via application specific way which is out of SA6 scope.

3)	The SEAL SMM server sends response message to the requestor with indication of success, assigned spatial map ID and information which includes three-dimensional space defined by the spatial map, stationary or moving objects with attributes of them. Otherwise, the response includes an indication of failure and may include a reason for failure.

9.3.1.1	Information flows for Create spatial map

Editor’s Note: The definition of information flow for create spatial map procedure is FFS.

9.3.2	Discover spatial map

Figure 9.3.2-1 depicts the procedure for an authorized spatial map consumer (VAL server or SEAL SM client) to discover spatial map(s). The service is provided by SEAL SMM server.

Figure 9.3.2-1: Discover spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SMM server to discover spatial map(s). The request includes requestor ID, security credentials, three-dimensional area of interest or localization information to discover spatial maps in the area, optionally discovery filters, i.e., a set of characteristics to search the matching spatial maps when multiple spatial maps are allowed in the area of interest.

NOTE:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this document.

2)	The SEAL SMM server authorizes VAL server (or SEAL SM client), and validates the request. Then the server finds spatial map(s) which matches requested discovery requirements.

3)	The SEAL SMM server sends the discover spatial map response message to the requestor with spatial map ID(s) when matching spatial map is found, otherwise it sends failure indication with reason.

9.3.2.1	Information flows for Discover spatial map

Editor’s Note: The definition of information flow for discover spatial map procedure is FFS.

9.3.3	Get spatial map information

Figure 9.3.3-1 depicts the procedure for an authorized spatial map consumer (VAL server or SEAL SM client) to get the spatial map. The service is provided by SEAL SMM server.

Figure 9.3.2-1: Get spatial map information procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SMM server to get the spatial map information. The request includes requestor ID, security credentials, spatial map ID to get the information for it.

2)	The SEAL SMM server authorizes VAL server (or SEAL SM client), and validates the request. Then the server collects the requested spatial map information.

3)	The SEAL SMM server sends the get spatial map information response message to the requestor with requested spatial map information when successful, otherwise it sends failure indication with reason.

9.3.3.1	Information flows for Get spatial map information

Editor’s Note: The definition of information flow for get spatial map information procedure is FFS.

9.3.4	Update spatial map

Figure 9.3.4-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SMM server updates the spatial map as requested.

Figure 9.3.4-1: Update spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SMM server to update a spatial map. The request includes requestor ID, security credentials, spatial map ID, information on what to update such as updated access control rules and updated spatial map coverage area, etc.

2)	The SEAL SMM server authorizes VAL server (or SEAL SM client) and validates the request. The SEAL SMM server updates the spatial map as requested.

3)	The SEAL SSM server sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID.

9.3.4.1	Information flows for Update spatial map

Editor’s Note: The definition of information flow for update spatial map procedure is FFS.

9.3.5	Delete spatial map

Figure 9.3.5-1 depicts the procedure to delete the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SMM server deletes the spatial map as requested.

Figure 9.3.5-1: Delete spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SMM server to delete the spatial map. The request includes requestor ID, security credentials, a spatial map ID to delete and optionally the reason to delete.

2)	The SEAL SMM server authorizes VAL server (or SEAL SM client) and validates the request. The SEAL SMM server deletes the spatial map as requested.

3)	The SEAL SMM server sends response message to the requestor with the result of success or failure.

9.3.5.1	Information flows for Delete spatial map

Editor’s Note: The definition of information flow for delete spatial map procedure is FFS.

9.3.6	Subscribe spatial map event

Figure 9.3.6-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to subscribe the spatial map event. The service is provided by SEAL SMM server. Spatial map consumer can get notification information when the the SMM server detects subscribed event.

Figure 9.3.6-1: Subscribe spatial map event procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SMM server to subscribe spatial map event. The request includes requestor ID, security credentials, a notification endpoint, a spatial map ID, event to subscribe which triggers the notification. The requestor (VAL server or SEAL SM client) may subscribe to “changes of objects” events to indicate the SMM server to notify the requestor when changes of the objects are detected. The notification triggering criteria for “changes of objects” event may include added or removed objects, position or direction changes of the spatial map objects.

2)	The SEAL SMM server authorizes VAL server (or SEAL SM client), and validates the request. Then server add the requestor into the subscriber list of the event. If the subscribed event is “changes of objects”, SMM server starts monitoring the status of the spatial map objects.

3)	The SEAL SMM server sends response message with the result. For the successful subscribe request, a subscription ID is included in the message. Otherwise, the response message includes an indication of failure and may include a reason for failure.

9.3.6.1	Information flows for Subscribe spatial map

Editor’s Note: The definition of information flow for subscribe spatial map procedure is FFS.

9.3.7	Notify spatial map event

Figure 9.3.7-1 depicts the notify spatial map event operation between the authorized spatial map consumer (VAL server or SEAL LM client) and SEAL SMM server.

Figure 9.3.7-1: Notify spatial map event

1)	The SEAL SMM server detects an event that triggers the notification to the subscribers for the corresponding event. If the subscribed event is “changes of objects” and the SMM server detects any changes of the spatial map objects, SMM server determines whether the detected event matchs triggering criteria and notifies subscribers accordingly.

2)	The SEAL SMM server notifies the VAL servers (or SEAL SM clients) which subscribed the corresponding event. The notification includes a subscription ID and event-specific information. If the event is “changes of objects”, spatial map notification message includes the information on the detected triggering event, e.g. added or removed objects, position or direction changes of the objects in the spatial map etc.

9.3.7.1	Information flows for Notify spatial map event

Editor’s Note: The definition of information flow for notify spatial map event is FFS.

9.3.8	Unsubscribe spatial map

Figure 9.3.8-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to unsubscribe from the spatial map event. The service is provided by SEAL SMM server.

Figure 9.3.8-1: Unsubscribe spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SMM server to unsubscribe from spatial map event. The request includes requestor ID, security credentials, spatial map ID and the subscription ID for the event.

2)	The SEAL SMM server authorizes VAL server (or SEAL SM client), and validates the request. Then SMM server removes the requestor’s subscription ID from the subscriber list for the spatial map event.

3)	The SEAL SMM server sends unsubscribe spatial map event response message with the result. If successful, the response includes an indication of success. Otherwise, the response includes an indication of failure and may include a reason for failure.

9.3.8.1	Information flows for Unsubscribe spatial map

Editor’s Note: The definition of information flow for unsubscribe spatial map procedure is FFS.

* * * End of Change * * * *