3GPP TSG-SA WG6 Meeting #62	S6-243600

Maastricht, The Netherlands, 19th – 23rd August 2024

Source:	Ericsson

Title:	Support Transfer of Intermediate AIML Operation Information

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added descriptions and procedures for supportting transfer of intermediate AIML operation information to TS 23.482.

2. Reason for Change

Solution#30 is recommended for normative work normative work to support transfer of intermediate AIML operation information. See more details in 3GPP TR 23.700-82. This paper proposes to add descriptions and procedure for supportting transfer of intermediate AIML operation information to TS 23.482.

3. Conclusions

This paper proposes to add descriptions and procedure for supportting transfer of intermediate AIML operation information to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.x	Support AI/ML Task Transfer

This clause describes procedure for supportting AI/ML task transfer.

### 8.x.1	General

The following clauses specify procedures to support AI/ML task transfer.

### 8.x.2	Procedures for AI/ML task transfer

Due to various reasons (e.g., changes of available resource, changes of available time), an AI/ML member (e.g., AIMLE Client, VAL Client) finds itself cannot finish the assigned AI/ML task during the performing process. Here AI/ML member means the participant of an AI/ML task. To save resource and time, the AI/ML member (source AI/ML member) decides to transfer the intermediate AI/ML information (e.g., the intermediate AI/ML operation status and results) to another AI/ML member (target AI/ML member) for further operations to complete the AI/ML task.

#### 8.x.2.1	Request AIMLE Server assist AI/ML task transfer

Pre-conditions:

-	The information of target AI/ML member (e.g. another AIMLE Client or VAL Client different from the source AI/ML member) is unknown at the source AI/ML member. The source AI/ML member decides that assist from the AIMLE Server is needed.

Figure 8.x.2.1-1: Procedure for requesting AIMLE Server assist AI/ML task

transfer

Figure 8.x.2.1-1 illustrates the procedure for requesting AIMLE Server assist AI/ML task transfer. The corresponding procedure in detail is as follows:

- The source AI/ML member (e.g. an AIMLE Client) sends request to AIMLE Server for assisting AI/ML task transfer. The request message contains the information as specified in Table 8.x.3.2-1.

Editor’s Note:	The procedure can also be used for transfer AI/ML services (e.g., ML model performance evaluation, ML model update) between AI/ML members. The details is FFS.

- The AIMLE Server discovers other AI/ML members (e.g., other AIMLE Clients) directly or via ML repository, and selects one or more target AI/ML member(s) based on the request from the source AI/ML member.

- The AIMLE Server sends request to the selected target AI/ML member(s) for intermediate AI/ML operation information transfer, as defined in clause 8.X.2.2.

- The AIMLE Server determines the transfer mode, i.e. transfer with server-controlled or transfer directly from the source AI/ML member to the target AI/ML member.

- If target AI/ML member(s) is discovered based on the steps 2-4, the AIMLE Server generates assistance information for the AI/ML task transfer from the source AI/ML member to the target AI/ML member(s) (e.g. time window for the transfer).

- The AIMLE Server responses to the source AI/ML member with the information of the target AI/ML member(s), and assistance information generated in step 5. The response message contains the information as specified in Table 8.x.3.3-1.

#### 8.x.2.2	Request Target AI/ML member for AI/ML task transfer

Figure 8.x.2.2-1: Procedure for requesting target AI/ML member for AI/ML task transfer

Figure 8.x.2.2-1 illustrates the procedure for requesting target AI/ML member for AI/ML task transfer. The corresponding procedure in detail is as follows:

- The AIMLE Server sends request to the selected target AI/ML member(s) for AI/ML task transfer. The request message contains the information as specified in Table 8.x.3.4-1.

- The target AI/ML member(s) authenticates and authorizes the request from the AI/ML Enablement Server. If the request is authorized, the target AI/ML member(s) performs step 3.

- The target AI/ML member(s) responses to the AIMLE Server with information as specified in Table 8.x.3.5-1. If the information of the source AI/ML member is contained in the request message in step 1, the target AI/ML member(s) may determine to interact with the source AI/ML member directly based on its local configuration.

#### 8.x.2.3	Direct AI/ML task transfer

Pre-conditions:

-	The information of target AI/ML member (e.g., another AIMLE Client or VAL Client different from the source AI/ML member) is assumed to be known at the source AI/ML member, or the source AI/ML member may get the target AI/ML member information via ML repository. The source AI/ML member decides to transfer the AI/ML task to the target AI/ML member.

Figure 8.x.2.3-1: Procedure for direct AI/ML task transfer

Figure 8.x.2.3-1 illustrates the procedure for direct AI/ML task transfer. The corresponding procedure in detail is as follows:

- The source AI/ML member (e.g. an AIMLE Client) sends request to the target AI/ML member(s) for direct AI/ML task transfer. The request message contains the information as specified in Table 8.x.3.6-1.

- The target AI/ML member(s) authenticates and authorizes the request from the AIMLE Server. If the request is authorized, the target AI/ML member(s) performs the next steps.

- The target AI/ML member(s) checks its availability for continue the AI/ML operations (e.g. available resource and time).

- The target AI/ML member(s) responses to the source AI/ML member with information as specified in in Table 8.x.3.7-1.

- The source AI/ML member interacts with the target AI/ML member(s) to performs AI/ML task transfer to the target AI/ML member(s).

Editor’s Note:	The intermediate AI/ML operation information may be transferred directly between the source and target AI/ML members, or via the AIMLE Server (e.g. AIMLE Server controlled information transfer between two AI/ML members). How the AI/ML task transfer via the AIMLE Server is FFS.

### 8.x.3	Information flows

#### 8.x.3.1	General

The following information flows are specified for supporting AI/ML task transfer. In the following clauses 8.x.3.2-8.x.3.7, the source AI/ML member and the target AI/ML member could be e.g. VAL UE, AIMLE Client.

#### 8.x.3.2	AIMLE Server assist AI/ML task transfer request

Table 8.x.3.2-1 describes information elements for the AIMLE Server assist AI/ML task transfer request from the source AI/ML member to the AIMLE Server.

Table 8.x.3.2-1: AIMLE Server assist AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL UE ID (the source AI/ML member). |
| VAL service ID | O | The identifier of the VAL service for which the assistance informaiton is requested. |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Transfer Requirements | M | The requirement on the transfer (e.g. data volume, except time for finishing the transfer, requirement on the target AI/ML member on available resource and time). |
| Transfer Mode | O | The transfer mode (e.g. server-controlled AI/ML task transfer) |
| Time validity | O | The time validity of the request. |

#### 8.x.3.3	AIMLE Server assist AI/ML task transfer response

Table 8.x.3.3-1 describes information elements for the AIMLE Server assist AI/ML task transfer response from the the AIMLE Server to the source AI/ML member.

Table 8.x.3.3-1: AIMLE Server assist AI/ML task transfer response

| Information element | Status | Description |
| --- | --- | --- |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task to be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Assistance Information | M | The assistance information (e.g. time window for the transfer). |
| Transfer Mode | O | Indication the transfer mode (e.g. direct transfer) |

#### 8.x.3.4	AI/ML task transfer request

Table 8.x.3.4-1 describes information elements for the AI/ML task transfer request from the AIMLE Server to the target AI/ML member.

Table 8.x.3.4-1: AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL Server. |
| VAL UE ID | M | The identifier of the VAL UE (the source AI/ML member). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Transfer Requirements | M | The requirement on the transfer (e.g. data volume, except time for finishing the transfer, requirement on the target AI/ML member on available resource and time). |
| Transfer Mode | O | The transfer mode (e.g. server-controlled AI/ML task transfer) |
| Time validity | O | The time validity of the request. |

#### 8.x.3.5	AI/ML task transfer Response

Table 8.x.3.5-1 describes information elements for the AI/ML task transfer response from the target AI/ML member to the AIMLE Server.

Table 8.x.3.5-1: AI/ML task transfer response

| Information element | Status | Description |
| --- | --- | --- |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task to be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Information for transfer | M | The informaiton for transfer task (e.g. available time for continue performing the AI/ML operation). |
| Transfer Mode | O | Indication the transfer mode (e.g. direct transfer) |

#### 8.x.3.6	Direct AI/ML task transfer request

Table 8.x.3.6-1 describes information elements for the direct AI/ML task transfer request from the source AI/ML member to the target AI/ML member.

Table 8.x.3.6-1: Direct AIMLE Server assist AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL UE (the source AI/ML member). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Transfer Requirements | M | The requirement on the transfer (e.g. data volume, except time for finishing the transfer, requirement on the target AI/ML member on available resource and time). |
| Time validity | O | The time validity of the request. |

#### 8.x.3.7	Direct AI/ML task transfer Response

Table 8.x.3.7-1 describes information elements for the direct AI/ML task transfer response from the target AI/ML member to the source AI/ML member.

Table 8.x.3.7-1: Direct AI/ML task transfer response

| Information element | Status | Description |
| --- | --- | --- |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task to be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| Information for transfer | M | The informaiton for transfer task (e.g. available time for continue performing the AI/ML operation). |

Editor’s Note:	Whether need additional IEs or reduction of IEs is FFS.

* * * Next Change * * * *

### 9.Y.A	AIMLE AI/ML Task Transfer API

Editor’s Note:	This clause provides the corresponding AIMLE AI/ML task transfer API.

# *** End of Changes ***