3GPP TSG-SA WG6 Meeting #62	S6-243704

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243672)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0313 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dynamic geofencing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduce a new feature for dynamic geofencing, as concluded in 23.700-72 (KI#1, sol#4). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The monitoring location deviation procedure is enhanced to support dynamic geofencing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting dynamic geofencing feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.11.1, 9.3.11.2 9.3.2.11, 9.3.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.11.1	General

The VAL server requests the Location Management Server to monitor the location of the VAL UE in relation to an area of interest. The LMS fetches the VAL UE's location information periodically from 3GPP core network as specified in 3GPP TS 23.502 [11] and also, using the Location Information procedures specified in clause 9.3.7 and clause 9.3.10. With the periodic location information of the UE from 3GPP core network and SEAL procedures, the LMS server evaluates the current location of the VAL UE in relation to the area of interest configured by the VAL server. If subscribed, the VAL server is notified by the LMS server when the VAL UE relationship (e.g. inside or outside) to the area of interest changes along with current location information of the VAL UE.  The area of interest can be predefined or mobile where the latter one is used to support dynamic geofencing function.

#### 9.3.11.2	Monitoring Location Deviation procedure

Figure 9.3.11.2-1 describes the procedure for monitoring the VAL UE's location in a given area of interest.

Pre-condition:

-	The LM server authorized to consume the 3GPP core network service (Monitoring events provided by NEF as specified in 3GPP TS 23.502 [11]).

Figure 9.3.11.2-1: Monitoring VAL UE's location at a given location

1.	The VAL server sends Monitor Location Subscription Request to LM server including target VAL UE Identifier(s), predetermined or dynamic area of interest information, notification interval and notification URI where the VAL server intends to receive the notifications from LM server regarding target VAL UE's presence in a given area, interested notification information, dynamic geofencing conditions.

-	"Area of interest" is the location information, which the VAL server wishes to monitor the target VAL UE's location adherence. This parameter can include an area of interest information and other relevant parameters. In the case of dynamic area of interest, it includes a reference VAL UE Identifier and its proximity range.

-	"Notify_Interval" represents the periodic interval in which the LM server needs to notify target VAL UE's location information to the VAL server. When the target VAL UE moves away from the "Area of interest", then the LM server ignores the "Notify_Interval" and sends the location notification to the VAL server immediately.

-	"dynamic geofencing conditions" includes a temporal condition(s) (e.g. time of day and day of week), connectivity condition(s) (e.g. operator information, roaming status, access type) and/or a spatial condition(s) (e.g. location area(s)) for the reference VAL UE. For instance, the monitoring is valid when a reference UE is served by operator A without using satellite in location area X from 7:00am to 9:00am on working days.

2.	If dynamic area of interest information and dynamic geofencing conditions are received, the LM server requests 3GPP CN (e.g. NEF service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [11]) to monitor the reference UE location change, access type change and roaming status change, in order to determine spatial dynamicity in area of interest related to the target UE(s). If spatial condition is received, the LM server also requests 3GPP CN (i.e. NEF service for area of interest monitoring as specified in 3GPP TS 23.502 [11]) to know whether the reference UE is in the VAL server's predetermined area (e.g. PLMN x). The LM server will process the monitoring considering the received dynamic geofencing conditions for the reference UE.

3.	If dynamic area of interest information is received, the LM server requests 3GPP CN (i.e. NEF service for loss of connectivity as specified in 3GPP TS 23.502 [11]) for both target UE(s) and reference UE.

4a.	LM server processes the Area of interest information in the request, and then subscribes to target UE location monitoring as specified in 3GPP TS 23.502 [11] with appropriate parameters mapping. Based on the subscription, the LM server receives the target VAL UE location information periodically from the 3GPP core network. The LM server also invokes NEF service for Area of Interest monitoring as specified in clause 5.3.3 of 3GPP TS 23.256 [52] to know whether the target VAL UE is in the VAL server's predetermined or dynamic area of interest.

4b.	LM server shall use the Location information procedures as specified in clause 9.3.7 and clause 9.3.10, to periodically obtain the target VAL UE location information. Based on the geographic information from the VAL server, the LM server may determine to additionally include the positioning methods in SEAL LMS procedures to obtain location information.

4c.	LMS may periodically obtain the target VAL UE location information from the 3rd party location management server based on the geographic information from the VAL server.

NOTE 1:	How the LMS obtains the target UE location from the 3rd party location management server is out of scope of this specification.

NOTE 2:	Step 4a to 4c are performed for each target VAL UE if multiple target VAL UE Identifiers are provided by the VAL server.

5.	LM server, after successful subscription according to steps 2~4, sends Monitor Location Subscription response, indicating that the LM server accepts VAL server's request and will monitor the location of the target VAL UE(s) to verify if the target VAL UE(s) is in the area of interest.

6a and 6b. LM server processes the location information received from SEAL Location Information procedures, the 3GPP core network and/or the 3rd party location management server, and validates the information. If the location information is matching, then the LM server shall check if the VAL UE's current location is within the predetermined or dynamic area of interest received in step 1.

NOTE 3:	The location information received from the 3GPP core network and from LM client does not necessarily be exactly the same, the LM server can aggregate the location information if there is no big deviation.

6c.	LM server verifies its determined target VAL UE location status (i.e. presence or absence with regard to the predetermined area of interest) with the retrieved 3GPP core network result from the NEF monitoring service for Area of Interest. The LM server will continue with step 7, step 8 and step 9 as applicable.

7.	If the location information received from Location management client, the 3GPP core network and/or the 3rd party location management server do not match or the NEF reported Area of Interest monitoring result does not match with the determined target VAL UE location status (present/absence) in LM server, then the LM server shall consider the target VAL UE as outside from its specified area of interest and shall notify ("Notify Mismatch Location" message) the VAL server of the same, including target VAL UE ID and the location information from LM server, the 3GPP core network and/or the 3rd party location management server in the notification message.

8.	If the target VAL UE's current location is from Location management client, the 3GPP core network and/or the 3rd party location management server matches, and either not in the predetermined area of interest received from VAL server in Monitor Location Subscription Request message or not within the proximity range of the reference UE, and such out of area of interest result is also aligned with the NEF reported area of interest monitoring result, then the LM server considers the target VAL UE as outside from its specified area of interest and shall notify the VAL server that the target VAL UE's current location is outside of area of interest and target VAL UE ID in "Notify Absence" message.

9.	When the target VAL UE's current location is either in the predetermined area of interest or in the proximity range of the reference UE, and such in area of interest result is also aligned with the NEF reported area of interest monitoring result, then the LM server shall notify ("Notify Presence" message) the VAL server periodically, according to the "Notify_Interval" value in "Monitor Location Subscription Request" message, indicating the VAL server that the target VAL UE is within the area of interest, along with target VAL UE's current location information.

10.	If the LM server is notified by the 3GPP CN about loss of connectivity for the reference UE or target UE(s), then the LM server notifies ("Notify Unknown" message with detailed reason) the VAL server indicating that the LM server does not know whether the target UE is "Presence" or “Absence", therefore the dynamic geofencing monitoring for the impacted target UE(s) is suspended.

NOTE 4:	Upon "Notify Unknown", the VAL server can remind end user to check status for the reference/target UE, which is out of 3GPP scope.

Editor's note:	Detailed handling for “loss of connectity” in LM server is FFS. For instance, whether to address the case when reference UE and target UE(s) both lose connectivity (e.g., using UE-only operation for SL/Ranging).

Editor’s note:	It is FFS whether to use SL/Ranging mechanism to further figure out the target UE(s) are present or absent in the proximity range of the reference UE when the reference UE (or target UE(s)) lost the connectivity with CN but the target UE(s) (or the reference UE) can still be reached via ProSe.

NOTE 5:	Notifications in step 7-10 can be subject to explicit notification indication from the VAL server, for instance, the VAL server can be interested to know only "Notify Absence" and "Notify Unknown".

* * * Next Change * * * *

#### 9.3.2.11	Monitor Location Subscription Request

Table 9.3.2.11-1 describes the information flow from the VAL server to the LM server for monitoring the location deviation of the VAL UE(s) or for updating an existing monitor location subscription.

Table 9.3.2.11-1: Monitor Location Subscription Request

| Information element | Status | Description |
| --- | --- | --- |
| Target Identity | M | Identifier of the target VAL users or VAL UEs whose location monitoring is requested to be monitored in a given location (see NOTE). |
| Area of Interest | M | Geographic area location information where the VAL server wishes to monitor the VAL UE's location adherence. This could be geographical co-ordinates or VAL service area identified by the VAL service area ID. Alternatively, it includes identifier of the reference VAL user or VAL UE and its proximity range. |
| Notify Interval | M | Periodic time interval in which the LM server needs to notify the target VAL UE's location information to the VAL server. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about target VAL UE's location. |
| Timeout | O | A timeout period when subscription response is not received. |
| Interested notification information | O | One or more interested notification events (e.g. Notify Absence, Notify Presence, Notify Mismatch Location and Notify Unknown) the VAL server can subscribe to. |
| Dynamic geofencing conditions | O | Temporal, connectivity and/or spatial conditions for the reference VAL UE. |
| NOTE: This information element shall not be updated via monitor location subscription update procedure in clause 9.3.2.12a. |  |  |

* * * Next Change * * * *

#### 9.3.2.13	Notify Location Monitoring Event

Table 9.3.2.13-1 describes the information flow from LM server to the VAL server for notification of location monitoring events.

Table 9.3.2.13-1: Notify Location Monitoring Event message

| Information element | Status | Description |
| --- | --- | --- |
| Event | M | Information of the event to be reported. The event shall be one of the following: "Notify_Mismatch_Location" – When the location information of the target VAL UE, from the location management client and the core network do not match, or when the LM server determined area of interest result based on LM client location and the core network reported area of interest result do not match. "Notify_Absence" – When the target VAL UE's current location is deviating from the VAL server's area of interest information. "Notify_Presence" – When the target VAL UE's current location is within the VAL server's area of interest information. "Notify_Unknown" – When the LM server cannot determine whether the target VAL UE's current location is within the VAL server's dynamic area of interest information or not. |
| Identity | M | Identifier of the VAL UE whose location information is reported. |
| Location | M | Current location of the target VAL UE. |

* * * End of Changes * * * *