3GPP TSG-SA WG6 Meeting #64	S6-245335

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245052)

Source:	Ericsson

Title:	Pseudo-CR on clauses A.1, A.2 and A.6

Spec:	3GPP TR 23.700-09 v 0.3.0

Agenda item:	8.8

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR address KI#1 with the focus on clauses A.1, A.2 and A.6.

2. Reason for Change

This pCR looks into having clauses A.1, A.2 and A.6 of TS 23.180 agnostic to the network.

3. Conclusions

This pCR updates clauses A.1, A.2 and A.6.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-09 v 0.3.0

* * * First Change * * * *

## 6.x	Solution #X: Clauses A.1, A.2 and A.6 (Involved business relationships)

### 6.x.1	General

This solution addresses key issue #1 with the focus on clauses A.1, A.2 and A.6 to be written in a generic way to be a generic IOPS solution.

### 6.x.2	Solution

#### 6.x.2.1	Solution for clause A.1

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution.

# A.1	General

This Annex provides information about the static configuration data needed for the support of MC services in the IOPS mode of operation.

The configuration data belong to one of the following categories:

-	MC service UE configuration data (see subclause A.2);

-	MC service user profile configuration data (see subclause A.3);

-	MC service group configuration data (see subclause A.4); and

-	MC service configuration data (see subclause A.5).

-	Location user profile configuration data (see subclause A.8)

The configuration data in each configuration category corresponds to a single instance of the category type i.e. the MC service UE, MC service group, MC service user and MC service configuration data refers to the information that will be stored against each MC service UE, MC service group, MC service user and MC service.

NOTE:	The configuration data described in this Annex together with corresponding configuration data provided in 3GPP TS 23.280 [3], 3GPP TS 23.289[23.289], 3GPP TS 23.289 [23.289] 3GPP TS 23.379 [5] and 3GPP TS 23.282 [6] represent the complete set of data for each configuration data category element.

The columns in the tables have the following meanings:

-	Reference: the reference of the corresponding requirement in 3GPP TS 22.346 [9] and 3GPP TS 22.280 [11] or the corresponding clause from either the present document or the referenced document.

-	Parameter description: A short definition of the semantics of the corresponding item of data, including denotation of the level of the parameter in the configuration hierarchy.

-	When it is not clear to which functional entities the parameter is configured, then one or more columns indicating this are provided where the following nomenclature is used:

-	"Y" to denote "Yes" i.e. the parameter denoted for the row needs to be configured to the functional entity denoted for the column.

-	"N" to denote "No" i.e. the parameter denoted for the row does not need to be configured to the functional entity denoted for the column.

Parameters within a set of configuration data have a level within a hierarchy that pertains only to that configuration data. The level of a parameter within the hierarchy of the configuration data is denoted by use of the character ">" in the parameter description field within each table, one per level. Parameters that are at the top-most level within the hierarchy have no ">" character. Parameters that have one or more ">" characters are child parameters of the first parameter above them that has one less ">" character. Parent parameters are parameters that have one or more child parameters. Parent parameters act solely as a "grouping" of their child parameters and therefore do not contain an actual value themselves i.e. they are just containers for their child parameters.

Each parameter that can be configured online shall only be configured through one online reference point. Each parameter that can be configured offline shall only be configured through one offline reference point. The most recent configuration data made available to the MC service UE shall always overwrite previous configuration data, irrespective of whether the configuration data was provided via the online or offline mechanism.

#### 6.x.2.2	Solution for clause A.2

# A.2	MC service UE configuration data

MC service UE configuration data has to be known by an MC service UE after MC service authorization. The CSC-4 reference point, specified in 3GPP TS 23.280 [3], is used for configuration between the configuration management server and the configuration management client on the MC service UE when the MC service UE is on-network.

MC service UE configuration data can be configured offline using the CSC-11 reference point specified in 3GPP TS 23.280 [3], and 3GPP TS 23.289 [23.289].

Within each MC service, the MC service UE configuration data can be the same or different across MC service UEs.

The MCPTT UE configuration data specified in table A.2-1 in 3GPP TS 23.379 [5] is also used, as needed, in the IOPS mode of operation for the MCPTT service.

The MCData UE configuration data specified in table A.2-1 in 3GPP TS 23.282 [6] is also used, as needed, in the IOPS mode of operation for the MCData service.

#### 6.x.2.2	Solution for A.6

# A.6	Initial MC service UE configuration data

Initial MC service UE configuration data is essential to the MC service UE to successfully connect to the MC system, as described in 3GPP TS 23.280 [3] , and 3GPP TS 23.289 [23.289].

The configuration data defined in table A.6-1 is additionally provided to the MC service UE's clients to successfully connect to the IOPS MC system in the IOPS mode of operation. The MC service UE's clients (e.g. MC service client, IOPS connectivity client) obtain the data during the bootstrap process (described in clause 10.1.1 in 3GPP TS 23.280 [3]), and can be configured on the MC service UE offline using the CSC-11 reference point or via other means e.g. as part of the MC service client's provisioning on the UE, using a device management procedure.

Table A.6-1: Initial MC service UE configuration data (IOPS)

| Reference | Parameter description |
| --- | --- |
| Clause 5.4.3 | PDN connectivity information in IOPS (NOTE 1) |
|  | > IOPS HPLMN ID and optionally IOPS VPLMN ID to which the data pertains |
|  | > MC services PDN in IOPS |
|  | >> APN |
|  | >> PDN access credentials |
|  | PDU session information in IOPS (NOTE 2) |
|  | > IOPS HPLMN ID and optionally IOPS VPLMN ID to which the data pertains |
|  | > MC service PDU in IOPS |
|  | >> DNN |
|  | >> PDU access credentials |
|  | >Default configured slice(s) S-NSSAI(s) |
|  | Application plane server identity information |
|  | > Indication of whether the UE shall use IPv4 or IPv6 for the support of MC services in IOPS |
|  | > IOPS MC connectivity function |
|  | >> Server URI |
| NOTE 1: These parameters shall only be used to access via local IOPS EPS system. NOTE 2: These parameters shall only be used to access via local IOPS 5GS system. |  |

#### 6.x.2.3	solution for location user profile configuration

A location user profile configuration data is needed to be included in 3GPP TS 23.180 to reflect the configurations related to location information reporting. The location user profile configuration in annex A.8 of 3GPP TS 23.280 [3] are applicable as needed in the IOPS mode of operation.

### 6.x.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.x.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 into an access generic IOPS solution.

* * * End of Changes * * * *