3GPP TSG-SA WG6 Meeting #64	S6-245578

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245192)

Source:	Convida Wireless LLC

Title:	Remove ENS for ML model management

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, Liu.Lu@convidawireless.com

1. Introduction

This contribution proposes updates to clause 8.11 ML model management to align with other procedures and address Editor’s Notes.

2. Reason for Change

In Table 8.7.3.1-2 AIMLE client profile, supported AI/ML model types and AIMLE client capabilities are specified to indicate whether the AIMLE client is able to train or use a ML model. These parameters should also be reflected in the ML model information in Table 8.14.3.6-1 to indicate the requirements of using a ML model, so that the AIMLE server can determine or select the appropriate ML model for the AIMLE client, as in clause 8.2 ML model retrieval and clause 8.3 ML model training procedures. It is proposed to add “model type” and “model usage requirements” to ML model information.

In addition, it is suggested to remove the following Editor’s Notes:

- If the owner of the model doesn’t specify the list of allowed vendors, then the AIMLE server can determine whether the model can be shared based on the Security and access requirements defined in the ML model information. How to get consent from model owner is out of scope. Therefore, it is suggested the following Editor’s Note to be removed.

Editor’s Note 1: If List of allowed vendors is not present then how to get consent from model owner to share the model to the requestor is FFS.

- Whether a model is discoverable or not can be specified in Security and access requirements in the ML model information. Therefore, it is suggested the following Editor’s Note to be removed.

Editor’s Note 2: How to consider whether model is discoverable or not is FFS.

- No other parameters for ML model phase have been defined or referenced in the related procedures. Therefore, it is suggested the following Editor’s Note to be removed.

Editor’s Note:	More parameters for ML model phase is FFS.

- ML model ID is included in the ML model information, which can be included in the filtering criteria. In a ML model information discovery procedure, the AIMLE server does not need to differentiate whether the model to be discovered will be used as a base model or not. Therefore, Base Model ID does not need to be included. It is suggested the following Editor’s Note to be removed.

Editor’s Note: The inclusion of ML model ID and Base Model ID are FFS.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

### 8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Pre-condition:

1.	Either AIMLE consumer has requested to discover ML model or AIMLE server decides to discover model for training.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the requestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model. Optionally, the ML repository can initiate a model transfer process to the consumer.

NOTE 1:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details is indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enabler consumer can rely on model repository to get the model details.

NOTE 2:	The model transfer process mentioned in step 2 can use the AI/ML data transfer policies.

* * * Next Change * * * *

### 8.11.4	Information flows

#### 8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| ML model | O (NOTE) | Provides the ML model. |
| ML model address | O (NOTE) | Represents the ML model address that can be used by Model repository to download the ML model. |
| ML model information | M | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE: At least one of these information elements shall be provided. |  |  |

Table 8.11.4.1-2: ML model information

| Information element | Status | Description |
| --- | --- | --- |
| ML model identifier | M | An identifier for the ML model |
| Analytics ID | O | Represents analytics ID for which the model can be used. |
| VAL service ID(s) | O | Identify the VAL service ID(s). |
| Domain | O | Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location predication, etc.). |
| List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to retrieve the ML model and thereby also are interoperable to the model. |
| ML model interoperability information | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| ML Model phase | O (NOTE 1) | Represents the ML model phase, e.g., in training, trained, re-training, deployed. |
| > Observed accuracy | O (NOTE 2) | Provides accuracy of the model (if ML model is in trained phase). |
| ML model storage and discovery requirements | O (NOTE 1) | Represents the requirements for the ML repository for the ML model storage and discovery. |
| > Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| > Security and access requirements | O | Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs). |
| ML model usage requirements | O | Represents the requirements for using the ML model (e.g. for inference or for training). The requirements are used by the AIMLE server to determine whether an AIMLE client is capable of using the model based on comparing the requirements with information in the AIMLE client profile in Table 8.7.3.1-2. |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available. |  |  |

* * * Next Change * * * *

#### 8.11.4.3	ML model information discovery request

Table 8.11.4.3-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information discovery.

Table 8.11.4.3-1: ML model information discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| Filtering criteria | M | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

* * * End of Changes * * * *