3GPP TSG-SA WG6 Meeting #64	S6-245593

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245191)

Source:	Convida Wireless LLC

Title:	AIMLE in edge data networks

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, Liu.Lu@convidawireless.com

1. Introduction

Add AIMLE service support in edge data networks.

2. Reason for Change

The conclusion for key issue #1 in TR 23.700-82 specified that solution #23 will be considered for normative work to support AIMLE services in edge computing scenarios. AIML operations can be offloaded to UEs in the network and managed by AIMLE servers hosted on edge data networks. In such a scenario, UEs that are mobile may need assistance from AIMLE servers on the EDNs to manage service continuity issues when UEs cross service area boundaries during AIML operations. Multiple AIMLE servers may communicate with each other using the AIML-E interface.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[7]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

…

[y]	3GPP TS 23.558: “Architecture for enabling Edge Applications”.

* * * Next Change * * * *

## 8.x	AIMLE context transfer

### 8.x.1	General

This clause describes AIMLE context transfer procedure between AIMLE servers (over AIML-E reference point).

### 8.x.2	Procedure

Pre-conditions:

1.	Each edge AIMLE server manages the AIMLE clients within its service area to perform AI/ML operations.

2.	A UE associated with an AIMLE client moves from a source service area (managed by a source edge AIMLE server) to a target service area (managed by a target edge AIMLE server). The transition triggers application context relocation (ACR) procedure between the two edge AIMLE servers (as source EAS and target EAS, respectively) as specified in 3GPP TS 23.558 [y].

Figure 8.x.2-1. AIMLE context transfer

- The source edge AIMLE server sends an AIMLE context transfer request to the target AIMLE server as described in Table 8.x.3.1-1. The request includes AIMLE context information which is generated based on the responses/notifications received from the transitioned AIMLE client (e.g. step 7 of clause 8.12.2 or step 3 of clause 8.20.1).

The AIMLE context information is used by the target edge AIMLE server to determine whether the information (e.g. AI/ML operation output/result) received from the AIMLE client should be transferred to the source edge AIMLE server (or another edge AIMLE server that has been associated with the AIMLE client). For example, the target AIMLE server can forward the AIMLE service results received from the AIMLE client to the source AIMLE server if the results are only applicable to the source service area or the AIMLE client is part of a split operation pipeline formed in the source service area.

- The target edge AIMLE server sends an AIMLE context transfer response to the source edge AIMLE server as described in Table 8.x.3.2-1.

### 8.x.3	Information flows

#### 8.x.3.1	AIMLE context transfer request

Table 8.x.3.1-1 shows the AIMLE context transfer request that is sent by a source AIMLE server to a target AIMLE server.

Table 8.x.3.1-1: AIMLE context transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g. AIMLE server). |
| AIMLE Context Information | M | The AIMLE context information as described in Table 8.x.3.1-2. |

Table 8.x.3.1-2: AIMLE context information

| Information element | Status | Description |
| --- | --- | --- |
| AIMLE client ID | M | The identifier of the AIMLE client associated with the context. |
| Current managing AIMLE server | O | The identifier of the AIMLE server that is currently managing the AIMLE client, i.e. the AIMLE server associated with the service area that the AIMLE client is currently in. |
| Previous managing AIMLE server | O | List of identifiers of AIMLE servers that have been associated with the AIMLE client. The list is populated by adding the identifier of the source edge AIMLE server whenever the UE transitioned from a source edge area to a target edge area. |
| AIMLE service status | O | Status of the AIML operations (task) at the AIMLE client, e.g. “active”, “paused”, “completed”, percentage of completion. |
| AIMLE service results | O | Results of the AIML operations (task) performed by the AIMLE client. |
| AIMLE service applicability | O | Applicability information of the AIML operations performed by the AIMLE client, e.g. the operation results are applicable within a certain edge service area, the operations are applicable within a certain split operation pipeline. |

#### 8.x.3.2	AIMLE context transfer response

Table 8.x.3.2-1 shows the AIMLE context transfer response that is sent by the target edge AIMLE server to the source edge AIMLE server.

Table 8.x.3.2-1: AIMLE context transfer response

| Information element |  | Status |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Status |  | M |  | The status for the request: success or fail. |  |
| Successful response |  | O |  | Indicates that the request was successful. |  |
| Failure response |  | O |  | Indicates that the request failed. |  |
| > Cause |  | O |  | Indicates the cause of request failure |  |

* * * Next Change * * * *

### 9.2.z	AIMLE context transfer API

#### 9.2.z.1	General

Table 9.2.z.1-1 illustrates the API for AIMLE context transfer. This API enables an AIMLE server to communicate AIMLE context to another AIMLE server in edge scenarios.

Table 9.2.z.1-1: Aimles_ContextTransfer API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimles_ContextTransfer | Request | Request/Response | AIMLE Server |

#### 9.2.z.2	Aimles_ContextTransfer_Request operation

API operation name: Aimles_ ContextTransfer_Request

Description: The consumer requests AIMLE context transfer to an AIMLE server.

Inputs: See clause 8.x.3.1.

Outputs: See clause 8.x.3.2.

See clause 8.x.2 for details of usage of this operation.

* * * End of Changes * * * *