3GPP TSG-SA WG6 Meeting #63	S6-244517

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244112)

Source:	China Mobile

Title:	Pseudo-CR on Updated solution evaluation and conclusion of KI#1

Spec:	3GPP TS 23.700-92 v1.2.0

Agenda item:	8.2

Document for:	Approval

Contact:	Yue Liu, liuyueyjy@chinamobile.com

1. Introduction

Update the solution evaluation and conclusion of KI#1 based on LS replied from SA2 (S6-244009)

2. Reason for Change

Update the solution evaluation and conclusion of KI#1 based on LS replied from SA2 (S6-244009)

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-92 v1.2.0.

* * * First Change * * * *

10.3.1	Key issue #1: Application calling service based on eMMTel service

The open issues of this KI are as follows:

1.	Identify whether and how to provide API based Call control capabilities utilizing architecture and procedure provide by IMS subsystem. The control capabilities include the creation, updating, and terminating of a call which includes audio, video and Data Channel.

To address the open issues the following solutions has been proposed:

-	Solution #2: Application calling service between application and DCMTSI client

-	Applicable to calls from the application layer application (such as non-IMS applications) to UE. In the procedure, the eMMTel Enabler Server acts as http proxy and transfers the http traffic to the IMS Core, the  IMS Core transforms http streams to data channel streams.

-	Solution #6: third-party Call Service

-	Applicable to the scenario where a third-party application initiates a call. In the procedure, the eMMTel Enabler Server functions as the unified service entry and re-exposure SA2’s interface provides third-party call service to third-party call applications.

The two solutions are complementary in different scenarios of application calling service, and both rely on IMS capability exposure in the context of IMS data channel sessions. Solution #2 and Solution #6 can be considered in the normative work.

In addition, the IMS DC capability exposure is planned to be supported in NG_RTC_Ph2 WID in Rel-19, i.e.

-	establishing, updating, terminating of bootstrap data channels;

-	establishing, updating, terminating of (P2A, P2A2P, P2P) application data channels.

But  new APIs for IMS session management will not be defined in NG_RTC_Ph2 WID. In NG_RTC_Ph2 WID, it is assumed that the existing APIs, e.g., OMA APIs, provides such functionality.

* * * Next Change * * * *

11.1	Conclusion of KI#1

For KI#1 on "Application calling service based on eMMTel service" the following conclusions are agreed:

1.	Scenarios including call service between application and UE and call service between UE and UE in this Release.

2.	procedures including establishing a call with data channel media, terminating a call with data channel media and modifying a call to add data channel media in this Release.

Based on the gap analysis of OMA OneAPI framework in clause 4.2, IMS data channel is not supported by OMA API, therefore, a set of new APIs for MMTel enabler will be considered to be specified for technical specification based on Solution#2 and Solution#6, and such new APIs needs to be aligned with existing OMA APIs and also provide data channel capability (based on 3GPP TS 23.228 [3]) in Rel-19.

Editor’s note:	How to use the existing IMS session management APIs provided by OMA in MMTel service, without impacting SA2 architecture and considering potential security requirement is FFS and needs to be solved in the normative phase.

* * * Next Change * * * *

<Proposed change in revision marks>