3GPP TSG-SA WG6 Meeting #63	S6-244525

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244204)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Update Solution #13

Spec:	3GPP TR 23.700-92 v1.2.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to update Solution#13 based on the LS reply from SA2.

2. Reason for Change

The LS which asked SA2 whether they have a plan to define an interface for application providers to manage the applications to DCAR has replied. Solution#13 can be updated based on the LS reply from SA2.

The DC application retrieval procedure mentioned in key issue is not included in the solution.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-92 v1.2.0.

* * * First Change * * * *

#### 8.13.2.1	Data channel application upload

Figure 8.13.2.1-1 below illustrates how to upload a data channel application to DCAR via eMMTel Enabler Server.

Pre-conditions:

1)	The DC application has been uploaded to Controlling Application Server.

Figure 8.13.2.1-1: Upload a data channel application to DCAR via eMMTel Enabler Server

1.	The Controlling Application Server sends the DC application package configuration request to the eMMTel Enabler Server, the request message includes information elements as specified in Table 8.13.2.1-1.

Table 8.13.2.1-1: Information elements in the DC application package configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application version | M | Version of the Data Channel Application |
| DC Application Package | M | Code package of the Data Channel Application |

NOTE:	The DC application profile configuration procedure from Controlling Application Server to eMMTel Enabler Server has been captured in solution#4.

2.

The eMMTel Enabler Server invokes the API of the DCAR to upload the DC application package.

3.	The eMMTel Enabler Server sends the DC application package configuration response to the Controlling Application Server, the response message includes information elements as specified in Table 8.13.2.1-2.

Table 8.13.2.1-2: Information elements in the upload DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of upload DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the upload DC application package result is Failure. |  |  |

#### 8.13.2.2	Data channel application upgrade

Figure 8.13.2.2-1 below illustrates how to upgrade the version of a data channel application via eMMTel Enabler Server.

Pre-conditions:

1)	The DC application has been uploaded to DCAR via eMMTel Enabler Server.

Figure 8.13.2.2-1: Upgrade the data channel application version via eMMTel Enabler Server

- The Controlling Application Server sends the DC application package upgrade request to the eMMTel Enabler Server, the request message includes information elements as specified in Table 8.13.2.2-1.

Table 8.13.2.2-1: Information elements in the upgrade DC application package request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application version | M | Version of the Data Channel Application |
| Application related parameters | O | Other application information that may need to be updated, e.g. the elements in DC application profile. |
| DC Application Package | M | Code package of the Data Channel Application |

- The eMMTel Enabler Server invokes the API of the DCAR to upgrade the DC application package.

- The eMMTel Enabler Server sends the DC application package upgrade response to the Controlling Application Server, the response message includes information elements as specified in Table 8.13.2.2-4.

Table 8.13.2.2-2: Information elements in the upgrade DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of upgrade DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the upgrade DC application package result is Failure. |  |  |

#### 8.13.2.3	Data channel application delete

Figure 8.13.2.3-1 below illustrates how to delete a data channel application via eMMTel Enabler Server.

Pre-conditions:

1)	The DC application has been uploaded to DCAR via eMMTel Enabler Server.

Figure 8.13.2.3-1: Delete a data channel application via eMMTel Enabler Server

- The Controlling Application Server sends the DC application package delete request to the eMMTel Enabler Server, the request message includes information elements as specified in Table 8.13.2.3-1.

Table 8.13.2.3-1: Information elements in the delete DC application package request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |

- The eMMTel Enabler Server invokes the API of the DCAR to delete the DC application package.

- The eMMTel Enabler Server sends the DC application package delete response to the Controlling Application Server, the response message includes information elements as specified in Table 8.13.2.3-2.

Table 8.13.2.3-2: Information elements in the delete DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of delete DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the delete DC application package result is Failure. |  |  |

* * * Next Change * * * *

#### 8.13.2.x	Getting Data channel application profile information

Figure 8.13.2.x-1 below illustrates how to get the data channel application profile information via the eMMTel Enabler Server.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.13.2.x-1: Getting DC application profile information

- The Controlling Application Server invokes the API of the eMMTel Enabler Server to get the DC application profile information, the request message includes information elements as specified in Table 8.13.2.x-1.

Table 8.13.2.x-1: Information elements in the Getting DC application profile information request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |

- The eMMTel Enabler Server verifies that the sender is authorized to get the DC application profile information.

- The eMMTel Enabler Server sends a Getting DC application profile information response to the Controlling Application Server. The response includes information elements is the same as specified in Table 8.4.2-2.

* * * End of Changes * * * *