3GPP TSG-SA WG6 Meeting #64	S6-245360

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245330)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0630 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC Recording procedures using Session Recording Protocol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus, Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Description of recording procedures between MC service servers and recording server are missing from TS 23.280. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 10.18.3 for Recording procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for the recording and replay feature is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.18.3 (new), 10.18.3.1 (new), 10.18.3.2 (new), 10.18.3.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[3]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[4]	3GPP TS 22.281: "Mission Critical Video services".

[5]	3GPP TS 22.282: "Mission Critical Data services".

[6]	3GPP TS 23.002: "Network Architecture".

[7]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2"

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[10]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[11]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[12]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[13]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[14]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[15]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows".

[16]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[17]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[18]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[19]	3GPP TS 29.283: "Diameter Data Management Applications".

[20]	Void

[21]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[22]	IETF RFC 5245 (April 2010): "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[23]	GSMA PRD IR.92 v10.0: "IMS Profile for Voice and SMS".

[24]	GSMA PRD IR.88 v15.0: "LTE and EPC Roaming Guidelines".

[25]	3GPP TS 33.180: "Security of the mission critical service".

[26]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[27]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[28]	3GPP TS 22.011: "Service accessibility".

[29]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[30]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[31]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[32]	IETF RFC 2865 (June 2000): "Remote Authentication Dial In User Service (RADIUS)".

[33]	IETF RFC 3162 (August 2001): "RADIUS and IPv6".

[34]	3GPP TS 25.501: "System architecture for the 5G System (5GS)".

[35]	IETF RFC 1541 (October 1993): "Dynamic host configuration protocol".

[36]	IETF RFC 8415 (November 2018): "Dynamic host configuration protocol for IPv6 (DHCPv6)".

[RFC7866]	IETF RFC 7866 (May 2016): “Session Recording Protocol”

# * * * * Second change * * * *

### 10.18.3	Recording procedures

#### 10.18.3.1	General

The MC recording procedures for MCPTT and MCVideo group and private communications utilize Session Recording Protocol [RFC7866]. The Session Recording Client (SRC) is located in the MC service server. Recording server represents the Session Recording Server (SRS) in the Session Recording Protocol architecture.

Recording of communication sessions involves delivering real-time media and metadata of an MC service user or MC service group from an MC service server to the recording server.

#### 10.18.3.2	Procedure for recording communication sessions of an MC service user

Figure 10.18.3.2-1 below illustrates the recording procedure for communication sessions of an MC service user (MCPTT ID, MCVideo ID).

Pre-conditions:

- 	The user (MCPTT ID or MCVideo ID) has been configured as a target for recording in the MC service user profile (see TS 23.281 [12] clause A.3 and TS 23.379 [16] clause A.3) and the user profile has been uploaded to the MC service server.

-	The target MC service user has performed MC user authentication and service authorization (see TS 33.180 [25], clause 5.1.1 and 5.1.2) and has fetched its user profile(s) from the configuration management server.

Figure 10.18.3.2-1: Recording procedure for a target MC service user

- An MC communication session (MCPTT/MCVideo private call) involving the target user is initiated.

- A Session Recording Client (SRC) in the MC service server starts a recording session (see RFC 7866 [RFC7866]) with the recording server.

- The MC communication session ends.

- The recording session ends.

- The recording server stores the recording session, and it is available for download and/or replay.

NOTE:	Steps 1-5 are repeated as long as none of the conditions of step 6 is true.

- When the user (MCPTT ID / MCVideo ID) is removed from the recording targets or the user logs off from the MC service or the UE is switched off, an ongoing recording session (if exists) is terminated.

#### 10.18.3.3	Procedure for recording communication sessions of an MC service group

Figure 10.18.3.3-1 below illustrates recording procedure for communication session of an MCPTT group or MCVideo group service.

Pre-conditions:

- 	The target group (MCPTT group ID or MCVideo group ID) has been configured as a target for recording in the common group configuration data (A.4) and the group configuration data has been uploaded to the MC service server.

Figure 10.18.3.3-1: Procedure for recording communication sessions of an MC service group

- An MC group communication session (MCPTT/MCVideo group call) involving the target group is initiated.

- A Session Recording Client (SRC) in the MC service server starts a recording session [RFC7866] with the recording server.

- Group communication session ends.

- The recording session ends.

- The recording server stores the recording session, and it is available for download and/or replay.

NOTE:	Steps 1-5 are repeated as long as the none of the conditions of step 6 is true.

- When the group is removed from the recording targets (see 10.18.2.1) or the group is deleted (see 10.2.8), an ongoing recording session (if exists) is terminated.

# * * * * End of changes * * * *