3GPP TSG-SA WG6 Meeting #64	S6-245435

# Orlando, US, 18th – 22nd November 2024 								  (Revision of S6-245111)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0713 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Event Reporting Information clarification for out of service area event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 11 Nov 2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During the SA6#61, a new ACR management event was introduced in TS 23.558 under S6-242639. The purpose of the “out of service area” event is to detect whether a UE has moved into or out of the service area. In TS 29.558 clause 8.6.5.2.3, the ACR Management Event Subscription includes the following attributes: event (OUT_OF_SERVICE_AREA in this case), evtReq (which are the event reporting requirements), and the tgUeId (which allows the selection of a unique UE or a group of UEs). The data type of evtReq attribute is ReportingInformation, and ReportingInformation is defined in TS 29.523 clause 5.6.2.4 where some of the attributes that can include are immRep (immediate reporting flag), notifMethod (notification method) and repPeriodic (time interval between each notification). The notifMethot enumeration is defined in TS 29.508 clause 5.6.3.4 and can be PERIODIC, ONE_TIME and ON_EVENT_DETECTION, where ON_EVENT_DETECTION is set by default. For out of service area event, the immediate report shall not apply because it will return the last known location saved in cache, and it can be incorrect. As for the periodic notification, it will return the same value until there is a change in the Area of Interest of a UE or group of UEs, so it is not optimal and can produce some congestion and overload in the network. Even if the event detection notification is set by default, it shall be defined clearly in the specifications. As a consecuence, we shall guarantee that the notifMethod value is ON_EVENT_DETECTION and that immRep and repPeriodic are not included in the Management Event Subscription request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indicate that the UE Identifier or UE Group ID shall be included for the out of service area event Indicate that the event reporting mode shall be included for the out of service area event Clarify the use of based in the ACR management event subscribe request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handling of the reporting for the “out of service area” event is unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.3.2.2, 8.6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Move clarification to Event Report in table 8.6.3.3.2-1, and reword NOTE 5 in table 8.6.3.3.2-1. |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

##### 8.6.3.2.2	Subscribe

Figure 8.6.3.2.2-1 illustrates the subscribe operation between the EAS and the EES for ACR management event notifications.

Figure 8.6.3.2.2-1: ACR management event API: Subscribe operation

1.	The EAS sends ACR management event subscribe request (e.g. tracking the UE's user plane path change continuously). The EAS shall include UE Identifier or UE Group ID for "user plane path change", "out of service area", "ACR monitoring" and "ACR facilitation" events.

a.	The EAS may include the "user plane path change" event to indicate the EES to notify the EAS when the EES detects there is a user plane path change for the application traffic and the EAS may include Subscription Type (Early and/or Late notification defined in clause 5.6.7 of 3GPP TS 23.501 [2]) and/or Indication of EAS Acknowledgement in the event subscription.

b.	The EAS may include the "out of service area" event to indicate the EES to notify the EAS when the EES detects that a UE has moved into or out of the EAS service area.

c.	The EAS may include the "ACR monitoring" event to indicate the EES to notify the EAS when the EES detects there is a need for the ACR (e.g. when T-EAS is available at the target DNAI). The EAS may also include the Event Filters as defined in clause 5.3.4.4 of 3GPP TS 23.501 [2] to specify the conditions to match for notifying the event, e.g., inter-EDN mobility, intra-EDN mobility.

d.	The EAS may include the "ACR facilitation" event to request the EES to make the decision for ACR, discover the T-EAS(s), influence the traffic for the selected T-EAS and notify the S-EAS of the selected T-EAS. If required, the EAS can add an indication to request service continuity planning.

e.	The EAS may include the "ACT start/stop" event to indicate the EES to notify the EAS of the need for start or stop ACT to or from another EAS for a particular UE. The EES may also use "ACT start" event to notify the EAS of the ACR parameters.

f.	The EAS may include the “ACR Selection” event to indicate the EES to notify the EAS of the selected ACR scenario list applicable to ACs using the EAS.

2.	The EES checks if the EAS is authorized for this operation.

If authorized, and if the subscription in step 1 includes at least one of the "user plane path change", "ACR monitoring" and "ACR facilitation" events, the EES may invoke the PFD management procedure with the 3GPP Core Network as described in 3GPP TS 23.682 [10] and 3GPP TS 23.502 [8] with an application id. The traffic filter information sent by the EAS is used in requesting PFD management service. Further the EES provides the same application id for requesting user plane path management event service.

NOTE 1:	PFD management can be optionally supported in MNO. If EES cannot invoke step 2, it responds EAS with appropriate error.

NOTE 2:	The EES can map the EASID into the application id that is used to invoke the PFD management procedure.

3.	If the subscription in step 1 includes at least one of the "user plane path change", "ACR monitoring" and "ACR facilitation" events, the EES checks if there exists a subscription with the 3GPP core network for the user plane path management event notifications corresponding to the UE information obtained in step 1 as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], which may be triggered by other EAS for the same UE. The EES checks the availability of the user plane path management event service for the UE(s).

a.	if a subscription with 3GPP core network does not exist, then the EES subscribes with the 3GPP core network (PCF, NEF or SCEF+NEF) for the user plane path management event notifications of the UE(s) as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] If the EAS provides Subscription Type and/or Indication of EAS Acknowledgement, the EES include the type of subscription and/or the indication of "AF acknowledgement to be expected" as information on AF subscription to corresponding SMF events within the AF Request;

b.	if a subscription with 3GPP core network exists, then the EES uses the locally cached user plane path management event notification information of the UE(s) to respond to the EAS.

The EES stores the subscription related to the EAS.

If the subscription in step 1 is for "out of service area", the EES checks for an existing subscription with the 3GPP core network for area of interest UE mobility event monitoring notifications as described in TS 23.502 [3] clause 4.15.3 corresponding to the UE information obtained in step 1.

4.	If the event is "user plane path change" or "out of service area", the EES may subscribe to UE expected behaviour analytics (UE mobility and UE communication) for the group of UEs as described in 3GPP TS 23.288 [18].

5.	If EAS is authorized, the EES responds with ACR management event subscribe response. If EAS is not authorized, the EES provides a rejection response with cause information.

If the target UE(s) and the 3GPP network support mobility between 5GC and EPC, the EES monitors the availability of the user plane path management event notification from the 3GPP network by utilizing Nnef_APISupportCapability or Availability of service APIs event notifications provided by the CAPIF core function.

# * * * * 2nd Change * * * *

##### 8.6.3.3.2	ACR management event subscribe request

Table 8.6.3.3.2-1 describes the information elements for an ACR management event subscribe request from the EAS to the EES.

Table 8.6.3.3.2-1: ACR management event subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Security credentials | M | Security credentials of the EAS |
| UE ID (NOTE 1) | O | The identifier of the UE (i.e., GPSI) |
| UE Group ID (NOTE 1) | O | Identifies a group of UEs as defined in clause 7.2.7 |
| Application Group ID (NOTE 3) | O | Application group identifier as defined in clause 7.2.11. |
| Event ID(s) | M | Event ID: - user plane path change - out of service area - ACR monitoring - ACR facilitation - ACT start/stop - ACR selection |
| Indication of service continuity planning (NOTE 3) | O | Indicates that whether the service continuity planning is required i.e. whether EES shall monitor UE entering the predicted location. |
| Traffic filter information | O | The traffic filter information includes IP flow description, domain description (domain name, applicable protocol and matching criteria) or URI. Applicable for the "user plane path change" event, the "ACR monitoring" event and "ACR facilitation" event. |
| Event Report (NOTE 2) (NOTE 4) (NOTE 5) | O | Event Reporting Information as specified in clause 5.2.8.3.1 of 3GPP TS 23.502 [3] for user plane path change events and clause 4.15.3.1 of 3GPP TS 23.502 [3] for UE mobility events. The EAS shall include the event reporting mode as defined in clause 4.15.1 of 3GPP TS 23.502 [3] to be notified when the UE has moved into/out service area. |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Type of subscription | O | Indicates Early and/or Late notification to inform if the notification needs to be received before and/or after UP path configuration. Applicable for the "user plane path change" event. |
| Indication of EAS acknowledgement | O | This IE indicates the EES to include indication of "AF acknowledgement to be expected" within the AF request for subscribing UP path management events to 3GPP network and that the EAS will provide an acknowledgement as a response for the notifications of UP path management events to the EES. Applicable for the "user plane path change" event. |
| Indication of EAS acknowledgement for service continuity planning | O | This IE indicates that the EAS will provide an acknowledgement as a response to the notification of ACR management notification related to service continuity planning. |
| Event Filter (NOTE 4) | O | Event filter as specified in clause 5.3.4.4 of 3GPP TS 23.501 [2] |
| EAS characteristics for ACR | O | Set of characteristics to determine required EAS as detailed in Table 8.5.3.2-2. Applicable for the "ACR monitoring" event and "ACR facilitation" event. |
| NOTE 1: Either UE ID or UE Group ID shall be provided if the Event ID(s) includes any of the "user plane path change", "out of service area", "ACR monitoring" and "ACR facilitation" events. UE ID or UE Group ID are not applicable to "ACT start/stop" and "ACR selection" event. NOTE 2: This IE shall be present if the Event ID(s) includes any of the "user plane path change", "out of service area", "ACR monitoring" and "ACR facilitation" events. This IE is not applicable to "ACT start/stop" and "ACR selection" event. NOTE 3: This IE is applicable for "ACR monitoring", "ACR facilitation". NOTE 4: A subscription to the Event Report of "Area Of Interest" can be used by the EES to detect the need for ACR. The EAS service area can be used to indicate the area of interest. NOTE 5: For "out of service area" event, the event triggered reporting is prefered. When using the periodical reporting, care needs to be taken to avoid congestion. |  |  |

# * * * * End of Changes * * * *