3GPP TSG-SA WG6 Meeting #64	S6-245636

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245260)

Source:	Ericsson

Title:	Spatial Anchor Usage Analytics API clause

Spec:	3GPP TS 23.437 v0.3.0

Agenda item:	9.11

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

Adding the API clause for the Spatial anchor usage analytics information procedure defined in 3GPP TS 23.437 clause 8.4

2. Reason for Change

The API clause was missing.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.3.0

* * * First Change * * * *

## 8.4	Spatial anchor usage information

### 8.4.1	General

The spatial anchor usage information procedure enables the VAL server to request or subscribe with the SAn server to receive spatial anchor usage information.

* * * Next Change * * * *

## 8.5	SEAL APIs for spatial anchor management

### 8.5.1	General

Table 8.5.1-1 illustrates the SEAL APIs for spatial anchor management.

Table 8.5.1-1: List of SEAL APIs for spatial anchor management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SAnManagement | Create | VAL server, SAn client | Request/Response |
|  | Update | VAL server, SAn client |  |
|  | Delete | VAL server, SAn client |  |
|  | Discover | VAL server, SAn client |  |
| SS_SAnUsageInfo | Request | VAL server | Request/Response |
|  | Subscribe | VAL server | Subscribe/Notify |
|  | Notify | VAL server |  |

### 8.5.2	SS_SAnUsageInfo API

#### 8.5.2.1	General

Service description: This API enables a VAL server to get spatial anchor usage information from the SEAL SAn server.

#### 8.5.2.2	Request operation

API operation name: SS_SAnUsageInfo_Request

Description: The consumer requests for spatial anchors usage information.

Inputs: See clause 8.4.4.4.

Outputs: See clause 8.4.4.5.

See clause 8.4.3 for details of usage of this operation.

#### 8.5.2.3	Subscribe operation

API operation name: SS_SAnUsageInfo_Subscribe

Description: The consumer subscribes for spatial anchors usage information.

Inputs: See clause 8.4.4.1.

Outputs: See clause 8.4.4.2.

See clause 8.4.2 for details of usage of this operation.

#### 8.5.2.4	Notify operation

API operation name: SS_SAnUsageInfo_Notify

Description: The consumer is notified with spatial anchors usage information.

Inputs: See clause 8.4.4.3.

Outputs: None.

See clause 8.4.2 for details of usage of this operation.

* * * End of Changes * * * *