3GPP TSG-SA WG6 Meeting #64	S6-245605

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245150)

Source:	Huawei, Hisilicon

Title:	Gap for the SEAL services within MNO trusted domain

Spec:	3GPP TR 23.700-35

Agenda item:	8.9

Document for:	Approval

Contact:	Yang Yanmei (yangyanmei@huawei.com)

1. Introduction

In existing TS 23.434, the deployment case where SEAL services deployed by MNO is covered. But the architectures and solutions for some SEAL service are not correctly support such case.

This P-CR is to address the gap about the un-agligned description for the MNO deployed SEAL service.

2. Reason for Change

According to 3GPP TS23.501 and TS23.502, a MNO‘s AF(trusted AF) can invoke all the APIs exposed by NF(s) from SBA interface. SEAL sever(s) deployed by MNO exactly taking the role of a trusted AF should be able to invoke service APIs exposed by 3GPP core network functions directly over the Core network SBA.

However, in existing SEAL specification, the SEAL service server invoking 5GC APIs via Core network SBA is missing in some SEAL service. For example, in clause 9.2.2 of TS23.434, the Location management server is represented as the following:

“Figure 9.2.2-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [10].

Figure 9.2.2-3: Architecture for location management utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 9.2.2-4 illustrates the service-based representation for utilization of the Core Network (5GC, EPC) northbound APIs via CAPIF.

Figure 9.2.2-4: Utilization of Core Network Northbound APIs via CAPIF – service based representation

The Location management server acts as authorized API invoker to consume services from the Core Network (5GC, EPC) northbound API entities like SCEF, NEF, SCEF+NEF which act as API Exposing Function as specified in 3GPP TS 23.222 [6].”

So the overall revisit to SEAL specification for the gap and corrections should be addressed in the TR.

4. Proposal

It is proposed to agree the following changes to TR 23.700-35 V0.1.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications; Stage2".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Stage 2".

[4]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service; Stage 2".

[5]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service; Stage 2".

[6]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial mapping and Spatial anchors management Stage 2".

[7]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital asset".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2"

[10]	3GPP TS 23.501: "System architecture for the 5G System (5GS)" 2"

[11]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

* * * Next Change * * * *

4.x	Existing SEAL service: SEAL service server accessing network service via NEF/SCEF or directly without NEF/SCEF

4.x.1	Analysis

According to 3GPP TS 23.501 [10] and TS 23.502 [11], a MNO‘s AF(trusted AF) can invoke all the APIs exposed by NF(s) from SBA interface. Thus, SEAL server(s) deployed by an MNO within the MNO’s domain acts as the trusted AF/LCS client and should be able to invoke network service APIs exposed by 3GPP core network functions directly over the core network SBA interface.

However, in existing SEAL specification, the SEAL service server invoking 5GC APIs directly via core network SBA (without go through the NEF) is missing for some of the SEAL services. Take the location management service as an example. 3GPP TS 23.273 [23.273] clause 4.3.3 also indicates that “AFs and NFs may access GMLC directly or via NEF.”. However, in clause 9.2.2 of 3GPP TS 23.434 [3], only the location management server invoking 5GC location API via NEF/SCEF is shown, which may lead to the mis-understanding that the SEAL location management server cannot be deployed in the MNO’s domain.

4.x.2	Gaps

Gap#4.x.2-1: Investigate and enhance the location management service and other SEAL services architecture and deployment to identify if the aspect of deploying SEAL server within the MNO’s domain is not covered.

Gap#4.x.2-2:	Investigate and enhance the location management service and other SEAL services procedures and information flows to identify if the SEAL server directly interacting with the 3GPP NF (rather than via NEF) is not covered.

* * * End of Changes* * * *