3GPP TSG-SA WG6 Meeting #64	S6-245694

Orlando, US, 18th – 22nd November 2024	(Revision of S6-245294, S6-245570)

Source:	Ericsson

Title:	Support of AIML Client participation configuration provisioning in the registration

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

1. Introduction

The AIMLE client registration is a core functionality of the AIMLE service ; however, it contains a few inconsistencies in definitions.

2. Reason for Change

The following inconsistencies are corrected and improved in the AIMLE client registration:

-	AIMLE Client profile shall be associated with VAL service ID(s) in order to support multiple services;

-	Service ID need clarification, i.e., this is VAL service ID not a new entity;

-	The location-based conditions are missing according to the recommended solution 10, see clause 12.7 of TR 23.700-82.

-	Automatic AIMLE Client selection indicator is introduced to assist the AIMLE Client selection in automatic mode, i.e., reduction of signalling in AIMLE-Uu interface to safe energy, reduce traffic costs, etc. Also, this is user-friendly feature that reduces complexity of the solutiuons.

3. Conclusions

This pCR proposes to correct and include the member participation configurations in the registration of AIML Client.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.482 V0.3.0.

* * * First Change * * * *

#### 8.7.3.1	AIMLE client registration request

Table 8.7.3.1-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration procedure.

Table 8.7.3.1-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of supported services and profiles | M | Supported service information and AIML client profile. For each client profile provided in the list, the list of VAL services IDs with corresponding permissions is provided. |
| > List of VAL service ID(s) | O | List of VAL service ID(s) that the AIMLE client supports |
| > Corresponding service permission level(s) | O | Service permission level(s) (e.g., premium resource usage, standard resource usage, limited resource usage). |
| > AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations for the VAL services ID(s) as detailed in Table 8.7.3.1-2. |

Editor’s Note: The addition of AIMLE client profile ID is FFS.

Table 8.7.3.1-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split). |
| AIMLE client time schedule configurations | O | Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations in the given time slot(s) and/or day(s) of the week. |
| AIMLE client location configurations | O | Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIMLE client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

# *** End of Changes ***