3GPP TSG-SA6 Meeting #64	S6-245362

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024           (revision of S6-245110)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0623 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC service server to Recording server interface description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions, Airbus, UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Technical correctness of description for REC-6 interface between MC service server and recording server to address following problems in the specifications : - The current specs reads that “The REC-6 reference point shall use the HTTP-1 and HTTP-2 reference points for transport and routing of non-subscription/notification related signalling”, which excludes the possibility of other suitable protocols for the purpose. - The current spec reads that “The REC-6 reference point shall use SIP-2 reference point for transport and routing of subscription/notification related signalling” , however the recording server is not expected to initiate a transction/session towards the MC service server, this seems incorrect, hence needs to be updated. Further, the reference architecture updates to Rel-19 scope definition for logging/recording to include only MCPTT and MCVideo group/private communication sessions requires the REC-6 interface be renamed as REC-4 interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The REC-6 interface descriptions are updated as follows: - REC-6 interface is updated as REC-4 - The use of SIP-2/SIP-3 to allow SIP based interface for signalling transportation (This will allow use of existing protocol such as SIPREC) - Updates related to narrowed Rel-19 Scope to limit Recording to include only MCPTT and MCVideo communication sessions - The use of Session Recording Protocol RFC 7866 for Private and Group Communication sessions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definitions and description of the one of key recording interface continue to be incorrect and may not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.5.2.34	Reference point REC-4 (between recording server and MC service servers)

The REC-4 reference point, which exists between recording server and MC service servers, is used for transmitting metadata and media of the communication sessions of target users and target groups.

NOTE:	In this version of this specification the communication sessions that may be recorded are MCPTT and MCVideo group and private calls.

The REC-4 reference point shall use the SIP-2 reference point  for transport and routing of signalling and communication sessions related metadata. If an MC service server and a recording server are served by different SIP cores, then the REC-4 reference point shall also use the SIP-3 reference point for transport and routing of signalling and communication sessions related metadata.

MC service server and recording server shall use Session Recording Protocol as specified in RFC-7866 [RFC7866] for establishing and controlling recording sessions between MC service servers and recording server for MCPTT and MCVideo private and group communications. RFC-7866 [RFC7866] specifies the use of SIP, Session Description Protocol (SDP), and RTP to estabslish a recording session.

# * * * * End of change * * * *