3GPP TSG-SA WG6 Meeting #64	S6-245532

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245255)

Source:	Ericsson

Title:	Resource Owner Authentication

Spec:	3GPP TR 23.700-22 v1.1.0

Agenda item:	8.7

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This contribution provides a proposal for solution for KI#1 on resource owner authentication.

2. Reason for Change

As described for KI#1, one of the open issues in CAPIF when managing the resource owner consent is the how to authenticate the resource owner via CAPFI-8.

3. Proposal

It is proposed to agree the new key issue solution for 3GPP TR 23.700-22 V1.1.0.

* * * First Change * * * *

## 6.X	Solution #Z: Resource Owner Authentication

### 6.X.1	Solution description

#### 6.x.1.1	General

This solution is related to Key issue #1, open issue #5, Resource Owner authentication via CAPIF-8.

Currently, 3GPP TS 23.222 [2] clause 4.17.1, the scope of an API invoker on a UE in RNAA is limited to accesing its own resources only, i.e., the resource owner is a user of the UE hosting the API invoker that can authorize the API access. Similarly, in 3GPP TS 33.122 [3], clause 6.5.3, it is clarified that "In the present document, only a UE accessing its own resources is considered if the API invoker is on a UE."

In addition, there are cases as per key issue#6 in which it is needed to support for API invoker(s) which are deployed on a UE accessing resources of other UEs.

For all these use cases, it is needed to specify the framework in CCF that supports RO authentication and the provisioning of RO authorization information.

#### 6.x.1.1	General

This solution is related to Key issue #1, defining the framework in CCF that supports RO authentication and the provisioning of RO authorization information.

#### 6.x.1.2	Procedure for Resource Owner Authentication

Precondition:

1.		The resource owner ID is the GPSI of the corresponding UE if the resource is related to a UE.

2.	The CCF is the Authorization server that holds the RO account and provides access grants for accessing RO resources,

2.	The ROF interacts with the CCF (Authentication and Authorization server) using CAPIF-8 to authenticate the Resource Owner and to provide Resource Owner authorization (e.g. to grant access to API invoker(s) deployed on other UEs.

Figure 6.x.1.2: Procedure for Resource Owner Authentication in CAPIF

- When the RO is accessing to a service, and RNAA procedures apply, the ROF initiates the flow by interacting with the CCF AuthF. The ROF includes the Resource Owner identifier (i.e., the GPSI). The CCF AuthF authenticates the Resource Owner.

- The ROF may provide to the CCF Resource Owner information about whether the Resource Owner allows access request for a requested scope (e.g.AEF service API) and a purpose. The ROF provided authorization information may also include additional information, as e.g., the authorized applications (e.g., App Id) or whether other UEs are allowed to access the indicated resources. The CCF authorization, based on the ROF provided authorization information, manages the requests for RO authorization to access RO resources.

NOTE:	The detailed authentication and authorization aspects, i.e., how the resource owner authorization is provisioned by the RO into CCF,  are FFS and need to be specified in 3GPP TS 33.122 [3] by SA3.

### 6.x.2	Architecture Impacts

This solution is based on the existing architecture of CAPIF as described in 3GPP TS 23.222 [2] and relates to the CAPIF-8 reference point.

### 6.x.3	Corresponding APIs

Whether new or enhanced APIs are required to support this solution is in the scope of SA3.

### 6.x.4	Solution evaluation

This solution addresses KI#1 on resource owner authentication via CAPIF-8. Coordination with SA3 is needed to align the framework with the authentication and authorization procedures in 3GPP TS 33.122[3].

* * * Next Change * * * *

## 9.1	Evaluation of key issue#1

Clause 5.1 captures the key issue on managing resource owner consent. The following open issues are listed:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	Whether (and how) “purpose of data processing” will be captured and where will it be stored?

3.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

4.	How to enable CAPIF resource owner’s consent in the context of group.

Solution#2 specified in clause 6.3 proposes a solution to address open issue#1 in nested API scenario. This solution requires coordination and feedback on the security aspects for the appropriate user consent at AEF.

Solution#3 specified in clause 6.4 proposes a solution to address open issue#3 on finer granular access control for service API by enhancing the existing access control by introducing the resource level access.

Solution#9 specified in clause 6.10 proposes a solution to address open issue#1 and open issue#2 by enabling more granular information about API invoker being used in the context of RNAA.

Solution#10 specified in clause 6.11 proposes a solution to address open issue#3 by emphasing the need for authorization based on granular service access (e.g., per service API or per service operation or per service resource). The impact on stage 2 aspects of this procedure are covered by Solution#3 and the specific impact for authorization requires further study.

Solution#11 specified in clause 6.12 proposes a solution to address open issue#1 on communication between resource owner and Authorization function (CCF) for user consent revocation.

Solution#16 specified in clause 6.17 proposes a solution to address open issue#1 on communication between resource owner function and Authorization function (CCF) for obtaining user consent.

Solution#12 specified in clause 6.13 addresses open issue#1 to manage consent of resource owner through communication between ROF and CCF (Authorization Function) for processing user consent requests related to one or more API invokers.

Solution #13 specified in clause 6.14 addresses open issue#1 on obtaining resource owner consent via the CAPIF-8 reference point, whereby a request for such consent can be on behalf of one or more API invokers and relate to one or more API exposing functions to reduce the required number of interactions between the CCF (Authorization Function) and resource owner via the ROF.

Solution#13 specified in clause 6.14 proposes a solution to address open issue#1 on communication between resource owner function and Authorization function (CCF) for bulk user consent requests processing for pre-configured API invokers.

Solution#14 specified in clause 6.15 proposes a solution to address open issue#2 on inclusion of purpose for data processing in the mechanim for API invoker requesting/obtaining authorization information.

Solution#15 specified in clause 6.16 proposes a solution to address open issue#1 by describing the role of Authorization function in storage of authorization related information from the resource owner.

Solution#18 specified in clause 6.19 proposes a solution to address open issue#1 by describing the usage of group context information for enabling CAPIF resource owner’s consent in the group.

Solution #20 specified in clause 6.21 proposes a solution to address open issue#1 by introducing a new response message to the requesting entity (API invoker or AEF) from the AEF, upon the service API invocation without the resource consent information. The response message triggers the requesting entity to obtain resource owner consent for service API invocation. This solution is complementary to solution#2, which proposes the requested entity is configured with a policy to enforce user consent checking prior to service API invocation.

Solution#Z specified in clause 6.X proposes a solution to address open issue#5, by describing a framework for RO authentication and the provisioning of RO authorization information,. This solution describes how the CCF can obtain RO provided authorization information to be able to provide granted access information for an API invoker deployed in the UE, and for API invoker(s) deployed on other UEs.

Editor's note:	Further evaluation of KI#1 considering the related solutions is FFS.

* * * Next Change * * * *

## 10.2	Conclusions of key issue #1

Solution#3 and Solution#9 can be considered as candidate for normative work for addressing the aspects of granular access and granular API invoker information.

Solution#11, Solution#16, Solution#12 and Solution#13 can be considered as candidate for normative work for addressing the communication between resource owner and Authorization function (CCF) over CAPIF-8. The authentication and authorization procedures for CAPIF-8 are required.

Solution#14 can be considered as candidate for normative work for addressing the inclusion of purpose for data processing in the mechanism for API invoker requesting/obtaining authorization information.

Solution#15 can be considered as candidate for normative work for enhancing the description of the Authorization function role.

Solution#2 requires coordination and feedback on security aspects for the appropriate user consent at AEF to be considered for stage 2 impacts in normative work.

Solution#18 can be considered as candidate for normative work for enabling resource owner’s consent in the context of group.

Solution#20 can be considered as candidate for normative work for enhancing the procedure to obtain resource owner consent upon service API invocation at AEF.

Solution#Z can be considered as candidate for normative work for the definition of Resource Owner authentication via CAPIF-8.

NOTE 1:	Security aspects are in scope of SA3 and further coordination with SA3 is required.

NOTE 2:	Solution#10 is related to security aspect and is in SA3 scope.

NOTE 3:	A consistent terminology for the terms "user consent", "resource owner consent", "resource owner authorization" is to be defined during normative work.

* * * End of Changes * * * *