3GPP TSG-SA WG6 Meeting #64	S6-245552

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245262)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0120 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Multi-modal flows synchronization monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SEALDD (as XR application enabler) Multi-modal flow synchronzation monitoring was concluded in TR 23.700-23 (KI#1, KI#2, sol#6). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds support for Multi-modal flows synchronization monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting new feature in Rel19 XR work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.2.3, 9.7.3.6, 9.7.3.8, 9.12.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.7.2.3	Data transmission quality measurement reported by SEALDD client

Figure 9.7.2.3-1 illustrate the procedure for SEALDD enabled data transmission quality measurement for VAL traffic. The SEALDD client receives transmission quality measurement requirement, decides to start VAL data transmission monitoring and generates measurement reports.

Figure 9.7.2.3-1: VAL data transmission quality measurement reported by SEALDD client

1.	An on-going regular data transmission connection is established according to clause 9.2.2.2.

The transmission quality measurement can be triggered by VAL server or VAL client, which is described in step 2 to step 5 and step 6, correspondingly.

The request may include the crossflow measurement information (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2/DL]}) associated in the same multi-modal service for crossflow RTT measurement.

2.	The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition.

NOTE 1:	The spatial and/or temporal condition can be used by SEALDD client to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday.

3.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server responds to the VAL server.

4-5.	The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server. The SEALDD client, based on the received service quality guarantee policy including thresholds and action, can take corrective action as described in clause 9.7.2.3.

If crossflow RTT measurement requirement is received from the VAL server, the SEALDD server identifies the SEALDD connections for the involved VAL UEs and sends to the SEALDD client(s) with crossflow RTT measurement requirement including corresponding UL/DL flow information.

6.	The VAL client triggers the SEALDD transmission quality measurement procedure to the SEALDD client, in order to collect the measurement report information.

7.	After SEALDD client determines to start measurement process, upon UL packet arrival, the SEALDD client initiates the UL packet delay measurement. The SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload for VAL data transmission quality monitoring) with local time T1 when the SEALDD client sends out the UL monitoring packet. The SEALDD client considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD client stops/suspends the transmission quality measurement.

For crossflow RTT measurement, the SEALDD client received UL flow information starts recording local time T1 when sending out the 1st UL packet matching the UL flow information to the SEALDD server. The T1 is sent in the encapsulated UL monitoring packet to the SEALDD server.

8.	The SEALDD server receives the UL monitoring packet, and records the local time T2.

9.	Similarly, the SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring in case there is no DL VAL traffic for DL packet delay monitoring) with local time T2 recorded in step 8 and local time T3 when the SEALDD server sends out the DL monitoring packet.

NOTE 2:	When the SEALDD server sends the dummy UL packet as monitoring response to the SEALDD client depends on SEALDD server implementation.

For crossflow RTT measurement, when the 1st DL packet matching the received DL flow information is to be sent, the SEALDD server encapsulates the DL monitoring packet with previously received T1 and sends the DL monitoring packet to the SEALDD client.

10.	The SEALDD client records the local time T4 when the SEALDD client receives the DL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD client can also calculate the bitrate and jitter over a certain period over a specific SEALDD connection by recording the status of the SEALDD monitoring packets. The SEALDD client also evaluates the reporting criteria and the flow alignment reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report.

For crossflow RTT measurement, if T1 is received in the DL packet, the SEALDD client records local time T2 and calculates RTT based on T1 and T2.

Depending on which entity triggers the data transmission quality measurement, step 11 and step 12 corresponds to step 2 to step 5, step 13 corresponds to step 6.

If the delay difference, i.e., the bound of the difference of the end-to-end time delay among all associated flows, is requested, the SEALDD client calculates the delay difference between VAL server and SELADD client, based on the arrival time and the timestamp (e.g., timestamp as recorded in the RTP header) among all associated flows identified by the Multi-modal SEALDD flow ID or SEALDD-UU flow ID(s). The arrival time of first detected flow and last detected flow, Ta and Tb, are recorded, and the delay difference is the difference between Ta and Tb. Then SEALDD client sends the calculated delay difference to the SEALDD server.

11-12.	The SEALDD client reports the data transmission quality measurement results (e.g. latency, jitter, bitrate) to the VAL server via the SEALDD server.

13.	The SEALDD client reports the data transmission quality measurement results to the VAL client.

NOTE 3:	The crossflow RTT measurement calculated by the SEALDD client in step 10 can be used as an estimation of the lower bound for the Motion-to-Photon latency in the XR scenarios.

When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 11 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server collects and aggregates the needed report for the VAL server.

Editor's note: Whether identifying packet in PDU set is necessary in crossflow RTT measurement is FFS.

* * * Next Change * * * *

#### 9.7.3.6	Transmission quality measurement subscription request

Table 9.7.3.6-1 describes the information flow from the SEALDD server to the SEALDD client for data transmission measurement subscription.

Table 9.7.3.6-1: Transmission quality measurement subscription request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD-UU flow ID | M | Identifier of the SEALDD-UU flow. |
| Multi-modal SEALDD flow ID | O | Identity of the multi-modal SEALDD flows associated with the SEALDD multi-modal connection for the measurement. |
| Measurement conditions | O | Indicates the temporal and/or spatial conditions. |
| Transmission quality measurement requirements list | M | The measurement requirement information |
| > Measurement ID | M | Measurement identifiers, e.g. latency, bitrate, jitter |
| > Reporting frequency | O | The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting. |
| > Reporting periodicity | O | If the reporting frequency is periodic, the reporting periodicity shall be provided. |
| > Measurement period window | O | Indicates the measurement period window for transmission quality measurements |
| > Measurement expiration time | O | Indicates the measurement expiration time |
| > Reporting criteria | O | Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria if more than one criteria is specified. |
| > Delay difference indicator | O | Indicates the delay difference needs to be measured. |
| > Flow alignment reporting criteria | O | Indicates the criteria for reporting flow alignment measurement results, e.g. if the flow alignment buffering reaches below or above a certain value for a certain amount of time. It also includes a unique identifier for each criteria if more than one criteria is specified. |
| > SEALDD policy | O | Specifies quality guarantee policies associated with the SEALDD connection |
| >> Quality guarantee policy | M | Indicates the event (e.g. measurement threshold) to be measured for, the quality guarantee. |

* * * Next Change * * * *

#### 9.7.3.8	Transmission quality measurement notification

Table 9.7.3.8-1 describes the information flow from the SEALDD client to the SEALDD server for notifying the transmission quality measurement reports.

Table 9.7.3.8-1: Transmission quality measurement notification

| Information element | Status | Description |
| --- | --- | --- |
| Transmission quality measurement reports list | M | The generated transmission quality results in SEALDD server |
| > Measurement ID | M | Measurement identifiers, e.g. latency, bitrate, jitter, delay difference, flow alignment measurement |
| > Average measurement value | O | The average measurement value of measurement results |
| > Minimum measurement value | O | The minimum measurement value of measurement results |
| > maximum measurement value | O | The maximum measurement value of measurement results |
| > Standard deviation measurement value | O | Standard deviation measurement value of measurement results |
| > kPercentile measurement value | O | Indicates the kpercentile measurement value of measurement results |
| > Measurement period | O | Indicates the measurement period |
| > Timestamp | O | Indicates the timestamp of measurement results |

* * * Next Change * * * *

##### 9.12.2.2.2	SEALDD enabled multi-modal flow synchronization

Figure 9.12.2.2.2-1 illustrate the procedure for SEALDD enabled multi-modal flow synchoronization, the SEALDD server determines/updates the required QoS information for multi-modal flow(s), and further interacts with 5G network.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

-	The SEALDD server has been provisioned to multi-modal XR policy including the synchronization threshold, as specified in clause 9.10.3.1.

Figure 9.12.2.2.2-1: SEALDD enabled multi-flow synchronization procedure

1.	An on-going multi-modal data transmission connection is established according to the steps 1-8 of clause 9.12.2.1.2.

2.	The SEALDD server performs data transmission quality measurement, as defined in clause 9.7.2.1 or clause 9.7.2.3, in SEALDD-UU interface based on the mapping information for multiple flow association information between SEALDD-S interface and SEALDD-UU interface. Upon receiving the packets from multiple associated flows in SEALDD-S interface, the SEALDD server performs the packet encapsulation with sending timestamp information in the corresponding SEALDD-UU interface, and can calculate the transmission delay measurement result of multiple associated flows after obtaining the receiving timestamp from the SEALDD client.

3.	Based on the data transmission quality measurement results obtained for multiple associated flow over SEALDD-UU interface in step 2, and the synchronization threshold for multi-modal application as described in pre-condition, the SEALDD server can determine the service flow(s) (i.e. address/port for SEALDD-UU flow) that needs to be adjusted among the multiple associated flows in SEALDD-UU interface, and the corresponding required QoS information (i.e. transmission delay).

4.	The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-UU flow) and the corresponding required QoS information determined in step 3, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [6]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side.

NOTE:	This procedure is applicable for both downlink and uplink synchronization of multi-modal flow. For downlink and/or uplink, the step 3 is determined according to the measured downlink and/or uplink data transmission quality measurements in step 2.

After requesting the transmission quality optimization on 5G network with the required QoS for the adjusted flow(s), the multi-flow synchronization of multi-modal application can be satisfied.

* * * End of Changes * * * *