3GPP TSG-SA6 Meeting #64	S6-245707

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0336 | rev | 4 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Application QoS coordination for Mobile Metaverse Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution provides a new feature related to the concluded solution #11 of TR 23.700-21. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The change includes mainly the introduction of new procedure in SEAL NRM capability related to “QoS/resource management for network-assisted UE-to-UE/VN group communications” for capturing the procedure for supporting mobile metaverse use cases (covering interactions among VAL UEs and VAL servers in both physical and virtual world). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New capability based on mobile metaverse use cases will not be added. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses: 14.3.5.1, 14.3.5.X (new), 14.3.5.X.1 (new), 14.3.5.X.2 (new), 14.3.2.x (new), 14.3.2.y (new), 14.3.2.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 14.3.5	QoS/resource management for network-assisted UE-to-UE/VN group communications

#### 14.3.5.1	General

This feature provides the SEAL NRM support for coordinated QoS/resource management for network assisted UE-to-UE communications. Such capability may be required for guaranteeing end-to-end QoS fulfilment (primarily for meeting end-to-end latency requirements) in network assisted UE to UE communications and may accommodate various vertical-specific application services, e.g.:

-	Network-assisted Command and Control (C2) communications in UASAPP [TS 23.255], where the UAV controller navigates its UAV over the 5GS;

-	Teleoperated Driving (ToD) in eV2XAPP [TS 23.286], where the a V2X UE acting as server may remotely control a further V2X UE over the 5GS;

-	Network-assisted Device-to-Device communications in Factory of the Future (FF) use cases, such as control-to-control communications.

-	5G LAN-Type communication within a 5G VN group as specified in 3GPP TS 23.501 [10].

-	Application QoS coordination for Mobile Metaverse Services in distributed VAL servers.

* * * Second Change * * * *

#### 14.3.5.X	Application QoS coordination for Mobile Metaverse Services in distributed VAL servers

#### 14.3.5.X.1	General

This clause provides a mechanism for application session QoS coordination for mobile metaverse (MM) services where these services are offered by more than one VAL server.

It is assumed that the MM service consists of the following VAL sessions which can be present for multi-user interactions. The sessions may include VAL UE1 and VAL UE2 and digital counterparts of UE1 and UE2 instantiated in one or more DNs (called as virtual or avatar VAL UEs).

-	VAL session #1: VAL client 1 sends to VAL server 1 (including virtual VAL UE 1 at DN side) sensor data / measurements on the physical environment related to VAL UE1 over VAL-UU interface. VAL server (virtual VAL UE1) sends back haptic feedback to VAL UE1 (for UE1 and/or UE2 and the environment).

-	VAL session #2: VAL client 2 sends to VAL server 2 (including virtual VAL UE 2 at DN side) sensor data / measurements on the physical environment related to VAL UE2 over VAL-UU interface. VAL server (virtual VAL UE2) sends back haptic feedback to UE2 (for UE1 and/or UE2 and the environment).

-	VAL session #3: Exchange of service related / feedback data between VAL server 1 and 2 for interactions between virtual UE 1 and 2 (for example micro-transactions such as avatar updates/modifications or environment changes).

-	VAL session #4: Sensor data / measurements are exchanged between VAL UEs over VAL-PC5 (communication can be over side link) for traffic related to the MM service.

There is a coupling in the performance requirements for all the above VAL sessions which is corresponding to the end-to-end MM service.The end-to-end MM service may be related e.g., to 5G-enabled Traffic Flow Simulation and Situational Awareness, where the physical objects including UEs (e.g. V2X UEs) in cars and trucks in each lanes, will have a corresponding digital twin in the virtual world in distributed VAL servers, and the virtual and physical UEs form the mobile metaverse.

One possible coupling of such requirements is about the expected sequence of certain messages to be transferred via the 5GS. For example, the delay of the collection of sensor information from the physical environment may have impact on the expected delay of haptic feedback from the server to the VAL UE which will result in degrading the QoE for the MM service.

In the procedure as defined in 14.3.5.X.2, the NRM server takes the performance of VAL Session # 1, VAL Session # 2, VAL Session # 3, VAL Session # 4 into consideration to ensure the E2E performance (UE#1 - VAL server#1 - VAL server#2 - UE#2). For example, if the delay of VAL session # 1 is long, the delay of other sessions needs to be reduced.

NOTE: This capability is only applicable to multiple VAL server deployments (e.g., virtual UEs in different edge or cloud deployed VAL servers). In case of single VAL server offering the MM service, the application QoS coordination is performed based on procedure in clause 14.3.5.3.1 for single pair and 14.3.5.3.2 for group of UEs..

#### 14.3.5.X.2	Procedure on application QoS coordination for Mobile Metaverse Services

Figure 14.3.5.X.1-1 illustrates the procedure where the NRM server is supporting application QoS coordination for the VAL sessions within a MM Service.

Pre-conditions:

1.	The NRM clients are connected to the NRM server.

2.	The VAL servers are connected to the NRM server.

3.	The VAL sessions which are coupled to support the Mobile Metaverse (MM) Service (e.g., VAL client 1 to VAL server 1, VAL client 2 to VAL server 2) are established.

Figure 14.3.5.X.1-1 application QoS coordination for VAL MM services

- The NRM client 1 (acting on behalf of VAL client 1) sends to the NRM server an MM-specific QoS management request to trigger the application QoS coordination between the physical and virtual VAL UEs within MM service. This request may also include the end-to-end QoS/service provisioning requirement for the MM service, and the expected dependencies among different sessions (e.g. sequence of data to be exchanged among VAL servers and clients).

- The NRM server decomposes the MM service performance requirement (end to end application QoS target) to per VAL session requirement taking into account the dependencies among sessions, where in this step the NRM server identifies the VAL sessions for which the application QoS parameters need to be configured or updated.

- The NRM server configures the application QoS parameters per VAL session, for the identified VAL sessions to ensure meeting the end-to-end QoS requirements for the MM service.

- The NRM server sends the MM-specific QoS management response to the NRM client 1 to provide the configuration information for the QoS attributes for the VAL sessions related to VAL UE 1.

- The NRM server sends a QoS configuration notification to the VAL servers 1 and 2 as well as to the NRM client 2 to provide the configuration information for the QoS attributes for the corresponding VAL sessions. The QoS configuration notification may include per session QoS requirements for each metaverse session received/sent at/by the UE, where each metaverse session may be composed of multi-modal type of communication conveying audio, video or haptic/sensor information from/between the UE.

* * * Third Change * * * *

#### 14.3.2.x	MM-specific QoS management request

Table 14.3.2.x-1 describes the information flow MM-specific QoS management request from the NRM client to the NRM server.

Table 14.3.2.x-1: MM-specific QoS management request

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL UEs or VAL Group ID | M | List of VAL UEs or the group of VAL UEs for whom the MM QoS management occurs. |
| >VAL UE/user ID | M | Identity of the VAL UE. |
| > Connection information | M | Connection information including address / port and / or URL of the VAL UE. |
| List of VAL server IDs | M | The list of VAL servers involved in the mobile metaverse service. |
| > VAL server ID | M | Identity of the VAL server. |
| > Connection information | M | Connection information including address / port and / or URL of the VAL server. |
| MM service or session(s) ID | O | The VAL service or session(s) identity for whom the QoS management occurs. |
| MM QoS requirements | O | The mobile metaverse QoS requirements / KPIs (latency, error rate, ..) for the MM end to end service. This may optionally include information which will support the NRM server to identify the per session QoS requirements. |
| VAL session information | O | Information and dependencies (e.g. sequence or priority) among VAL sessions within MM service. |
| Service area | O | The area where the QoS management request applies. This can be geographical area, or topological area. |
| Time validity | O | The time of validity of the requirement. |

#### 14.3.2.y	MM-specific QoS management response

Table 14.3.2.y-1 describes the information flow MM-specific QoS management response from the NRM server to the NRM client.

Table 14.3.2.y-1: MM-specific QoS management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the MM-specific QoS management request. |
| QoS report configuration | O | The configuration of the NRM client's report triggering by NRM server (e.g. setting thresholds for reporting a QoS downgrade / notifications based on channel loss great than threshold value) |

#### 14.3.2.z	MM service QoS configuration notification

Table 14.3.2.z-1 describes the information flow MM service QoS configuration notification from the NRM server to the NRM client or to the VAL server.

Table 14.3.2.z-1: MM service QoS configuration notification

| Information element | Status | Description |
| --- | --- | --- |
| MM service ID | M | The VAL service identity for whom the QoS management occurs. |
| VAL session ID | M | The VAL session for which the QoS is configured. |
| >MM QoS configuration | M | The configuration of the QoS attributes (latency, jitter) for the VAL sessions within the MM service |
| QoS report configuration | O | The configuration of the NRM client's or VAL server report triggering by NRM server (e.g. setting thresholds for reporting a QoS downgrade / notifications based on channel loss great than threshold value) |

* * Fourth Change * * * *