3GPP TSG-SA WG6 Meeting #64	S6-245714

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245297, S6-245594)

Source:	Ericsson

Title:	Support of AIML Services for Assisting Edge Computing

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added descriptions and procedure for support of AIML services for assisting edge computing on execution node selection to TS 23.482.

2. Reason for Change

Edge computing is a distributed computing model that brings computation and data storage closer to the sources of data. Such design can reduce the latency compared to when an application runs on a centralized data centre. Some examples of edge use cases include self-driving cars, autonomous robots, smart equipment data and automated retail, etc. Solution#29 is recommended for normative work on support AIML services for assisting edge computing. See more details in 3GPP TR 23.700-82. This paper proposes to add descriptions and procedure for supporting of AIML services for assisting edge computing on execution node selection to TS 23.482.

3. Conclusions

This paper proposes to add descriptions and procedure for support of AIML services for assisting edge computing on execution node selection to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[10]	3GPP TS 22.261: "Service requirements for the 5G system".

[11]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[X1]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

* * * Next Change * * * *

## 8.x	Support of AIML Services for Assisting Edge Computing

This clause describes procedure for supporting AIML services for assisting edge computing.

### 8.x.1	General

This clause describes the procedure for assisting edge computing by the AIMLE server. A CAS or EAS (which are defined in 3GPP TS 23.558 [X1]) can have different roles in different types of edge computing, e.g. hiearchitical computing (with one root node (e.g., CAS, EAS), the root has one or more children which are also known as sub-root node(s) (e.g., EAS), and multiple leaf nodes (e.g. EAS) with no children), distributed computing (with one coordination node (e.g., CAS) and multiple execution nodes (e.g., EAS)). Here, hierarchical computing represents a computation architecture with multiple computation entities involved and multiple levels of computations for a computation task. An AI/ML task can be seen as a special computing task, and edge comptuting can be used for completing the AI/ML tasks.

### 8.x.2	Procedure for assisting edge computing process

Figure 8.x.2-1 illustrates the procedures for AIMLE server to assist an edge computing process.

Pre-conditions:

-	An AI/ML task be treated as a specical computing task being completed at consumer (e.g., CAS, EAS).

-	The consumer decides its role in the edge computing architecture (e.g., hierarchical computing, distributed computing) for an AI/ML task (e.g. FL training), and the type of edge computing operations, based on its local configuration.

-	The AIMLE server can assist an edge computing process by providing time window(s) recommendation for computing task distribution if the consumer is a root node in a hierarchical computing process or a coordination node in a distribution computing process, providing time window(s) recommendation for intermediate output delivery if the consumer is a leaf node in a hierarchical computing process, candidate execution node list provisioning or edge computing preparation status provisioning if the consumer is a coordination node of a distributed computing process, etc.

-	The consumer decides that assistance from AIMLE server to support the edge computing process (AI/ML task) is needed, due to lack of capability on e.g. execution node selection.

Figure 8.x.2-1: Procedure for assist an edge computing process

Figure 8.x.2-1 illustrates the procedure for AIMLE servers assist an edge computing process. The corresponding procedure in detail is as follows:

- The VAL server (e.g. CAS, EAS) sends edge computing assistance request to the edge AIMLE server for an edge computing process (AI/ML task). The request message includes information as described in Table 8.x.3.1-1.

- The edge AIMLE server authenticates and authorizes the request from the consumer, and checks its capability to provide the requested assistance information.

If the request is authorized and the edge AIMLE server can generate the required assistance information (e.g. edge computing preparation status at an execution node which is registratered to it), it performs step 3 and 4.

If the request is authorized but the edge AIMLE server cannot generate the required assistance information (e.g. execution nodes are registratered to different edge AIMLE servers), it performs step 5 and skips steps 3 and 4.

- The edge AIMLE server performs operations to generate assistance information which is requested in step 1. For example, for edge computing preparation status at an execution node, the edge AIMLE server may subscribe/request analytics from ADAE server (e.g. edge load analytics, edge computing preparation analytics) and aggregate the information received to generate assistance information.

- The edge AIMLE server sends edge computing assistance response to the consumer (e.g. edge computing preparation status at the execution node). The response message contains the information as described in Table 8.x.3.2-1.

- If the edge AIMLE server cannot generate the required assistance information, it sends edge computing assistance request to central AIMLE server. The request message includes information as described in Table 8.x.3.1-1.

- The central AIMLE server authenticates and authorizes the request from the edge AIMLE server.

- If the request is authorized, the central AIMLE server performs operations to generate assistance information which is requested in step 1.

For example, for execution node selection, according to the role of the VAL server in request message in step 1, the central AIMLE server derives the requirements on the execution node (e.g. high computation capability, high communication capability, or both). Then, the central AIMLE server may subscribe/request analytics from ADAE server (e.g. edge load analytics, edge computing preparation analytics), trigger generation of split operation assistance information, retrieve FL member information from ML Repository (the FL member with EAS ID). The existing services can be reused for execution node selection, e.g., the step 3 in clause 8.12.2.

- The central AIMLE server sends edge computing assistance response with the generated assistance information (e.g. a list of selected candidate execution nodes). The response message contains the information as described in Table 8.x.3.2-1.

8a.	The central AIMLE server sends the edge computing assistance response to the edge AIMLE server.

8b.	The edge AIMLE server sends the edge computing assistance response to the consumer.

The VAL server (e.g. CAS, EAS) uses the assistance information for decision making on its edge computing operations.

* * * Next Change * * * *

### 8.x.3	Information flows

#### 8.x.3.1	Edge computing assistance request

Table 8.x.3.1-1 shows the request sent by VAL server (e.g. CAS, EAS) to the AIMLE server for assistance of an edge computing process (AI/ML task).

Table 8.x.3.1-1: Edge computing assistance request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Original requestor identifier | O | The identifier of the original requestor, e.g. VAL server ID, EAS ID, CAS ID. |
| Role | M | Represents the role of the VAL server in an edge computing architecture (e.g. root node, sub-root node or leaf node of a hierarchical computing process, coordination node or execution node of a distributed computing process). |
| Computing task type | M | The type of computing task (e.g. VFL, HFL). |
| Assistance information type | M | Represents the assistance information type, which is used to indicate the assistance information needed, e.g. candidate execution node list, edge computing preparation status at an execution node. |
| Execution node(s) | O | Represent one execution node or a list of candidate execution nodes. |

#### 8.x.3.2	Assist edge computing response

Table 8.x.3.2-1 shows the response sent by AIMLE server to the VAL server (e.g. CAS, EAS) for assisting the edge computing process.

Table 8.x.3.2-1: Edge computing assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE 1) | Indicates that the assist edge computing request was successful. |
| >Assistance informaiton | M | The assistance informaiton for assisting edge computing process, e.g. candidate execution node list, edge computing preparation status at an execution node. |
| >>List of candidate execution nodes | O (NOTE 2) | A list of selected candidate execution nodes. |
| >>Preparation status | O (NOTE 2) | Edge computing preparation status at the execution node provided in request. |
| Failure response | O (NOTE 1) | Indicates that the assist edge computing request was failure. |
| >Cause | M | Reason for the failure. |
| NOTE 1: One of the IEs shall be present. NOTE 2: At least one of the IEs shall be present. |  |  |

* * * Next Change * * * *

### 9.2.Y	AIMLE Assistance of Edge Computing API

### 9.2.Y.1	General

Table 9.2.Y.1-1 illustrates the API for assist Edge computing. This API enables the communication between VAL server (e.g. CAS, EAS) and the AIMLE server for assistance of an edge computing process.

Table 9.2.Y.1-1: Aimles_EdgeComputingAssist API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimles_EdgeComputingAssist | Request | Request/Response | VAL Server (e.g. CAS, EAS) |

### 9.2.Y.2	Aimles_EdgeComputingAssist_Request operation

API operation name: Aimles_EdgeComputingAssist_Request

Description: The consumer requests for assistance of an edge computing process.

Inputs: See clause 8.x.3.1.

Outputs: See clause 8.x.3.2.

See clause 8.x.2 for details of usage of this operation.

# *** End of Changes ***