3GPP TSG-CT WG1 Meeting #150	C1-244503

# Maastricht, Netherlands, 19-23 August 2024

(revision of C1-244075)

# Source:	China Unicom

# Title:	New WID on CT Aspects of Indirect Network Sharing (TEI19_NetShare)

# Document for:	Endorsement

# Agenda Item:	19.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	CT Aspects of Indirect Network Sharing (TEI19_NetShare)

Acronym:	TEI19_NetShare

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI19_NetShare | SA2 | 1030011 | Indirect Network Sharing |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification: None

3	Justification

SA#103 has agreed New 3GPP Rel-19 WID on Indirect Network Sharing (TEI19_NetShare, UI: 1030011), see SP -240488.

In SA WG2, the new network sharing mechanism called as “Indirect Network Sharing” has been studied and specified for operators who intend to use the network sharing mechanism to complement the existing network, taking operators’ business considerations into account.

SA#104 has approved related CRs on TS 23.501 and TS 23.502 as stage 2 normative works. And the TEI19 WI has been completed after SA#104.

The outcome of stage 2 WI is stable to initiate the stage 3 normative implementations.

4	Objective

The objective of this CT WID is to implement the outcome of the Stage 2 TEI19_NetShare WID. Below is an outline of implications on CT specifications.

CT1 Potential Impacts:

- Clarify that the list of equivalent PLMNs can be used in the case of Indirect Network Sharing.

- Clarify that the existing cause value(s) can be used to reject the UE not belonging to the corresponding participating operator properly for Indirect Network Sharing case.

- Add the description of the network slicing handling from network side during registration procedure, UE configuration update procedure and PDU session establishment procedure for Indirect Network Sharing case.

CT4 Impacts:

- NSSF interface:

  - Update the Nnssf_NSSelection Service to support the network slicing handling in the case of Indirect Network Sharing.

- NRF interface:

  - Update the Nnrf_NFManagement Service and the Nnrf_NFDiscovery Service to support the H-SMF discovery and selection in the case of Indirect Network Sharing.

  - Support the Indirect Network Sharing case in N27 interface.

- SMF interface:

  - Support the Indirect Network Sharing case in the Nsmf_PDUSession Service.

  - Support the Indirect Network Sharing case in N16 interface.

CT4 Potential Impacts:

- AUSF interface:

  - Clarify the serving network name used in the Nausf_UEAuthentication Service in the case of Indirect Network Sharing.

- UDM interface：

  - Clarify the serving network name used in the Nudm_UEAuthentication Service in the case of Indirect Network Sharing.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 24.501 | Clarification of network behaviour on the list of equivalent PLMNs, the existing cause value(s) and the related network slicing handling for Indirect Network Sharing. | TSG#107 (March 2025) | CT1 |
| TS 29.531 | Update the Nnssf_NSSelection Service to support the network slicing handling in the case of Indirect Network Sharing. | TSG#107 (March 2025) | CT4 |
| TS 29.510 | Update the Nnrf_NFManagement Service and the Nnrf_NFDiscovery Service to support the H-SMF discovery and selection in the case of Indirect Network Sharing. Support the Indirect Network Sharing case in N27 interface. | TSG#107 (March 2025) | CT4 |
| TS 29.502 | Support the Indirect Network Sharing case in the Nsmf_PDUSession Service. Support the Indirect Network Sharing case in N16 interface. | TSG#107 (March 2025) | CT4 |
| TS 29.509 | Clarify the serving network name used in the Nausf_UEAuthentication Service in the case of Indirect Network Sharing. | TSG#107 (March 2025) | CT4 |
| TS 29.503 | Clarify the serving network name used in the Nudm_UEAuthentication Service in the case of Indirect Network Sharing. | TSG#107 (March 2025) | CT4 |

6	Work item Rapporteur(s)

Xing Tianqi, China Unicom, xingtq@chinaunicom.cn

7	Work item leadership

CT4

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

China Unicom

Huawei

ZTE

China Telecom

Tencent

CATT

vivo

OPPO