3GPP TSG-RAN WG2 Meeting #127														                 R2-2406869

Masstricht, Netherlands, Aug 19th – 23rd, 2024                                        Revision of R2-2405133

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Discussion on enhanced EDT

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#104 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we share our view on this enhanced EDT objective.

# Discussion

Details of Procedure

RAN2 agreed that the enhanced EDT will be a contention-based and EDT-like transaction. And before the enhanced EDT, the UE needs to verify/update the uplink synchronization. The procedural steps are FFS.

RAN2#126 Agreements: 1. RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

Self-evidently, the first step should be the acquisition of the satellite ephemeris and GNSS position to get the UL TA. Then the next step will be a contention based Msg3, followed by a Msg4 containing contention resolution information, DL RRC response message (optional), and (optional) DL user data.

Proposal 1: The steps of the enhanced EDT are as follows:

- Step 1: Acquire SIB31 and GNSS position.

- Step 2: A contention based Msg3.

- Step 3: A Msg4 contains contention resolution (+ DL response message) (+ DL user data).

Similar to legacy EDT, enhanced EDT can fallback to legacy connection establishment if there is more DL data to be transmitted.

Proposal 2: Enhanced EDT can fallback to legacy RRC connection establishment.

Details of Msg3

The aim of the Msg3 of the EDT is to convey the UL user data. Without Msg1 and Msg2, the difference will be the lack of the TAC, UL grant, and TC-RNTI.

The TAC may not be necessary. The UE can calculate a precise UE specific TA based on the latest satellite ephemeris and GNSS position. The accuracy should be sufficient without a UE specific network adjustment (TAC). Anyway, it is still subject to RAN1/RAN4 discussion. We can wait for the response to the outgoing LS R2-2405769.

The UL grant for Msg3 can be provided via system information or dedicated signalling. The difference is that dedicated signalling allows the network to control the number of UEs that use the resource, i.e., the PUSCH resource could be UE specific or a grouped UE specific. For the system information option, although the resources can be divided into groups to reduce the number of UEs using the shared resource, the total number cannot be controlled.

It can be complicated and overhead consuming to control the number of UEs for the network, but it can also help to reduce the collision chance. Without the RACH procedure, UE is going to send Msg3 on PUSCH directly, which increases the cost of UE collisions on shared resource. That is what we are trying to avoid.

On the other hand, the benefit of Msg3 PUSCH resource via system information is that UE can initiate enhanced EDT without having established a connection with the cell. For example, UE can initiate enhanced EDT right after a cell reselection.

Proposal 3: RAN2 to discuss the Msg3 PUSCH resource for enhanced EDT is provided via system information or RRC dedicated signalling.

If the Msg3 PUSCH resource for enhanced EDT provided via system information, the TC-RNTI should be derived by the index of resources. If Msg3 PUSCH resource for enhanced EDT provided via RRC dedicated signalling, the TC-RNTI can be assigned by network, or derived by the index of resources.

Proposal 4: RAN2 to discuss how to derive the TC-RNTI.

Regarding the signalling, we think the legacy RRC signalling can be reused.

Proposal 5: For CP solution, legacy RRCEarlyDataRequest(-NB) can be used to perform enhanced EDT as Msg3.

Proposal 6: For UP solution, legacy RRCConnectionResumeRequest(-NB) can be used to perform enhanced EDT as Msg3.

Details of Msg4

Because the Msg3 is contention based, contention must be resolved in Msg4. Hence, a contention resolution MAC CE should be included in the Msg4. The details of contention resolution MAC CE can be further discussed.

Proposal 7: A contention resolution MAC CE must be included in the Msg4 for enhanced EDT procedure.

Since the contention resolution is a must and cannot be done in DCI due to the size limitation, using the L1 ACK as the Msg4 is not suitable.

Proposal 8: Using the L1 ACK as the Msg4 is not suitable.

Regarding the RRC signalling, we think the legacy RRC signalling can be reused.

Proposal 9: For CP solution, legacy RRCEarlyDataComplete(-NB) can be used to perform enhanced EDT as Msg4. For UP solution, legacy RRCConnectionRelease(-NB) can be used to perform enhanced EDT as Msg4. For EDT fallback to legacy RRC connection establishment, the legacy RRCConnectionSetup(-NB) can be used as Msg4.

When there is no DL response, there will be only a contention resolution MAC CE in the Msg4. There is enough room for other information. The signalling overhead can be improved. As one of the work item objectives, to improve the efficiency of Msg4, multiple Msg4s for different UEs can be included in one MAC PDU. The UEs aggregated in one MAC PDU can be from different Msg3 transmissions. The details can be further discussed.

Proposal 10: Multiple Msg4s for different UEs can be included in one MAC PDU.

CRDSA vs. OCC

In the last RAN2 meeting, companies bring new methods for Msg3 transmission.

RAN2#126 agreements: 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

The purpose of the new methods, known as DSA and CRDSA, is to reduce the collision probability and improve the transmission robustness. However, in DSA and CRDSA, UE will have to transmit multiple replicas of Msg3, which will increase the channel occupancy. Also, the system complexity will increase due to the introduction of new mechanism. The network will be impact by the need to buffer all the data among the slots for CRDSA.

On the other hand, Orthogonal Covering Codes (OCC), as another Rel-19 IoT NTN improvement, can also reduce the collision probability and improve the transmission robustness.

Considering the OCC has been an objective of Rel-19 IoT NTN improvement, and it is too complex to support both OCC and CRDSA, we believe it is best to only support OCC in the enhanced EDT. The total specification impact will be minimized. The details of supporting OCC in enhanced EDT can be further discussed. If RAN2 cannot reach an agreement on this, RAN1 can be consulted for their preference.

Proposal 11: Support OCC in the enhanced EDT instead of CRDSA.

# Conclusion

Details of Procedure

Proposal 1: The steps of the enhanced EDT are as follows:

- Step 1: Acquire SIB31 and GNSS position.

- Step 2: A contention based Msg3.

- Step 3: A Msg4 contains contention resolution (+ DL response message) (+ DL user data).

Proposal 2: Enhanced EDT can fallback to legacy RRC connection establishment.

Details of Msg3

Proposal 3: RAN2 to discuss the Msg3 PUSCH resource for enhanced EDT is provided via system information or RRC dedicated signalling.

Proposal 4: RAN2 to discuss how to derive the TC-RNTI.

Proposal 5: For CP solution, legacy RRCEarlyDataRequest(-NB) can be used to perform enhanced EDT as Msg3.

Proposal 6: For UP solution, legacy RRCConnectionResumeRequest(-NB) can be used to perform enhanced EDT as Msg3.

Details of Msg4

Proposal 7: A contention resolution MAC CE must be included in the Msg4 for enhanced EDT procedure.

Proposal 8: Using the L1 ACK as the Msg4 is not suitable.

Proposal 9: For CP solution, legacy RRCEarlyDataComplete(-NB) can be used to perform enhanced EDT as Msg4. For UP solution, legacy RRCConnectionRelease(-NB) can be used to perform enhanced EDT as Msg4. For EDT fallback to legacy RRC connection establishment, the legacy RRCConnectionSetup(-NB) can be used as Msg4.

Proposal 10: Multiple Msg4s for different UEs can be included in one MAC PDU.

CRDSA vs. OCC

Proposal 11: Support OCC in the enhanced EDT instead of CRDSA.

# References

[1] RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)