3GPP TSG-RAN WG3 #125	R3-244197

Maastricht, Netherlands, 19 – 23 Aug. 2024

Agenda Item:	11.4. R18 leftovers

Source:	NTT DOCOMO INC.

Title:	Discussion on AIML for energy saving

Document for:	Discussion, agreement

Introduction

At the last RAN3 meeting [1], the discussion on the predicted energy cost could not be converged regarding the energy saving use case. On the other hand, the following FFS was captured:

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

In this contribution, we discuss further granularity of energy cost.

Discussion

Use of finer granularity of EC

In Release 18, RAN3 supports per gNB level measured energy cost. In the previous meeting, Nokia proposed per HO granularity energy cost [2], but it was difficult to support it as a measured EC enhancement because the energy consumption of a cell is not corresponding to any hardware of the RAN node thus it is not measurable.

First, we discuss the use of EC at each granularity. The figure shows the general procedure for energy saving in AI/ML for NG-RAN. In the figure, gNB#2 receives the EC (value X1 and X3) at the time before the offload and the EC (value X1' and X3') at the time after the offload, and it learns from the difference how the offload to each gNB affects its energy consumption. However, the learning at the gNB#2 with this flow may be inaccurate in the following cases:

- If gNB#1 and gNB#3 are the same gNB and Cell#A and Cell#C are different Cells, the difference between the impact of offload to Cell#A and offload to Cell#C on the target gNB's energy consumption cannot be learned properly.

- If there is a large change in the number of served UEs or traffic (e.g., due to offload action from gNB#4 to gNB#3) among the EC reports before and after the offload, the impact of the offload action on the target gNB's energy consumption cannot be accurately learned.

If per cell level EC reporting is introduced and supported as the energy currently consumed by the gNB to serve the UEs in the cell, it is possible to accurately learn the impact of the offload action on the target gNB's energy consumption even in the case 1. If the source gNB is inaccurate in predicting the impact of the offload action on the target gNB's energy consumption when deciding the offload action, it may select an inappropriate target cell and thus fail energy saving. On the other hand, in the case 2, it is still not possible to perform accurate learning.

If per HO level EC reporting is introduced and supported as the increased energy consumption of a gNB to serve a given UE in the gNB, accurate learning can be achieved in case 2 in addition to case 1. The source gNB can isolate the impact of a particular offload action from other gNB’s actions based on information related to the UE that performed the offload, the target cell, time, etc.

Legacy measured EC is defined as a measurable metric in a real because it is energy consumption associated with gNB hardware, but per-cell or per-HO EC is not associated with  3GPP node hardware directly, so how to define and report this is a problem.

Observation 1:	Measured EC per gNB level alone supports limited scenario of energy saving.

Observation 2:	On the granularity of EC, there are trade-off between feasibility of implementation and accurate energy consumption prediction after offload for energy saving.

Fig. General procedure of AI/ML for NG-RAN energy saving function.

How to calculate finer granularity of EC

Since per cell level EC (or beam, DRB level, etc.) cannot be measured, the gNB needs to estimate its value according to its implementation. Where the ground truth cannot be measured, it is difficult to improve the accuracy of this estimation. There are two possible ways to improve the accuracy.

- Ensure that the sum of the ECs for all cells associated with the gNB is always equal to the per gNB level EC.

- Compare the difference between the per cell level EC before and after receiving a single/particular offload(s) and the per gNB level EC, and learn whether the change in the per cell level EC matches the change in the per gNB level EC.

Neither method is perfect for improvement of per cell level EC accuracy, and this accuracy is completely dependent on the implementation of the gNB.

The per HO level EC also cannot be measured, so the gNB should estimate its value, and the ground truth also cannot be measured, thus perfect accuracy of the estimation cannot be guaranteed. However, for per HO level EC, the difference between the per gNB EC (i.e., legacy measured EC) immediately before and after a single HO is the per HO level EC, so gNB may be able to improve its accuracy e.g. with machine learning.

Both per cell level and per HO level allow gNB vendors to test whether their implementation can properly estimate EC before deploying it, however, per HO level is relatively easier to prepare the samples than per cell level. For example, in the case of per cell level, it is necessary to prepare samples for each number of UEs and traffic to be served in a cell, but in the case of per HO level, it is sufficient to prepare several samples for a single UE, its type and traffic pattern.

Observation 3:	Per HO EC is more feasible than per cell EC in terms of implementing its estimation in the gNB.

The measured EC in release 18 is per gNB level, and even if per gNB-DU level is supported in release 19, the method of measuring energy consumption is still not clearly defined, and virtualized gNBs are not explicitly excluded. Therefore, the method of calculating EC is highly dependent on the implementation of gNBs.

Based on the above analysis, we propose to capture support for per HO level EC to TR38.743.

Proposal:	RAN3 should capture reporting of current/estimated EC per HO granularity with attached TP.

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	Measured EC per gNB level alone supports limited scenario of energy saving.

Observation 2:	On the granularity of EC, there are trade-off between feasibility of implementation and accurate energy consumption prediction after offload for energy saving.

Observation 3:	Per HO EC is more feasible than per cell EC in terms of implementing its estimation in the gNB.

Proposal:	RAN3 should capture reporting of current/estimated EC per HO granularity with attached TP.

References

- RAN3#124 Chair Notes

- R3-243493, “AI/ML Energy Saving Enhancements”

Annex: TP for TR 38.743

- -------- start of changes --------

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[y]	3GPP TS 38.423: " NG-RAN; Xn application protocol (XnAP); (Release 18) ".

- -------- skip unchanged part --------

- 5	Rel-18 Leftovers and solutions

Editor Note: Such topics are listed here for further selection/down selection for normative work.

- -------- skip unchanged part --------

- 5.3	Energy saving enhancements

Editor Note: Capture the description and its potential standard impacts.

- 5.3.1	Use case description

- In Rel-18, measured energy cost [y] was introduced as a metric representing the energy consumption of a gNB. Introducing new metric representing how much each handover increases the energy consumption of the target gNB when a UE is offloaded from the source gNB to the target gNB as estimated energy cost per HO, and reporting this from the target gNB to the source gNB, the source gNB can learn the relationship between the gNB level measured energy cost before the offload and how much the target gNB's power consumption increases with the type and number of UEs to be offloaded.

- Based on this learning, the source gNB can predict how much the offload action will increase the energy consumption of the target gNB and can therefore determine the offload action that will minimize the total energy consumption of all gNBs in a particular area.

- 5.3.2	Potential standard impacts

Following standard impacts are expected to be needed for support of estimated energy cost per HO granularity.

Xn interface:

- Enhance Data Collection procedure to carry estimated energy costs per HO.

- Specify reporting procedure considering interaction between Data Collection procedure and Handover procedure.