3GPP TSG-RAN WG3 Meeting #125	R3-244392

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	12.2

Source: 				LG Electronics Inc.

Title:			(TP for TR 38.799) Remaining open issues in WAB

Document for:		Discussion and Decision

1.	Introduction

There are some remaining open issues on the WAB study [1]. In order to complete this study item [2] on WAB, these open issues need to be resolved. Also, the WAB architecture and protocol stack should be determined for a normative phase. In this contribution, we will discuss the open issues raised by the SA2 in the LS and provide our view on it.

2.	Discussion

2.1 Remaining open issues on WAB study

During WAB-node movement, the WAB-gNB may need to remove the NG interface with old AMF. When the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the WAB-gNB also needs to disconnect the NG connection with the UE’s AMF. Therefore, in last RAN3 meeting, it was agreed that the NG Removal procedure is introduced during the normative phase. However, in [3], it is still FFS on whether NG connection(s) can be suspended.

Our understanding is that for the case where the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the NG connection(s) removal should be triggered instead of the suspension. In other words, there is no need to suspend the NG connection(s) because the WAB-gNB does not have the right to serve UE(s) any more. For the WAB-node mobility, the suspension of NG connection(s) may be beneficial if the WAB node has cyclic travel routes. However, we think that since it is possible for the operator to have a priori knowledge on WAB (e.g., the travel routes of a vehicle, the location of fixed gNB, and so on), the operator can try to properly provide the WAB-gNB’s configuration considering the cyclic travel routes of the WAB node. As a result, this enables the WAB-gNB to connect same AMF set in the travel routes of the WAB node. Therefore, it is not needed to newly define the NG connection suspend procedure.

Proposal 1: Suspension of NG connection(s) is not supported in this release.

Second open issue in [4] is whether the TAC can be changed without the change of UE’s AMF needs to be confirmed. We think that in the current specification, there is no clear limitations or requirements on this issue. How to (re-)configure the radio configuration parameters (e.g., TAC, PCI) is just up to OAM. Therefore, although the TAC is changed, the UE’s AMF may be not changed. In this case, the WAB-gNB can first indicate the change of the TAC to the UE’s AMF by using the RAN Configuration Update procedure, and then trigger the Handover procedure for the UE in RRC_CONNECTED state if needed. Also, the UE in RRC_IDLE state can initiate the Mobility Registration Update procedure if the new TAC broadcast by the WAB-gNB is not included in the UE’s Registration Area. Therefore, it seems that the TAC change without change of UE’s AMF does not critical specification impact.

Proposal 2: The TAC can be changed without the change of the UE’s AMF.

The following Editor’s notes are also captured into Clause 4.3.3.2.2 of TR 38.799.

- Editor’s NOTE: FFS whether a new logical WAB-gNB needs to be established for the NG connections with the new AMF(s).

- Editor’s NOTE: FFS whether the following steps are needed to support the AMF change for UE.

Due to the mobility, the WAB-gNB may obtain the new cell configuration parameters related to the WAB-gNB’s current location. Then, the WAB-gNB can decide to establish a new logical WAB-gNB, because the old logical WAB-gNB cannot connect to the new AMF(s). Note also that a new logical WAB-gNB may be established because the radio parameters such as the gNB ID and TAC need to be changed based on the new configuration parameters from OAM.

Hence, the WAB-gNB can instantiates new cell(s) with the new radio parameters (e.g., gNB ID, TAC) configured for the new AMF(s) as in Rel-18 mobile IAB-DU inter-CU migration procedure. Then, the UE in RRC_CONNECTED state can be handed over between an old cell and a new cell served by the WAB-gNB via the NG-based handover with AMF re-allocation. Finally, the NG connection(s) between the WAB-gNB and the old AMF(s) are removed.

Proposal 3: The new logical WAB-gNB can be established for the NG connections with the new AMF(s).

However, it is possible that the gNB can connect to the different AMF sets. We think that although the new configuration parameters related to the WAB-gNB’s current location is received, the old logical WAB-gNB can connect to the new AMF(s). That is, the old and new cells belong to the same WAB-gNB. In this case, Steps 3~5 in Clause 4.3.3.2.2 of TR 38.799 can be performed as well.

Proposal 4: The steps 3~5 should be performed to support the AMF change for UE.

2.2 Conclusion on WAB study

During the study phase, three examples of WAB architecture are captured into Figures 4.2-1, 4.2-2, and 4.2-5. However, as described in Clauses 4.1 and 4.2, the solutions using non-3GPP backhaul and L2TP are out-of-scope of this study. Therefore, it is recommended that the architecture as shown in Figure 4.2-1 is used as the basis for normative work. Also, it is concluded that the establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported. Therefore, it is recommended that the protocol stack of NG and Xn as shown in Figures 4.2-3 and 4.2-4 are used as the basis for normative work.

After studying various WAB design aspects, it is concluded that it is feasible to support the following requirements and functionalities with the recommended WAB architecture and protocol stack:

-	WAB-node integration procedure;

- The overall procedure shown in Figure 4.3.1-1 is used as the basis for normative work;

-	WAB authorization;

- It is consisted of Authorization of the WAB-MT and service authorization of the WAB-gNB;

- Removal of NG connection(s) is supported;

- The stage 3 details can be further discussed during the normative work;

-	Configuration of WAB-node;

- Certain configurations of the WAB-node may need to be updated as the node moves, e.g.:

- The parameters that enable the WAB-gNB to select and connect to the AMF(s) to serve the UE(s);

- The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system;

- The configuration parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s);

-	Mobility handling;

- The following scenarios for WAB-node mobility are supported:

- The UE´s AMF/UPF remains unchanged as the WAB-gNB moves;

- The UE´s AMF/UPF changes as the WAB-gNB moves;

- The BH UPF/AMF remains unchanged as the WAB-MT moves inside a PLMN;

- The BH UPF/AMF changes as the WAB-MT moves inside a PLMN;

-	Resource multiplexing;

- In scenarios where WAB-node’s access link and backhaul link mutually interfere, the resource coordination mechanism introduced for IAB can be considered as the starting point;

- It is recommended to introduce the solution on how the BH-gNB discovers the co-location of the WAB-MT and the WAB-gNB during the normative work.

-	Preventing multi-hop backhauling;

- During the Registration procedure, the UE’s AMF rejects the registration request from the WAB-MT in IDLE mode based on the information from the WAB node;

- During the Handover procedure, the UE’s AMF or the target WAB-gNB rejects the handover of the WAB-MT in CONNECTED mode.

Proposal 5: It is proposed to capture above texts into the conclusion part of TR.

In order to capture the above proposals, the following proposal is also suggested to RAN3:

Proposal 6: It is proposed to agree the corresponding TPs in Appendix.

3.	Conclusion

In this contribution, we focused on the remaining open issues and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: Suspension of NG connection(s) is not supported in this release.

Proposal 2: The TAC can be changed without the change of the UE’s AMF.

Proposal 3: The new logical WAB-gNB can be established for the NG connections with the new AMF(s).

Proposal 4: The steps 3~5 should be performed to support the AMF change for UE.

Proposal 5: It is proposed to capture above texts into the conclusion part of TR.

Proposal 6: It is proposed to agree the corresponding TPs in Appendix.

4.	Reference

- R3-243844, “Summary of Offline Discussion on additional topological enhancement,” NTTDOCOMO (moderator), May, 2024.

- RP-234041, “New SID: Study on additional topological enhancements for NR,” AT&T, December, 2023.

- R3-243860, “(TP to TR 38.799) On WAB authorization,” CATT, Xiaomi, Lenovo, Ericsson, Nokia, Nokia Shanghai Bell, Samsung, Qualcomm, Huawei, ZTE, May, 2024.

- R3-243960, “(pCR for TR 38.799): Mobility Support for WAB-Nodes,” Ericsson, May, 2024.

5.	Appendix: TP for TR 38.799

This appendix provides the Text proposal for TR 38.799 based on the proposals of this contribution.

----------------Start of the First Change---------------

### 4.3.2	WAB authorization

WAB authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB. The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the co-located WAB-gNB’s traffic via BH PDU session(s).

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

The WAB-gNB’s service authorization status may change. In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed.

### 4.3.3 Configuration of WAB-node

Certain configurations of the WAB-node may need to be updated as the node moves, e.g.:

- The parameters that enable the WAB-gNB to select and connect to the AMF(s) to serve the UE(s).

- The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

- The configuration parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB-node.

Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

#### 4.3.3.1 IP address configuration for WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The WAB-gNB can use the IP address(es) of the WAB-MT for the PDU sessions that backhaul the NG, Xn and OAM traffic. The WAB-gNB supports security protection of NG and Xn via IPsec, as defined by TS 33.501.

In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

It is possible to transport OAM, NG or Xn traffic over other types of tunnel protocols on top of the WAB-MT’s PDU session(s), e.g., such as L2TP. In this case, the WAB-gNB uses different IP address(es) from WAB-MT. Since the support of these tunnel protocols is not defined for NG and/or Xn, such tunnel protocols are out-of-scope of this study.

#### 4.3.3.2	TAC/RANAC (re-)configuration for WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

### 4.3.4	Mobility handling

The following scenarios for WAB-node mobility are supported:

- The UE´s AMF/UPF remains unchanged as the WAB-gNB moves.

- The UE´s AMF/UPF changes as the WAB-gNB moves.

- The BH UPF/AMF remains unchanged as the WAB-MT moves inside a PLMN.

- The BH UPF/AMF changes as the WAB-MT moves inside a PLMN.

RAN3 assumes that the WAB-gNB does not change PLMN while the WAB-node moves across the network.

#### 4.3.4.1	WAB-MT mobility

The WAB-MT reuses legacy mobility handover procedures for the UE as it moves through the BH-RAN.

##### 4.3.4.1.1 The BH-UPF/BH-AMF remains unchanged as the WAB-MT moves inside a PLMN.

In case the WAB-MT’s PSA UPF does not change during these mobility procedures, the IP addresses allocated for the WAB-MT’s PDU sessions do not change. Therefore, the NG and Xn connections of the WAB-gNB carried over the BH PDU session(s) remain unaffected.

##### 4.3.4.1.2 BH-UPF/BH-AMF changes as the WAB-MT moves inside a PLMN.

When the WAB-MT’s PSA UPF need to be changed due to WAB-MT’s mobility, new BH PDU session(s) need to be established reusing existing mechanisms defined in TS 23.501/TS 23.502. In this case, the (outer) IP addresses used by the WAB-gNB for the transport of NG and Xn connections of the WAB-gNB will change.

##### 4.3.4.1.3 Roaming of the WAB-MT

During WAB-node movement, the WAB-MT may connect to a PLMN different than its HPLMN.

#### 4.3.4.2	WAB-gNB mobility

During WAB-node movement, establishment, and removal of the WAB-gNB’s NG and/or Xn connections may be needed.

Establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported, and it can follow legacy procedures.

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502.

##### 4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- The WAB-gNB establishes NG connection(s) towards one or more new AMF(s).

- The WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) remain(s) active.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell and a new cell served by the WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502.

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502) is performed.

- The NG connection(s) between the WAB-gNB and the old AMF(s) are removed.

### 4.3.5 Resource multiplexing

In scenarios where WAB-node’s access link and backhaul link mutually interfere, resource coordination may be needed to facilitate the resource multiplexing of the WAB-node’s access links and backhaul link. For this purpose, the resource coordination mechanism introduced for IAB can be considered as the starting point. For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB.

## 4.4	Other

## 4.5	Conclusions

During the study phase, three examples of WAB architecture are captured into Figures 4.2-1, 4.2-2, and 4.2-5. However, as described in Clauses 4.1 and 4.2, the solutions using non-3GPP backhaul and L2TP are out-of-scope of this study. Therefore, it is recommended that the architecture as shown in Figure 4.2-1 is used as the basis for normative work. Also, it is concluded that the establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported. Therefore, it is recommended that the protocol stack of NG and Xn as shown in Figures 4.2-3 and 4.2-4 are used as the basis for normative work.

After studying various WAB design aspects, it is concluded that it is feasible to support the following requirements and functionalities with the recommended WAB architecture and protocol stack:

-	WAB-node integration procedure;

- The overall procedure shown in Figure 4.3.1-1 is used as the basis for normative work;

-	WAB authorization;

- It is consisted of Authorization of the WAB-MT and service authorization of the WAB-gNB;

- Removal of NG connection(s) is supported;

- The stage 3 details can be further discussed during the normative work;

-	Configuration of WAB-node;

- Certain configurations of the WAB-node may need to be updated as the node moves, e.g.:

- The parameters that enable the WAB-gNB to select and connect to the AMF(s) to serve the UE(s);

- The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system;

- The configuration parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s);

-	Mobility handling;

- The following scenarios for WAB-node mobility are supported:

- The UE´s AMF/UPF remains unchanged as the WAB-gNB moves;

- The UE´s AMF/UPF changes as the WAB-gNB moves;

- The BH UPF/AMF remains unchanged as the WAB-MT moves inside a PLMN;

- The BH UPF/AMF changes as the WAB-MT moves inside a PLMN;

-	Resource multiplexing;

- In scenarios where WAB-node’s access link and backhaul link mutually interfere, the resource coordination mechanism introduced for IAB can be considered as the starting point;

- It is recommended to introduce the solution on how the BH-gNB discovers the co-location of the WAB-MT and the WAB-gNB during the normative work;

-	Preventing multi-hop backhauling;

- During the Registration procedure, the UE’s AMF rejects the registration request from the WAB-MT in IDLE mode based on the information from the WAB node;

- During the Handover procedure, the UE’s AMF or the target WAB-gNB rejects the handover of the WAB-MT in CONNECTED mode.

-----------------End of the First Change---------------