3GPP TSG RAN WG3 Meeting #125	R3-244020

Maastricht, Netherlands, 19 - 23 August, 2024

SA WG2 Meeting #163	S2-2407351

Jeju, Korea, May 27 – May 31, 2024

Title:	LS on FS_XRM Ph2

Response to:

Release:	Rel-18

Work Item:	FS_XRM Ph2

Source:	SA2

To:	SA4, RAN2, RAN3

Cc:

# Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1. Overall Description:

SA2 has started to evaluate solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). SA2 thanks the LS replies for XRM Ph2 from RAN2, RAN3 and SA4 and SA2 would like to further coordinate on the following aspects:

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

  - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

  - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

# 2. Actions:

# To SA4/RAN2/RAN3:

ACTION: SA2 kindly asks SA4/RAN2/RAN3 to provide answers to the questions above.

# 3. Date of Next SA2 Meetings:

SA2#164	19th Aug. – 23th Aug. 2024			Maastricht, Netherlands

SA2#165	14th Oct. – 18th Oct. 2024				Hyderabad, India