3GPP TSG-RAN WG4 Meeting #112	R4-2412925

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.1.1.3.1

Source:            	Google

Title:                	Discussion on REFSENS requirement for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the REFSENS requirements for 6Rx handheld and FWA UE.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#111, the 6Rx REFSENS requirements for handheld and FWA UEs has been discussed and some open issues are captured in the approved WF [2]. In WF, these open issues are mainly about how to specify ΔRIB,6R value for handheld and FWA UEs, and several other related issues also need further discussion. In this paper, we would like to share our view in the following.

# Topic 1: REFSENS (delta RIB,6R) ## Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: For ΔRIB,6R, include n77, n78, n79 and [n104] into the same group. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Way Forward: Further discuss the following options. - Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately - Option 2: Same value for handheld UE and FWA - Option 3: Different value for handheld UE and FWA Issue 1-1-3: Release independence of ΔRIB,6R value Way Forward: Follow RAN4 Chair guidance and defer the release independence discussions until the requirements are stable. ## Sub-topic 1-2: ΔRIB,6R values for handheld UE and FWA Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2. - Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion. Operating bands; ΔRIB,6R for FWA; ΔRIB,6R for Handheld. n77, n78, n79, [n104]; [-3.0 ~ -3.3] dB; [-3.0 ~ -3.2] dB. n41; [-3.0 ~ -4.0] dB; [-3.0 ~ -4.0] dB. - Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion. - Option 3: Other options are not precluded.

# Discussion

In 3GPP technology, multiple antennas technique is a key feature to enhance system data DL throughput. In modern commercial implementation, the handheld UE is often designed to be equipped with four Rx antennas when the frequency ranges are around mid/high band (MHB), e.g., n7 or n41 and so on, and ultra-high band (UHB), e.g., n77 or n79, and so on. However, due to physical form factor constraints, the number of multiple antennas cannot be increased without limitation. Hence, in current specification, 4Rx has been introduced for handheld UE and FWA UE from Rel-15 and 8Rx was introduced only for FWA UE in Rel-18.

For the RF requirement with multiple Rx antennas, the REFSENS is the major requirement which needs to be determined. In 38.101-1, the REFSENS requirements with 2Rx antenna ports for each applicable operating band are defined as the baseline in sub-clause 7.3. As for the UE(s) equipped with 4 Rx antenna ports, the REFSENS requirement should be modified by the amount given in ΔRIB,4R in following Table 7.3.2-2 on top of the 2Rx REFSENS requirements for the applicable operating bands. In addition, due to different hardware implementation complexity, it is also observed that LB 4Rx REFSENS requirement ΔRIB,4R for FWA UE is separated from handheld UE.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

Additionally, RAN4 has also introduced 8Rx REFSENS requirement for FWA UE in Rel-18. The 8Rx requirement is that for the UE(s) equipped with 8 Rx antenna ports, the REFSENS requirement should be modified by the amount given in ΔRIB,8R in following Table 7.3.2-2a on top of the 2Rx REFSENS requirements for the applicable operating bands.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n7 | -4.5 |
| n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

In RAN4#111, REFSENS requirement for 6Rx handheld and FWA UEs has been discussed and some open issues for ΔRIB,6R on the example bands are captured in the WF. One of the open issues for ΔRIB,6R is to determine whether to include n104 as the high band categories (n77, n78, and n79). In our understanding, it is only observed that n104 ΔRIB,4R is defined as the high-band category in Table 7.3.2-2 but n104 ΔRIB,8R is not introduced in Table 7.3.2-2a. The n104 ΔRIB,4R is 0.8 dB (3 dB - 2.2 dB) worse than the ideal value 3dB. In our view, although there is the same ΔRIB,4R requirement between n77/n78/n79 and n104, it may not be the same situation for 6Rx ΔRIB,6R requirements. Considering the frequency range of n104 is much higher than n77/n78/n79, it is expected that PCB tracing loss and RFFE insertion loss may be higher for n104 6Rx UE implementation. Hence, it is proposed to differentiate n104 ΔRIB,6R requirements from the high-band categories n77/n78/n79, and the n104 ΔRIB,6R value can be further discussed.

Proposal 1: Considering the frequency range of n104 is much higher than n77/n78/n79, PCB tracing loss and RFFE insertion loss may be higher for n104 6Rx UE implementation. Hence, it is proposed to differentiate ΔRIB,6R requirements for n104 from the high-band categories n77/n78/n79, and the ΔRIB,6R value for n104 can be further discussed.

In WF, another open issue for ΔRIB,6R is to determine the ΔRIB,6R value for handheld and FWA UE. In our understanding, it is observed that ΔRIB,4R for LB 4Rx has been divided into the handheld UE requirement and FWA UE requirement because the implementation complexity for LB 4Rx may be more challenging for handheld UE than FWA UE. In order to save the cost for 6Rx handheld UE implementation, it is highly possible that n104 can be collocated with WIFI 6G module from implementation perspective. In our view, it would be beneficial to have some more relaxation for ΔRIB,6R due to different implementation complexity. Hence, considering the different implementation complexity for handheld UE and FWA UE, it is proposed to differentiate the n104 ΔRIB,6R requirements for handheld UE from FWA UE.

Proposal 2: In order to save the cost for 6Rx handheld UE implementation, it is highly possible that n104 can be collocated with WIFI 6G module from implementation perspective. Hence, considering the different implementation complexity for handheld UE and FWA UE, it is proposed to differentiate n104 ΔRIB,6R requirements for handheld UE from FWA UE.

Another open issue for ΔRIB,6R is to determine ΔRIB,6R value for the handheld UE and FWA UE. It is also agreed that the existing 4Rx ΔRIB,4R and 8Rx ΔRIB,8R requirements can be used as a starting point to discuss the performance of ΔRIB,6R for handheld UE and FWA UE.

For ΔRIB,6R, it is the reference sensitivity adjustment due to support for 6 antenna ports, which means that the additional gain can be improved compared to the baseline 2Rx REFSENS requirements. In our understanding, the ideal additional gain for 4Rx over the baseline 2Rx would be 3dB, for 6Rx over the baseline 2Rx would be 4.7 dB, and 8Rx over the baseline 2Rx would be 6dB. However, due to the implementation loss such as PCB tracing loss from different Rx path length and/or insertion loss from more RFFE antenna switches or more diplexers, the actual additional gain for 4Rx/6Rx/8Rx over the baseline 2Rx requirement would be less than the ideal value.

For the bands n77/n78/n79 in the high-band categories, it is observed in Table 7.3.2-2 that ΔRIB,4R is 0.8 dB (3 dB - 2.2 dB) worse than the ideal value 3dB, and in Table 7.3.2-2a that ΔRIB,8R is 2 dB (6 dB - 4.0 dB) worse than the ideal value 6 dB. In our view, considering the most frequently used averaging method, the additional worse value for n77/n78/n79 6Rx would be 1.4 dB ((2 dB + 0.8 dB)/2) from the ideal value 4.7 dB.

In addition, the implementation complexity may be different between FWA UE and handheld, so it is suggested to differentiate 6Rx ΔRIB,6R requirements for the handheld UE from the FWA UE. For the 6Rx handheld UE ΔRIB,6R requirements, the similar rule 0.3 dB relaxation for LB 4Rx requirement can be also applied to n77/n78/n79. Hence, for n77/n78/n79, considering different implementation complexity for the handheld UE and the FWA UE, it is proposed to determine ΔRIB,6R = -3.0 for the handheld UE and ΔRIB,6R = -3.3 for the FWA UE.

Proposal 3: For n77/n78/n79, considering different implementation complexity for the handheld UE and the FWA UE, it is proposed to determine ΔRIB,6R = -3.0 for the handheld UE and ΔRIB,6R = -3.3 for the FWA UE.

For the band n41 in the mid-band group, it is observed in Table 7.3.2-2 that ΔRIB,4R is 0.3 dB (3 dB - 2.7 dB) worse than the ideal value 3dB, and in Table 7.3.2-2a that ΔRIB,8R is 1.7 dB (6 dB - 4.3 dB) worse than the ideal value 6 dB. In our view, considering the most frequently used averaging method, the additional worse value for n41 6Rx would be 1.0 dB ((1.7 dB + 0.3 dB)/2) from the ideal value 4.7 dB.

In addition, the implementation complexity may be different between FWA UE and handheld UE, so it is suggested to differentiate 6Rx ΔRIB,6R requirements for the handheld UE from the FWA UE. For the 6Rx handheld UE ΔRIB,6R requirements, the similar rule 0.3 dB relaxation for LB 4Rx requirement can be also applied to n41. Hence, for n41, considering that the different implementation complexity for the handheld UE and the FWA UE, it is proposed to determine ΔRIB,6R = -3.4 for the handheld UE and ΔRIB,6R = -3.7 for the FWA UE.

Proposal 4: For n41, considering that the different implementation complexity for the handheld UE and the FWA UE, it is proposed to determine ΔRIB,6R = -3.4 for the handheld UE and ΔRIB,6R = -3.7 for the FWA UE.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: Considering the frequency range of n104 is much higher than n77/n78/n79, PCB tracing loss and RFFE insertion loss may be higher for n104 6Rx UE implementation. Hence, it is proposed to differentiate ΔRIB,6R requirements for n104 from the high-band categories n77/n78/n79, and the ΔRIB,6R value for n104 can be further discussed.

Proposal 2: In order to save the cost for 6Rx handheld UE implementation, it is highly possible that n104 can be collocated with WIFI 6G module from implementation perspective. Hence, considering the different implementation complexity for handheld UE and FWA UE, it is proposed to differentiate n104 ΔRIB,6R requirements for handheld UE from FWA UE.

Proposal 3: For n77/n78/n79, considering different implementation complexity for the handheld UE and the FWA UE, it is proposed to determine ΔRIB,6R = -3.0 for the handheld UE and ΔRIB,6R = -3.3 for the FWA UE.

Proposal 4: For n41, considering that the different implementation complexity for the handheld UE and the FWA UE, it is proposed to determine ΔRIB,6R = -3.4 for the handheld UE and ΔRIB,6R = -3.7 for the FWA UE.

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2410751, “WF on requirements for 6Rx”, AT&T, RAN4#111

- R4-2409049, “Discussion on REFSENS requirement for 6Rx UE”, Google, RAN4#111