# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415034

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.18.3

Source: 	OPPO

Title: 	Study of testability and interoperability for AI mobility

Document for:	Approval

- Introduction

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In RAN2#126 meeting, RAN2 has decided to focus on the following scenarios and conditions for RRM measurement prediction, and provided an explanation of the priorities.

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we will discuss the RAN4 impacts on testability based on RAN2 framework and assumptions.

- Discussion

The testing of RRM measurement prediction and the AI-based BM testing currently being discussed in R19 share many similarities. However, in the RAN2 AI mobility project, the potential testing scenarios involved are far greater than those in the BM cases. For example, different sub-use cases 1/2/3, various scenario assumptions that involve FR1 and FR2, intra/inter-frequency prediction, intra/inter-cell prediction, case A/B prediction, test goals 1/2, L1/L3-RSRP measurement, are taken into account.

Proposal 1: Regarding the testability issues for RRM measurement prediction, at least following aspects need to be considered:

- Testing in FR1

- Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction

- Testing L3-RSRP measurements instead of L1-RSRP

These different constraints pose significant challenges to the testing of AI mobility, especially on the testability issues. To kick off the study, at least following aspects need to be considered:

- Testing in FR1:

For the AI BM study, the focus is primarily on temporal or spatial prediction of different beams within a FR2 cell. However, for the AI mobility, besides the measurement prediction in FR2, prediction in FR1 is also introduced.

For FR2 prediction, the testability of the FR2 to FR2 intra-frequency temporal domain case A needs to be discussed with high priority, as agreed in RAN2. Proposals and conclusions that RAN4 made for AI BM can be reused, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it. Also, questions like how to build channel modeling and channel correlation in the temporal domain for FR2 temporal domain prediction should be evaluated.

For FR1 prediction, the temporal domain case B and frequency domain prediction related to FR1 to FR1 intra-frequency need to be handled with high priority. These two use cases closely resemble BM-case1 and BM-case2, but it's necessary to consider if any new testability issues arise with the FR1 assumption. First, we need to determine whether the FR1 test should be performed through OTA testing or conductive testing. In our understanding, for cases involving only FR1 to FR1, using conductive tests as the basic assumption for the test environment is more appropriate.

Proposal 2: For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it.

Proposal 3: For FR1 prediction, it is necessary to consider whether the testing assumption for FR1 rises any additional testing issues.

- RAN4 needs to determine whether the FR1 test should be performed through OTA testing or conductive testing.

Proposal 4: For FR1 prediction, using conductive testing as the basic assumption for the test environment is more appropriate

- Testing inter-carrier scenario

For AI mobility, an important feature is the introduction of inter-carrier scenario, e.g., inter-frequency inter-cell prediction.

For intra-cell predictions, FR1 to FR1 intra-frequency temporal domain case B and FR2 to FR2 intra-frequency temporal domain case A are high-priority assumptions. For temporal prediction within a cell, constructing temporal measurement correlation in the test environment needs to be considered.

For inter-frequency inter-cell prediction predictions, the priority is with the FR1 to FR1 inter-frequency domain measurement predictions. In such cases, the following issues should be considered:

- Spatial modeling: How to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighboring site. For instance, if we're using conductive testing in FR1, how do we reflect the assumption of co-location among different cells? For example, are there any new designs and limitations needed during channel modeling?

- Temporal modeling: Furthermore, it's important to consider how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship.

For example, if we consider predicting cell B's RSRP at time T based on cell A's RSRP at time T, we need to consider how to construct and obtain both cell A's RSRP and cell B's RSRP corresponding to the same time T in the testing environment. If cell B's RSRP is time-varying and if the RSRP of cell B serving as the label data is of a different time moment(not time T), it will create a problem for the test's validity. Building a stable time-domain RSRP signal for a single test might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time.

- Channel modeling: Besides, to ensure the testability of measurements for inter-cell prediction, on one hand, a certain correlation between the measurements(or channel models) of different cells should be introduced. On the other hand, the correlation between the measurements(or channel models) of different cells should not be too strong, as an extreme case where there is no distinction between different cells would undermine the necessity for measurement prediction testing. Channel models that proposed and analyzed in R19 BM discussions could be considered as the starting point.

Proposal 5: For inter-carrier scenario (inter-frequency inter-cell prediction), how to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighbouring site should be considered.

Proposal 6: For inter-carrier scenario (inter-frequency inter-cell prediction), how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship should be considered.

-  Stable time-domain RSRP signal for the tests might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time.

Proposal 7: For inter-carrier scenario (inter-frequency inter-cell prediction), to ensure the testability of measurements for inter-cell prediction, channel modelling for inter-cell prediction should be considered.

- Channel models proposed and analyzed in R19 BM discussions could be considered as the starting point.

- Testing L3-RSRP measurements instead of L1-RSRP

Regarding L3 RSRP, compare with L1 RSRP, the key difference lies in the L3 filter which may need to be included as part of the testing side conditions.

Proposal 8: Regarding L3 RSRP, compare with L1 RSRP, the key difference lies in the L3 filter which may need to be included as part of the testing side conditions.

- Conclusions

In this contribution, we discussed RAN4 impacts on testability based on RAN2 framework and assumptions, and got following proposals:

Proposal 1: Regarding the testability issues for RRM measurement prediction, at least following aspects need to be considered:

- Testing in FR1

- Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction

- Testing L3-RSRP measurements instead of L1-RSRP

Proposal 2: For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it.

Proposal 3: For FR1 prediction, it is necessary to consider whether the testing assumption for FR1 rises any additional testing issues.

- RAN4 needs to determine whether the FR1 test should be performed through OTA testing or conductive testing.

Proposal 4: For FR1 prediction, using conductive testing as the basic assumption for the test environment is more appropriate

Proposal 5: For inter-carrier scenario (inter-frequency inter-cell prediction), how to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighbouring site should be considered.

Proposal 6: For inter-carrier scenario (inter-frequency inter-cell prediction), how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship should be considered.

-  Stable time-domain RSRP signal for the tests might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time.

Proposal 7: For inter-carrier scenario (inter-frequency inter-cell prediction), to ensure the testability of measurements for inter-cell prediction, channel modelling for inter-cell prediction should be considered.

- Channel models proposed and analyzed in R19 BM discussions could be considered as the starting point.

Proposal 8: Regarding L3 RSRP, compare with L1 RSRP, the key difference lies in the L3 filter which may need to be included as part of the testing side conditions.

- References

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR