3GPP TSG-RAN WG4 Meeting #112bis	R4-2415133

Hefei, China, Oct 14th – 18th, 2024

Title: 	Testability for UE RF requirements for LP-WUS

Agenda Item:	6.23.2.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, testability for UE RF requirements for LP-WUS has been discussed [1] [2], and at this stage, the general framework for verifying LP-WUS performance is the main focus at this stage:

Issue 4-1-1: General framework on LP-WUS testing Agreements: - RAN4 further discuss the high-level approach on how to achieve LP-WUS performance verification.

In this contribution, we present several issues to be considered for the general framework for testability.

# 2 Discussion

It is understood that LP-WUS must be supported in all three RRC states (i.e., RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED). The primary goal of introducing LP-WUS/WUR is to allow the main radio (MR) to remain dormant while only the LP-WUR is active, thereby conserving power. Clearly, regardless of the UE’s RRC state, the process for waking the MR when it is dormant and only LP-WUR is active remains consistent. The LP-WUR receives and accurately decodes the wake-up signal intended for the UE. Thus, from a testing perspective, LP-WUR performance is uniform across all three RRC states. That’s the foundation for introducing the same performance requirements for all three RRC states.

Observation 1: The same LP-WUS performance requirements are defined for all three RRC states.

With such consideration, LP-WUS can be tested under only one RRC state for testing effort reduction.

Observation 2: Only one RRC state can be selected for verifying LP-WUS performance.

To reduce testing time, there is a proposal suggesting that the MR does not need to actually wake up during the test. However, implementing this approach may necessitate a dedicated test mode with altered UE behaviours, and potentially even auxiliary tools for accurately measuring metrics like MDR and FAR.

Observation 3: If MR does not actually wake up during tests, a dedicated test mode with different UE behaviours and some auxiliary tool may be required.

If the UE is tested in the RRC_CONNECTED state, a dedicated test mode for the UE is unnecessary. Additionally, metrics such as MDR and FAR statistics can be obtained via the MR, making the process more convenient and eliminating the need for auxiliary tools. Therefore, we propose to test LP-WUS performance solely in the RRC_CONNECTED state.

Proposal: LP-WUS is tested only in RRC_CONNECTED state.

# 3 Conclusion

In this contribution, we have the following observations and proposal for testability for UE RF requirements for LP-WUS:

Observation 1: The same LP-WUS performance requirements are defined for all three RRC states.

Observation 2: Only one RRC state can be selected for verifying LP-WUS performance.

Observation 3: If MR does not actually wake up during tests, a dedicated test mode with different UE behaviours and some auxiliary tool may be required.

Proposal: LP-WUS is tested only in RRC_CONNECTED state.

# Reference

[1] R4-2414309, “WF on LP-WUS UE RF requirements”, vivo

[2] R4-2412835, “Topic summary for [112][133] NR_LPWUS_UERF”, Moderator(vivo)