3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415159

# Hefei , CN, Oct 14 – 18, 2024

Title:	Draft reply LS on common TA in a regenerative payload scenario

Response to:	R2-2407623

Release:	Release 19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN 4

To:	RAN 2

Cc:	RAN 1

Contact person:

#### Name:	Lingyu Gao

#### E-mail Address:	gaolingyu@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN4 thanks RAN2 for the LS sent in R2-2407623 asking about the value setting of common TA for regenerative payload, based on the discussion in RAN4, RAN4 would like to provide the following feedback for RAN2’s question:

Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

RAN4 feedback:

In regenerative payload scenario, RAN4 think that sticking to 0 as the minimum possible value for TA-Common is reasonable.

From RAN4’s perspective, there is no need to introduce negative values for TA-Common. Firstly, the negative values for TA-Common don’t correspond to its real physical meaning. Secondly, RAN4 defaults that the UE needs to meet the UE UL transmit timing Te requirements (Te_NTN) defined in clause 7.1C.2 in TS38.133 for UL initial transmission [attached in Annex], and all of possible errors and enough margin has been taken into account when RAN4 specifying the Te requirements.

In addition, the values of Te_NTN are also smaller than half of CP length for the given SCS of UL channel/signal, except in the case of a 120 kHz SCS of uplink signals in FR2 for mobile VSAT is served by GSO. Therefore, it also means that the current Te requirements make it possible for the network side to demodulate the uplink information of different UEs through implementation in most cases, and there is no need to consider any errors when setting value for TA-Common.

2. Actions:

To RAN WG2 group

ACTION: 	RAN4 sincerely asks RAN2 to take into account the above feedback into consideration.

3. Date of Next TSG RAN WG4 Meetings:

3GPP TSG RAN WG4#113	18 - 11 November 2024	    Orlando, US

3GPP TSG RAN WG4#114	17 - 21 Feb 2025		    Athens, GR

A. Annex A: UE transmit timing for Satellite Access (TS 38.133)

### 7.1C.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te_NTN where the timing error limit value Te_NTN.

Te_NTN is specified in Table 7.1C.2-1 for FR1-NTN.

Te_NTN is specified in Table 7.1C.2-2 and Table 7.1C.2-3 for VSAT UE in FR2-NTN.

This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission..

The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc.

The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna.

NTA for PRACH is defined as 0. NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. or after the last update in NTA,adjcommon  or NTA,adjUE.

The value of NTA-offset depends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). NTA-offset is defined in Table 7.1.2-2 for FR1-NTN.

NTA-offset is defined in Table 7.1.2-2 for VSAT UE in FR2-NTN.

NTA,adjcommon and NTA,adjUE are as defined in TS38.211 [6].

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |