3GPP TSG-RAN WG4 Meeting #112bis		R4-2415493

Hefei, China, 14 – 18, 2024

Agenda Item:	6.25.2

Source: 	Apple

Title:	Discussion of RRM impact of XR for NR Phase 3

Document for:	Approval

# Introduction

RRM impact of XR for NR Phase 3 has been widely discussed in previous RAN4 meetings. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

The first issue we would like to discuss is about RAN1 LS on minimum time offset(s) X between indication to skip and skipped measurement occasion:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

RAN1 is asking the minimum time offset X between DCI and the skipped measurement occasion. Take gap cancellation for example:

In above example, the second gap occasion is already planed if there is no DCI. UE already canceled data Rx/Tx for the second gap occasion. Now with the new DCI received, UE needs to cancel the second gap and resume data Rx/Tx. Some lead time is necessary from UE implementation point of view. Regarding the exact value, RAN4 has a similar procedure, Pre-MG deactivation, which was introduced in R17. The deactivation delay is 5ms after the event. Take BWP switching for example, total Pre-MG deactivation delay is 5ms after BWP switching completes. BWP switching delay is 1ms and 3ms for type 1 and type 2 UE, repsectively. Thus total delay of Pre-MG deactivation is 6~8ms. To simplify RAN4 design, here we propose a single value 7ms for all UE.

Observation 1: Pre-MG deactivated delay is 6~8ms per RAN4 requirements.

Proposal 1: as baseline, 7ms minimum time offset is needed between indication to skip and skipped measurement occasion.

Another issue is to discuss whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to RRM measurement outside gap and L1 operation like RLM and BFD. We would like to highlight that intra-frequency handover is the most typical handover in real deployment, wherein UE is not configured with measurement gap. Therefore, if we only focus on the case with measurement gap configured, we cannot benefit from this feature in most typical scenario.

Observation 2: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap.

From feasibility point of view, we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions like power saving. It is straightforward to us that at least we can let NW keep scheduling UE, as necessary for XR traffic, when UE is not actually performing RRM measurement, RLM and BFD due to relaxation. Besides, we can also consider exchanging measurement order information between UE and NW so that NW can know whether there is scheduling restriction on each measurement occasion, when there are multiple carriers to measure. Thereby, NW can perform measurement scheduling restriction cancellation precisely.

Observation 3: we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions. At least it is feasible to schedule UE on RRM, RLM and BFD occasions which would cause scheduling restriction but UE is not actually performing measurement.

Proposal 2: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap.

Proposal 3: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

It is straightforward that measurement cancellation has negative impact on measurement. In order to maintain the same measurement performance, UE still needs same number of measurement samples in physical layer. Assuming during the measurement period, network cancels X measurement occasions, then the total measurement delay shall be extended by X measurement occasions. Take inter-frequency measurement with gap for example:

Proposal 4: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Note that if too many measurement occasions are cancelled on target carrier (UE cannot measure it for a long time), UE may need to adjust AGC and timing again before it can perform measurement on the carrier successfully. From UE measurement point of view, this is similar to measurement cancellation due to LBT failure. Similarly, max number of occasions shall be defined within a measurement period. If actual number of cancelled occasions exceeds the maximum value, UE shall be allowed to restart a measurement period.

Proposal 5: max number of occasions cancelled during a measurement period shall be defined.

However, measurement cancellation is not completely same as LBT failure. One aspect is that LBT only results per-cell measurement failure, while in XR measurement cancellation UE would lose opportunities for measurement on all cells (maybe even across all carriers). Thus, we open for any other solution, if justified.

Proposal 6: RAN4 can continue discussion on other solution to address negative impact on measurement due to cancellation for XR traffic.

Another issue discussed in RAN4#112 is about which measurement gap types to be supported in this work item. We continue supporting the following types:

- Type-1 gap

- Type-2 gap

- NCSG

- concurrent gaps

- pre-MG

Regarding MUSIM gaps and positioning gaps, we don’t have strong view.

Proposal 7: at least the following types of measurement gaps shall be included in this work item:

- Type-1 gap

- Type-2 gap

- NCSG

- concurrent gaps

- pre-MG

The last thing we would like to discuss is about UE assistance information. UE knows better about measurement performance. For instance, when UE is in low mobility or not-at-cell-edge, UE can tolerate more measurement cancellation. When UE is at cell edge, UE doesn’t expect too many measurement cancellation. A simple solution is to let UE indicates NW a status whether it can tolerate measurement cancellation.

Proposal 8: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation.

# Conclusion

In this contribution, we provide discussion on RRM impact of XR for NR phase 3. After discussion, the following conclusions are provided:

Observation 1: Pre-MG deactivated delay is 6~8ms per RAN4 requirements.

Proposal 1: as baseline, 7ms minimum time offset is needed between indication to skip and skipped measurement occasion.

Observation 2: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap.

Observation 3: we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions. At least it is feasible to schedule UE on RRM, RLM and BFD occasions which would cause scheduling restriction but UE is not actually performing measurement.

Proposal 2: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap.

Proposal 3: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

Proposal 4: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 5: max number of occasions cancelled during a measurement period shall be defined.

Proposal 6: RAN4 can continue discussion on other solution to address negative impact on measurement due to cancellation for XR traffic.

Proposal 7: at least the following types of measurement gaps shall be included in this work item:

- Type-1 gap

- Type-2 gap

- NCSG

- concurrent gaps

- pre-MG

Proposal 8: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation.

# Reference

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia